Supplemental Re-evaluation

for

CUY ANSEL ROAD PID 102532

Environmental Document Level: C2

Approved: June 28, 2017

Prepared by: Libby Rushley

Lawhon  AND  Associates

e-mail:

The environmental review, consultation, and other actions required by applicable Federal environmental laws for this project are being, or have been, carried-out by ODOT pursuant to 23 U.S.C. 327 and a Memorandum of Understanding dated December 11, 2015, and executed by FHWA and ODOT.

Contents

C2	3

Environmental Commitments, 	9

Environmental Commitments	11

C2	11

Preparers  & Approvals	13

Form Preparer(s)	13

Approvals & Electronic Signatures	13

Appendix	14

Uploaded Files by Report Section	14

Noise	14

Section 4(f)	14

General	14

ESA	14

Cultural Resources	14

Ecological	15

Other Resources	15

Environmental Justice	15

Public Involvement	15

Permits	15

Approved Documents	16

C2

| PID | 102532 |
| --- | --- |

| Project Sponsor | Cleveland, City of |
| --- | --- |

| ODOT District | 12 |
| --- | --- |

| Funding Source | Federal |
| --- | --- |

| Private Funding: | No |
| --- | --- |

Project Description:

Reevaluation Description

The project was incorrectly approved on 3-23-2017 before final approval from OHPO was documented.   ODOT had received a verbal approval to the No Adverse Affect determination from OHPO in March but did not have proper documentation of it.

ODOT confirmed the No Adverse Affect determination with OHPO on   May 25, 2017.   See OHPO approval e-mail in Project File/Cultural Resources/Coordination/OES Response to Agency Comments.pdf.

On June 19, 2017, OES determined that the proposed undertaking qualifies as an exception to the requirement for Section 4(f) approval for cultural resources.   The OES Section 4(f) Exception determination is in Project File/Section 4(f)/Project Information/CUY Ansel Road PID 102532 cultural 4f excempt notificiton.pdf.

The C2 determination for CUY-St. Casimir Way bridge project is now properly documented and valid.

Original Project Description

The correct name of the project location is St. Casimir Way. It was changed from Ansel Road and Dryden Road.

The City of Cleveland Departmentof Public Works Street Repair Section proposes to rehabilitate the bridge whichcarries St Casimir Way over Doan Brook, known as CUY Liberty 5:033M in the Cityof Cleveland, Ohio. The rehabilitation work includes replacing the existingsuperstructure with new galvanized steel beams with composite reinforced concrete deck,constructing new reinforced concrete abutment seats and semi-integral diaphragms, and installing new approachslabs. The project is located within Rockefeller Park and Cultural GardensHistoric District, which is listed on the National Register of Historic Places(NRHP) (NRHP#05000382). Construction is currently scheduled for 2019.

St Casimir Way is functionally classified as an Urban Local in the project area.

No right-of-way is required for this project.

A detour will be required to rehabilitate the structure.  The proposed detour route has been coordinated with local emergency services and school districts, and their comments were addressed during design.

OES issued the RGP B for the project.

The City of Cleveland issued the No Rise Certification for the proposed work within the FEMA floodplain for Doan Brook.

OES approved Section 4(f) No Use Determination based proposed work within City of Cleveland's Rockefeller Park.

ODOT addressed OHPO comments on the project design during the Cultural Resources Section 4(f) coordination.   OHPO issued a "No Adverse Effect" on historic properties.

The project type, bridge replacement, did not require air quality analysis or noise analysis.

No regulated material are identified in the project area during ESA investigation.

The project is in an urban area, therefore, no impacts to farmlands in accordance with Farmland MOU and 7 CFR 658.

The project is not within 1000-feet of national Wild or Scenic River.

There are minority and low-income populations with the project vicinity.   The EJ Form has been completed and determined that impacts to EJ populations are not anticipated.

No public controversy on environmental grounds has been received during pubic involvement activities and design.

| STIP Reference # | FY 2016-2019 STIP Statewide Line Item #16 |
| --- | --- |

| Select the appropriate project type: | (28) Bridge rehabilitation, reconstruction, or replacement or the construction of grade separation to replace existing at-grade railroad crossings, if the actions meet the constraints in 23 CFR 771.117(e). Examples include: Railroad projects that close or relocate at-grade crossings |
| --- | --- |

In accordance with 23 CFR 771.117(e), the proposed project cannot be processed as a C2 CE, if it involves -

- a. Acquisition of more than a minor amount of right-of-way

- b. Residential or non-residential displacements

- c. A Coast Guard, Individual Section 404 and/or a Section 10 permit

- d. A Section 106 finding of Adverse Effect

- e. A Section 4(f) Programmatic or Individual Evaluation

- f. A finding of May Affect, Likely to Adversely Affect to Threatened and Endangered Species

- g. Construction of temporary access, or the closure of existing road, bridge, or ramps, that would result in major traffic disruptions

- h. Changes in access control

- i. Floodplain encroachment other than functionally dependent uses (e.g., bridges, wetlands) or actions that facilitate open space use (e.g., recreational trails, bicycle and pedestrian paths)

- j. Construction activities in, across or adjacent to a river component designated or proposed for inclusion in the National System of Wild and Scenic Rivers

- k. No minor public or agency controversy on environmental grounds (no opposition from any organized groups or agencies and no unresolved environmental coordination)

- l. If an EJ Analysis Report is required, the project must be processed as a D-level CE or higher level document

For certification purposes, documentation is required to illustrate no significant impacts will occur to the following environmental resources and that no unusual circumstances exist that would warrant a higher level of NEPA document. Upload all supporting documentation to the project file.

| Waterways: | Present; No Coast Guard, Individual 404, and/or Section 10 Permit required |
| --- | --- |

| Waterways Permit Type: | Regional General Permit |
| --- | --- |

| Waterways Permit Approval Date | 02/03/2017 |
| --- | --- |

| Isolated Wetland Permit | No |
| --- | --- |

| Endangered Species: | Present; No finding of May Affect, Likely to Adversely Affect |
| --- | --- |

| Endangered Species - Coordination | May Affect, Not Likely to Adversely Affect |
| --- | --- |

| Endangered Species - Coordination Date | 10/20/2016 |
| --- | --- |

| Endangered Species - Critical Habitat Present/Impacted | Kirtland's warbler |
| --- | --- |
|  | Indiana bat |
|  | Northern long-eared bat |

| Endangered Species - Other Critical Habitat Present/Impacted: | No |
| --- | --- |

| 100-Year Floodplain: | Encroachment Within the SFHA is a Functionally Dependent Use |
| --- | --- |

| EO 11988/NFIP Coordination and Documentation Completed: | Yes |
| --- | --- |

| NFIP Local Floodplain Coordinator Notification Date: | 02/02/2017 |
| --- | --- |

| Section 4(f): | Present; No Programmatic Evaluation or Individual Evaluation Required |
| --- | --- |

| Section 4(f) Determination: | No Use |
| --- | --- |

| Section 4(f) Determination Date - No Use | 01/27/2017 |
| --- | --- |

| Section 6(f): | Not present |
| --- | --- |

| Cultural Resources: | Present; No Finding of Adverse Effect |
| --- | --- |

| Cultural Resources Coordination: | No Adverse Effect |
| --- | --- |

| Cultural Resources Coordination - SHPO Concurrence Date: | 09/19/2016 |
| --- | --- |

Projects that meet C2 criteria are not anticipated to have impacts to the following environmental resources. If resources are present, documentation is only required if there is a potential for impacts.

| Air Quality: | Studies Not Required |
| --- | --- |

| Air Quality - Coordination with OES: | No |
| --- | --- |

| Noise: | Studies Not Required |
| --- | --- |

| Noise Coordination - OES Approval Date: | 02/10/2017 |
| --- | --- |

| Hazardous Materials - ESA Screening Conducted | No ROW |
| --- | --- |

| Hazardous Materials - ODOT Approval Date: | 09/07/2016 |
| --- | --- |

| Farmland: | Urbanized Area; No Impacts in Accordance With the Farmland MOU and 7 CFR 658 |
| --- | --- |

| Scenic Rivers | No National Wild and Scenic River Within 1000 Feet of the Proposed Project Area |
| --- | --- |

Projects that meet C2 criteria must be in accordance with ODOT's EJ Guidance and activities conducted for Public Involvement are commensurate to the project's type and scope of work.

| Environmental Justice: | Does Not Exceed EJ Guidance Criteria; No EJ Analysis Report Required and No EJ Issues Raised During Public Involvement |
| --- | --- |

| Public Involvement: | Minimum PI Requirements Met; No Minor Public or Agency Controversy on Environmental Grounds |
| --- | --- |

| Environmental Commitments | Yes |
| --- | --- |

Environmental Commitments,

1. The contractor will advise the county Project Manager of the anticipated start date of any construction activities at least 14 days in advance, including but not limited to the placing of work zone signs. Throughout the duration of the project, the contractor will notify the project engineer and the others listed in this section in writing of all traffic restrictions and upcoming maintenance of traffic changes. Information will include all construction activities that impact or interfere with traffic and will list the specific location, type of work, road status, date and time of restriction, duration of restriction, number of lanes maintained, detour routes if applicable, and any other information requested by the project engineer and the District 12 Communications Office. The county will notify the public and emergency services via the media.

The contractor will notify:

- Project Manager

- Cuyahoga County Department of Public Works - Public Information Officer

- Ohio Department of Transportation District 12 - Public Information Office

- Cuyahoga County Board of Developmental Disabilities

- United States Postal Service

- Cleveland Board of Education

- City of Cleveland Police, Fire, and Service Departments

- City of Cleveland, Recreation Division Commissioner

2. Public access to all areas (except the immediate project construction area) of Rockefeller Park will be maintained during construction during normal Park hours.

3. Staging and/or storage of construction equipment will occur so as to minimize impacts to the Section 4(f) property.

4. Temporary construction fencing will be installed along proposed construction limits prior to the start of construction activities to protect the existing 4(f) property and the public.

5. Appropriate signage will be installed to alert users of the Park to the construction activities.

6. This project is located within the known habitat ranges of the federally listed and protected Indiana Bat and Northern Long-Eared Bat. No trees shall be removed under this project from April 1 through September 30. All necessary tree removal shall occur from October 1 through March 31. This requirement is necessary to avoid and minimize impacts to these species as required by the Endangered Species Act. For the purpose of this note, a tree is defined as a live, dying, or dead woody plant with a truck three inches or greater in diameter at a height of 4.5 feet above the ground surface, and with a minimum height of 13 feet.

7. This project is located within the migration range of the federally endangered Kirtland's warbler. In order to avoid impacts to any vegetation that the Kirtland's warbler would use as habitat during migration, trees and woody vegetation that is greater than 3-feet tall cannot be removed between April 22nd and June 1st or between August 15th and October 15th.

8. The project is covered by Regional General Permit Section B Maintenance.  The permit special provisions shall be adhered to and included in the project plans.

Environmental Commitments

C2

Commitments

1. The contractor will advise the county Project Manager of the anticipated start date of any construction activities at least 14 days in advance, including but not limited to the placing of work zone signs. Throughout the duration of the project, the contractor will notify the project engineer and the others listed in this section in writing of all traffic restrictions and upcoming maintenance of traffic changes. Information will include all construction activities that impact or interfere with traffic and will list the specific location, type of work, road status, date and time of restriction, duration of restriction, number of lanes maintained, detour routes if applicable, and any other information requested by the project engineer and the District 12 Communications Office. The county will notify the public and emergency services via the media.

The contractor will notify:

- Project Manager

- Cuyahoga County Department of Public Works - Public Information Officer

- Ohio Department of Transportation District 12 - Public Information Office

- Cuyahoga County Board of Developmental Disabilities

- United States Postal Service

- Cleveland Board of Education

- City of Cleveland Police, Fire, and Service Departments

- City of Cleveland, Recreation Division Commissioner

2. Public access to all areas (except the immediate project construction area) of Rockefeller Park will be maintained during construction during normal Park hours.

3. Staging and/or storage of construction equipment will occur so as to minimize impacts to the Section 4(f) property.

4. Temporary construction fencing will be installed along proposed construction limits prior to the start of construction activities to protect the existing 4(f) property and the public.

5. Appropriate signage will be installed to alert users of the Park to the construction activities.

6. This project is located within the known habitat ranges of the federally listed and protected Indiana Bat and Northern Long-Eared Bat. No trees shall be removed under this project from April 1 through September 30. All necessary tree removal shall occur from October 1 through March 31. This requirement is necessary to avoid and minimize impacts to these species as required by the Endangered Species Act. For the purpose of this note, a tree is defined as a live, dying, or dead woody plant with a truck three inches or greater in diameter at a height of 4.5 feet above the ground surface, and with a minimum height of 13 feet.

7. This project is located within the migration range of the federally endangered Kirtland's warbler. In order to avoid impacts to any vegetation that the Kirtland's warbler would use as habitat during migration, trees and woody vegetation that is greater than 3-feet tall cannot be removed between April 22nd and June 1st or between August 15th and October 15th.

8. The project is covered by Regional General Permit Section B Maintenance.  The permit special provisions shall be adhered to and included in the project plans.

Preparers  & Approvals

Form Preparer(s)

| Form Preparer | Libby Rushley Lawhon AND Associates 1441 King Avenue, Columbus OH 43212 lrushley@lawhon-assoc.com |
| --- | --- |

| Supporting Form Preparer(s) | Libby Rushley, Thomas Powell |
| --- | --- |

Approvals & Electronic Signatures

| Approved & Electronically Signed By: | Approval Date: |
| --- | --- |

| Mark Carpenter TRANS ENGR 3 | June 28, 2017 |
| --- | --- |

Appendix

Uploaded Files by Report Section

Noise

District Deter. of No Noise Analysis.pdf

Section 4(f)

Correspondence with OWJ.msg

OWJ Concurrence Request Letter.pdf

Determination Request Form for Recreational Properties.pdf

OES Recreational Determination - No Use.pdf

CUY Ansel Road PID 102532 cultural 4f excempt notificiton .pdf

General

County Map.pdf

Aerial Map.pdf

USGS Quadrangle Topographical Map.pdf

Stage 1 Design.pdf

Project Name Change Email 7-19-16.pdf

ESA

Asbestos Survey.pdf

Screening.pdf

OES Recommendations - Screening.pdf

Cultural Resources

Section 106 Request for Review.pdf

Section 106 No Adverse Effect .pdf

ODOT Determination of No Concrete Stain.pdf

Integrated Color Discussion Emails.pdf

Concrete Staining Feasibility Discussion Email .pdf

OES Response to Agency Comments - Cultural.pdf

Ecological

Level 1 ESR CUY-St Casmir Br PID 102532.pdf

Coordination with USFWS.pdf

Consult Form CUY-St Casmir Way PID102532.pdf

Other Resources

Water Source Protection Area Map.pdf

Site Specific Resource Mapping.pdf

Environmental Justice

Census Mapping.pdf

Environmental Justice Form.pdf

Public Involvement

Public Meeting Handouts.pdf

Public Meeting Exhibits.pdf

Public Meeting Summary Email.pdf

Public Meeting Agenda.pdf

Public Meeting Summary.pdf

Public Meeting Sign In Sheet.pdf

Response to Public Comments.pdf

Permits

FEMA FIRM.pdf

Permit Determination Request.pdf

Coordination with Local Floodplain Administrator.pdf

No Rise Certification - ODOT Form LD-50.pdf

Local Floodplain Administrator Approval.pdf

OES Permit Determination.pdf

Special Provisions.pdf

Regional General Permit B.pdf

Approved Documents

Approved CE.docx