Environmental Document

for

GEA US 422 13.04 Intersection PID 80963

Environmental Document Level: D1

Approved: December 20, 2016

Prepared by: District 12

The environmental review, consultation, and other actions required by applicable Federal environmental laws for this project are being, or have been, carried-out by ODOT pursuant to 23 U.S.C. 327 and a Memorandum of Understanding dated December 11, 2015, and executed by FHWA and ODOT.

Contents

Project Type	5

General Project Information	6

Project, Cost Schedule and Work Limits	6

Roadway Character	8

Maintenance of Traffic During Construction	9

Environmental Commitments, Maintenance of Traffic During Construction	10

Right of Way and Utility Involvement	10

Purpose & Need	12

Purpose & Need	12

Alternatives	15

Alternatives	15

Air	17

Mobile Source Air Toxics (MSATs)	17

Particulate Matter 2.5 (PM2.5)	17

Carbon (CO)	18

Ozone	18

Environmental Commitments	18

Noise	19

Noise	19

Environmental Commitments	19

ESA	20

ESA	20

Cultural Resources	22

Cultural Properties Present	22

Archaeological Resource Adverse Effect	23

History/Architecture Adverse Effect	23

Environmental Commitments	23

Ecological	24

ESR	24

Wetlands 	24

Streams & Rivers 	24

Other Surface Waters 	25

Terrestrial Habitat 	25

Threatened or Endangered Species 	26

Agency Coordination 	28

Environmental Commitments, Agency Coordination 	30

Other Resources	31

Farmlands	31

Drinking Water	33

Section 4(f)	34

Section 4(f) Determination	34

Identified Section 4(f) Properties	34

Section 6(f)	35

Section 6(f) Determination	35

Community Impacts	36

Community Impacts	36

Environmental Justice	38

Environmental Justice	38

Public Involvement	39

Public Involvement	39

Permits	40

Waterway Permits	40

Storm Water Permits	40

Floodplains	40

Landfills	41

Environmental Commitments	42

General, Maintenance of Traffic During Construction	42

Ecological, Agency Coordination 	42

Preparers  & Approvals	44

Form Preparer(s)	44

Approvals & Electronic Signatures	44

Appendix	45

Uploaded Files by Report Section	45

Section 6(f)	45

Purpose and Need	45

ESA	45

Cultural Resources	45

Ecological	45

Other Resources	46

Environmental Justice	46

Public Involvement	46

Project Type

| Please check all of the following actions that apply (Must check at least one): | (13) Actions described in 23 CFR 771.117 (c)(26), (c)(27), and (c)(28) that do not meet the constraints listed in 23 CFR 771.117(e). (a) Project types that exceed thresholds in Appendix A (b) Project types that exceed thresholds in Appendix B |
| --- | --- |

General Project Information

Project, Cost Schedule and Work Limits

| Environmental Document Level | D1 |
| --- | --- |

| PID | 80963 |
| --- | --- |

| Project Name | GEA US 422 13.04 Intersection |
| --- | --- |

| Project Sponsor | DISTRICT 12-TRAFFIC |
| --- | --- |

| ODOT District | 12 |
| --- | --- |

| Funding Source | Federal |
| --- | --- |

| Private Funding: | No |
| --- | --- |

| Local Public Funding: | No |
| --- | --- |

| STIP Reference #: | SLI 9, 15 & 32 |
| --- | --- |

| The next phase of the proposed project is listed on the STIP | Yes |
| --- | --- |

| The current cost estimate is in line with existing federal procedures for Ohio STIP Amendments and Administration Modifications | No |
| --- | --- |

| Planning and Engineering | $271,086.00 |
| --- | --- |

| Right of Way | $151,989.00 |
| --- | --- |

| Construction | $776,000.50 |
| --- | --- |

| Other | $0.00 |
| --- | --- |

| An Interchange Modification / Interchange Justification Study (IMS/IJS) was completed | No |
| --- | --- |

Project Description:

The project involves safety improvements at intersection of US-422 and SR-700 in Troy Township. The project will add turn lanes to US-422 east and west of the SR-700 Intersection. Additional right-of-waywill be needed from the southwest quadrant of the US 422/SR 700 Intersection. The project will require approximately 12 to 15 feet of property from the south side of US-422 for approximately 825 feet west ofthe US-422/SR-700 Intersection and 725 feet east of the US-422/SR-700 Intersection. Minor right-of-way will be needed from the northwest corner of the US-422/SR-700 Intersection to improve turning radius for trucks.

| Limits of Proposed Work | Troy Township; At SR-700 |
| --- | --- |

| Start (SLM): | 12.94 |
| --- | --- |

| End (SLM): | 13.14 |
| --- | --- |

| Total Work Length (Miles): | 0.4 |
| --- | --- |

Roadway Character

Tab Number: US00422

| Functional Classification: | Principal Arterial - Other (Rural) |
| --- | --- |

| Current Average Daily Traffic: | 15230 |
| --- | --- |

| Current Average Daily Traffic Year: | 2019 |
| --- | --- |

| Design Year Average Daily Traffic: | 17240 |
| --- | --- |

| Design Average Daily Traffic Year: | 2039 |
| --- | --- |

| Daily Hourly Volume: | 1540 |
| --- | --- |

| Truck %: | 10 |
| --- | --- |

| Setting: | Rural |
| --- | --- |

| Topography: | Rolling |
| --- | --- |

|  | Existing | Proposed |
| --- | --- | --- |
| Design Speed (MPH): | 45 | 45 |
| Legal Speed (MPH): | 45 | 45 |
| Number of Lanes: | 2 | 3 |
| Type of Lanes: | Through | Through and turning |
| Pavement Width (ft): | 22 | 36 |
| Shoulder Width (ft): | 2 | 4 |
| Median Width (ft): | 0 | 0 |
| Sidewalk Width (ft): | 0 | 0 |

No bridge data for PIDs associated with this CE Form

Maintenance of Traffic During Construction

| A roadway, bridge or ramp closure is required | No |
| --- | --- |

| A temporary bridge or roadway is proposed | No |
| --- | --- |

| A detour is required for the proposed project | No |
| --- | --- |

| The proposed MOT substantially impacts sensitive environmental resources | No |
| --- | --- |

| Substantial controversy is associated with the proposed MOT | No |
| --- | --- |

| Coordination has been initiated and/or completed with local emergency services, schools, public institutions/facilities, etc. | Yes |
| --- | --- |

Remarks:

ODOT attended a Troy Township Trustees Meeting and invited emergency services and the local school district.  The proposed  Maintenance of Traffic was discussed and no negative comments were submitted in the public comment sheets or in the meeting discussion.  See Project File/Public Information for all the Trustees' meeting information.

Under no circumstance are Temporary Right-of-way Easements to be used for storage of materials or equipment by the contractor unless noted otherwise.

| Are there any Environmental Commitments? | Yes |
| --- | --- |

Environmental Commitments, Maintenance of Traffic During Construction

1. The contractor is responsible for notifying all local, county, state and federal emergency services and every effected school facility of any anticipated road/ramp closures at least fourteen (14) days before the closure. Specifically, the road closure notice must disclose 1) the exact portion of the road to be closed 2) the date of the anticipated closure 3) the duration of the closure 4) and the detour route(s). A copy of this notification must be presented at the preconstruction meeting. The contractor is also responsible for notifying the project engineer when, including date and time the notification is distributed. A copy of the notification must be sent to ODOT communications.

2. Under no circumstance are Temporary Right-of-way Easements to be used for storage of materials or equipment by the contractor unless noted otherwise.

Right of Way and Utility Involvement

| The project requires Permanent Right-of-Way | No |
| --- | --- |

| The project requires Permanent easement(s) | Yes |
| --- | --- |

| The project requires Temporary Right-of-Way | Yes |
| --- | --- |

| Number of parcels impacted by Permanent Right-of-Way: | 4 |
| --- | --- |

| Number of parcels impacted by Temporary Right-of-Way: | 4 |
| --- | --- |

| Approximate acreage of Permanent Right-of-Way needed: | 0.237 |
| --- | --- |

| Approximate acreage of Temporary Right-of-Way needed: | 0.298 |
| --- | --- |

| Electrical lines, gas lines, water lines, sewer lines, phone lines or other utilities exist in the project area | Yes |
| --- | --- |

| Large scale transmission facilities are located within the project area | No |
| --- | --- |

| Private utility easements are located within the project area | Yes |
| --- | --- |

| The existing private utility easement will be impacted by the project | Yes |
| --- | --- |

| Coordination with identified utilities has been initiated and/or completed | Yes |
| --- | --- |

Remarks:

ODOT District 12 is currently in coordination with the utility agencies regarding the proposed project.

Purpose & Need

Purpose & Need

Project History:

TheProject area is located on US Route 422 (US-422) at its intersection with StateRoute (SR) 700 (Claridon-Troy Road) in Troy Township, Geauga, Ohio.   US-422is classified as a Rural Principal Arterial with a 2012 Average Daily Traffic(ADT) of 15,100.  The project locationwas listed in the Safe, Accountable, Flexible, Efficient Transportation EquityAct (SAFETEA-LU) in October 2006 SAFETEA-LU, as Earmark ID OH173 byRepresentative LaTourette for the construction of safety improvements at theintersection of US 422 and SR 700 in Geauga County, Ohio.  See the Earmark Status Report in the Project File/Project Documentation/Earmark Status Report.pdf..

Purpose Statement:

Thepurpose of this project is to reduce congestion and to improve safety at theintersection of US 422 and SR 700.

Need Element(s):

Congestion

Deficienciesare present at the US-422/SR-700 intersection during the afternoon peak hour, especiallyin the eastbound direction. The AM Peak Hour Traffic at the intersection is alsosubstandard with LOS D.   According to the GEA-422-10.93/13.31 CorridorStudy, the volume to capacity ratio of the EB approach is 1.10 during the PMpeak, which is 10% over capacity. The Level of Services in the EB PM Peak is anLOS F.  See Table 3 in theGEA-422-10.93/13.31 Corridor Study.  ODOT'sLocation and Design Manual Volume 1 recommend a minimum LOS C for ruralintersections.  In theGEA-422-10.93/13.31 Corridor Study, extended queues were documented on US Route422 during PM peak approaching SR-700 in the eastbound direction. These queuesextended west of the Cuyahoga River crossing (5100 feet from the SR- 700intersection).

GEA-422-10.93/13.31Corridor Study stated "Steep vertical grade of 7 percent exists along US-422approaching SR-700 in the eastbound direction. The steep grade affects trucksto have longer start up loss times from a stopped condition, resulting in muchlonger delays and travel times in that section of US Route 422."  US -422 has 13% truck traffic during peakhours.

Safety

According to the GEA-422-10.93/13.31 CorridorStudy, eighteen crashes have been recorded from 2011 to 2013 at the SR-700intersection within the study period. Crash types include nine rear-end (5 - eastbound, 4 - westbound), angle (4 crashes) and left turn (2 crashes).  These types of crashes can be attributed to congestion and the left turning vehicles are in the travel lanes. The project had been identified earlier in 2003 GEA-422 Corridor Study and later in ODOT's Safety Program Crash Priority List (2013).  See the list in Project File/Purpose and Need/Existing and Future Conditions/GEA-422-10.93 Corridor Study.pdf.  The SR-700 and US-422 Intersection was ranked #142 in the ODOT 2013 HSIP Priority Location Top 500 List.  See the list in Project File/Purpose and Need/Existing and Future Conditions/2013 HSIP Priority Location Top 500 List.pdf.

Goals and Objectives:

The Project Goal for the intersection is to improve Pedestrian Facilities.  The current configuration does not include pedestrian facilities.

Summary Statement:

The purpose of this project is to reduce congestion and to improve safety at the intersection of US 422 and SR 700.

Logical Termini and Independent Utility:

Theproject had been identified earlier in 2003 GEA-422 Corridor Study and later inODOT's Safety Program Crash Priority List (2013).  See the list in Project File/Purpose and Need/Existing and Future Conditions/2013 HSIP Priority Location Top 500 List.pdf.  See the list in Project File/Purpose and Need/Existing and Future Conditions/GEA-422-10.93 Corridor Study.pdf.  The Project is listed in the 2016-2019 STIPin State Line Item 9, 15 and 32.

Alternatives

Alternatives

Discuss No Build Alternative:

The No Build Option does not address the Congestion problems discussed in the Purpose and Need Statement.  The no build would not construct the left turn lanes and traffic on US-422 would continue to cause significant queuing  of vehicles in peak hour traffic.  The rear-end accidents, discussed in the Purpose and Need Statement,  would not decrease.  The traffic turning left on US-422 would still be in the travel lane as they wait to turn and the rear end and side swipe accidents would not be addressed.    In addition, the No-build provides no relief to the long delay time due to stopped trucks going eastbound up the 7% grade hill to SR-700.

| Was a Feasibility Study completed? | No |
| --- | --- |

| Was an Alternative Evaluation Report (AER) completed? | No |
| --- | --- |

| Name | Description | Reason Dismissed | Preferred Alternative |
| --- | --- | --- | --- |
| Build Alternative | Constructing Left turn lanes on US-422 on the eastbound and westbound lanes. | Alternative was not dismissed | Yes |

Discuss Reasons Alternative Identified as Preferred was selected:

The Build Alternative does address the Congestion problems discussed in the Purpose and Need Statement.  The Build Alternative would construct the left turn lanes and traffic on US-422 move the vehicles turning left into the dedicated left turn lane and allow traffic to move through the intersection.  This would alleviate the significant queuing  of vehicles during peak hour traffic.  The rear-end accidents, discussed in the Purpose and Need Statement,  would improve by moving the vehicles into the dedicated left turn lane and out of the travel lane.  The turn lanes would allow trucks to move through the intersection instead of waiting for vehicles to turn left and starting and stopping on an incline.

Air

Mobile Source Air Toxics (MSATs)

| Sensitive Areas are located within approximately 500' of the proposed project area | Yes |
| --- | --- |

| The proposed project is listed as a C1 in ODOT's CE Guidance and/or falls under 40 CFR 93.126 | Yes |
| --- | --- |

Remarks:

The project type is listed as exempt under CFR 93.126 as a Highway Safety Improvement Program implementation.

Particulate Matter 2.5 (PM2.5)

| The proposed project is in an air quality non-attainment or maintenance area | No |
| --- | --- |

Remarks:

Geauga County is in attainment for Particulate Matter 2.5.

Carbon (CO)

| The State of Ohio is in attainment for CO at this time and no coordination or analysis is required |  |
| --- | --- |

Ozone

| The proposed project is in an Ozone non-attainment or maintenance area | Yes |
| --- | --- |

| The proposed project is listed on the TIP | Yes |
| --- | --- |

Remarks:

Geauga County is identified as moderate for the 8-hour ozone non-attainment areas (2008 standard):

https://www3.epa.gov/airquality/greenbook/gmcs.html#OHIO

Environmental Commitments

| Are there any environmental commitments? | No |
| --- | --- |

Noise

Noise

| Noise Sensitive Areas located within approximately 500' of the proposed project area | Yes |
| --- | --- |

| Noise Analysis conducted | No |
| --- | --- |

| The proposed project is a Type I project | No |
| --- | --- |

| The proposed project is a Type II project | No |
| --- | --- |

Remarks:

The project is not a Type I project for noise, i.e. does not add capacity, a new highway on new location, significantly change the horizontal or vertical alignment, or add an auxiliary lane, therefore, a noise analysis is not required.

Environmental Commitments

| Are there any environmental commitments? | No |
| --- | --- |

ESA

ESA

| Does the project require any Permanent ROW or easement? | Yes |
| --- | --- |

| ESA Screening Report was completed by District Staff | Yes |
| --- | --- |

| Date when It was completed | 01/27/2016 |
| --- | --- |

| Date of ESA Screening IOC from OES: | 02/18/2016 |
| --- | --- |

| Do any sites require a Phase 1 ESA, Phase 2 ESA, or plan note according to the IOC from OES? | Yes |
| --- | --- |

| Site Name | Address | Phase 1 Required? | Phase 2 Required? | Plan Note Required? |
| --- | --- | --- | --- | --- |
| Site 1: PPN 32-056500 | 17740 Claridon-Troy Road, | Yes | Yes | No |
| Site 3: PPN 32-067100 | 13905 Main Market Road | Yes | No | No |

| Date of Phase 1 ESA IOC from OES: | 05/31/2016 |
| --- | --- |

| Date of Phase 2 ESA IOC from OES: | 08/22/2016 |
| --- | --- |

Remarks:

The Phase II ESA investigation for Site 3 tested the three monitoring wells on the property.  The testing detected VOC's but did not exceed drinking water standards.  OES concurred in the 8/22/16 IOC that no further ESA investigation or plan note is warranted.  See the 8/22/16 IOC from OES in the ESA/Coordination/ Phase 2 ESA IOC.pdf.

| According to the IOC from OES does the project require any Environmental Commitments (plan notes and/or other coordination)? | No |
| --- | --- |

Cultural Resources

Cultural Properties Present

Please describe all of the efforts made to identify Historic Properties (including lit review, field investigation, etc.):

ODOT District 12 Staff prepared a Section 106 Requested for Review on 3/17/15.

| Based on the letter or IOC you received from OES, is there an eligible or listed NRHP Historic Property in the Area of Potential Effects Pursuant to 36 CFR part 800? | No |
| --- | --- |

| OES/OSHPO Concurrence Date: | 01/27/2016 |
| --- | --- |

Remarks:

Per the 1/27/16 IOCfrom OES, In accordance with 36 CFR§ 800.4{d)(1) and the ProgrammaticAgreement Among The Federal Highway Administration,The Advisory Council On Historic Preservation, The Ohio Historical Society,State Historic Preservation Office, And The State of Ohio, Department ofTransportation Regarding The Implementation Of The Federal-Aid Highway Program In Ohio {Agreement No. 16734}, executed November 30, 2011, the ODOT Office of Environmental Services has determined:

No historic architectural properties will beaffected by the undertaking.

The area has been disturbed by modern development.

No further cultural resource investigations are warranted.

See the 5/19/16 IOC from OES In the Project File/ Cultural Resources/Coordination/OES Approval - No Historic Properties Affected.pdf

| What is the Section 106 effect determination in the OES letter or IOC? | No Historic Properties Affected |
| --- | --- |

Archaeological Resource Adverse Effect

History/Architecture Adverse Effect

Environmental Commitments

| Are there any Environmental Commitments? | No |
| --- | --- |

Ecological

ESR

| Has an ESR been completed? | Yes |
| --- | --- |

| Date ESR was completed: | 11/30/2015 |
| --- | --- |

Wetlands

| Are Wetlands Present? | No |
| --- | --- |

Remarks:

No wetlands were identified during the 7/13/15 ODOT District 12 field review and the 11/30/15 Ecological Survey Report.

Streams & Rivers

|  | Present |  |
| --- | --- | --- |
| Streams and Rivers: | No |  |
| National Scenic River: | No |  |
| State Scenic River: | No |  |
| Sec 9: | No |  |
| Sec 10: | No |  |

Remarks:

No streams or rivers were identified during the 7/13/15 ODOT District 12 field review and the 11/30/15 Ecological Survey Report.

Other Surface Waters

|  | Present |  |
| --- | --- | --- |
| Reservoirs: | No |  |
| Lakes: | No |  |
| Ponds: | No |  |
| Storm Water Management Facility: | No |  |
| Jurisdictional Ditch: | No |  |
| Other (If selected please explain in remarks): | No |  |

Remarks:

No reservoirs, lakes, pond, storm water management facilities or jurisdictional ditch were identified during the 7/13/15 ODOT District 12 field review and the 11/30/15 Ecological Survey Report.

Terrestrial Habitat

|  | Present | Impacted |
| --- | --- | --- |
| Terrestrial Habitat: | Yes | Yes |
| Unique or high Quality: | No |  |

Remarks:

The project will remove 0.32 acre of wooded habitat per the 11/30/15 Ecological Survey Report.

Threatened or Endangered Species

|  | Present | Impacted |
| --- | --- | --- |
| Within the known range of a Federal Species? | Yes | Yes |
| Federal Species/habitat found within the project area? | Yes | Yes |
| Within the known range of a State Species? | Yes | No |
| State Species/habitat found within the project area? | No |  |

Remarks:

Federally Listed Species and the 11/30/15 Ecological Survey Report

The closest Bald Eagle (Haliaeetus leucoocphalus) recorded nesting site was located 3.4 miles from the project area.  No impact to the Bald Eagle or its habitat is anticipated.

For all projects that will impact suitable roosting habitat for the Indiana bat Myotis sodalist and northern long-eared bat Myotis septentrionalis , USFWS has included the following comments:

Comment 1:  Clearing of all trees to be impacted by this project shall be performed only between October 1 and March 31.

Tree clearing of all trees over 3" dbh that are proposed for impact for this project will be cleared only between October 1 and March 31.

Comment 2:  For all projects that involve tree clearing:  if tree clearing is to occur prior to the issuance of the 404/401 permits, that Section 7 consultation for that project must be completed and that tree clearing cannot commence until both the USACE and Ohio EPA anticipate that the permits authorizing the project as a whole are imminent.

Tree removal will not occur until consultation with USFWS has been completed, and waterway permits (if required) have been submitted to USACE and Ohio EPA.

State Listed Species

The Prothonotary Warbler is the only cavity-nesting warbler, this species is an inhabitant of wooded swamps. Overall, this species is uncommon, and not found away from breeding sites. Their diet consists of insects and snails. This closest known sighting of the Prothonotary Warbler is 0.7 miles West of the project location. No nesting habitat for this species will be impacted by this project.

This native species of sweetflag is found in the northern U.S. and Canada. This species is found along mucky shorelines of lakes and in emergent wetlands. The more common Acorus calamus is a non-native sterile triploid.  A known sighting of the American Sweet-flag is approximately 0.7 miles Southwest of the project location in the woodlands. No wetlands are located directly adjacent to this project and no wetlands will be impacted by this project.

Agency Coordination

|  | Coordination Required | Comments Received Date |
| --- | --- | --- |
| Ohio Department of Natural Resources (ODNR): | Yes | 01/05/2016 |
| United States Fish and Wildlife Service (USFWS): | Yes | 12/31/2015 |
| Ohio Environmental Protection Agency (OEPA): | No |  |
| United States Army Corps of Engineers (USACE): | No |  |
| ODNR State Scenic River: | No |  |
| National Park Service (NPS) National Scenic River: | No |  |

Remarks:

ODNR provided the first 6 Comments for all projects in general and some of them do not apply to this project:

Comment 1:   Unavoidable impacts to streams andwetlands should be mitigated.

The project will not impact wetlands or Streams.

Comment 2: Existing riparian habitat zones should be maintainedto the maximum extent possible.

The project is not located within a riparian habitat zone.

Comment 3:  Culverts should be placed toallow free movement of aquatic fauna.

The project will not replace culverts.

Comment 4:  We recommend using nativevegetation to control erosion, or, at a minimum, using native vegetation incombination with riprap or similar materials.

The project will not install riprap or similar materials.

Comment 5:  Staging areas should be kept well away from these aquatic features.

No aquatic features are located within the project limits.

Comment 6.  All disturbed areas in theproject vicinity should be mulched and re-vegetated with native plant species.

Per ODOT Construction and Materials Specifications (104.04), allareas disturbed ground occupied by the Contractor in connection with the Work(with the exception of cultivated fields) will have suitable vegetative coverestablished by seeding and mulching according to Construction and MaterialsSpecifications Item 659.  When appropriate conditions apply, ODOT willconsider the use of native grasses and wildflowers for re-vegetating disturbedareas according to Construction and Materials Specifications Item 659.09 orsimilar seed mixes composed of native herbs and grasses.

For all projects that will impact suitable roosting habitatfor the Indiana bat and northern long-eared bat , USFWS has included thefollowing comments:

Comment 1:  Clearing of all trees to be impacted by thisproject shall be performed only between October 1 and March 31.

Tree clearing of all trees over 3" dbhthat are proposed for impact for this project will be cleared only betweenOctober 1 and March 31.

Comment 2:  For all projects that involvetree clearing:  if tree clearing is to occur prior to the issuance of the404/401 permits, that Section 7 consultation for that project must be completedand that tree clearing cannot commence until both the USACE and Ohio EPAanticipate that the permits authorizing the project as a whole are imminent.

Tree removal will not occur untilconsultation with USFWS has been completed, and waterway permits (if required)have been submitted to USACE and Ohio EPA.

| Are there any environmental commitments? | Yes |
| --- | --- |

Environmental Commitments, Agency Coordination

1. THE PROJECT IS LOCATED WITHIN THE KNOWN HABITAT RANGES OF THE FEDERALLY LISTED AND PROTECTED INDIANA BAT AND NORTHERN LONG-EARED BAT.  NO TREES SHALL BE REMOVED UNDER THIS PROJECT FROM APRIL 1 THROUGH SEPTEMBER 30.  ALL NECESSARY TREE REMOVAL SHALL OCCUR FROM OCTOBER 1 THROUGH MARCH 31.  THIS REQUIREMENT IS NECESSARY TO AVOID AND MINIMIZE IMPACTS TO THESE SPECIES AS REQUIRED BY THE ENDANGERED SPECIES ACT.  FOR THE PURPOSES OF THIS NOTE, A TREE IS DEFINED AS A LIVE, DYING, OR DEAD WOODY PLANT, WITH A TRUNK THREE INCHES OR GREATER IN DIAMETER AT A HEIGHT OF 4.5 FEET ABOVE THE GROUND SURFACE, AND WITH A MINIMUM HEIGHT OF 13 FEET.

2. All consultation under Section 7(a) of the ESA will be concluded and waterway permits (if applicable) will be submitted to the appropriate agencies prior to any tree-clearing for the project.

3. Per ODOT CMS (104.04), all areas of disturbed ground occupied by the Contractor in connection with the work (with the exception of cultivated fields) will have suitable vegetative cover established by seeding and mulching according to CMS (Item 659).  When appropriate conditions apply, ODOT will consider the use of native grasses and wildflowers for revegetating disturbed areas according to CMS (Item 659.09) or similar seed mixes composed of native herbs and grasses.

Other Resources

Farmlands

| The proposed project is located within an Urbanized Area | No |
| --- | --- |

| The proposed project is located within a Non-Urbanized Area | Yes |
| --- | --- |

| The proposed project involves new permanent right-of-way (ROW) | Yes |
| --- | --- |

| The proposed project involves temporary ROW | Yes |
| --- | --- |

| The proposed project is a type of action listed below and meets allowable ROW thresholds: - Bridge replacement requiring ROW of three (3) acres or less to accommodate bridge piers, wingwalls, and/or approach work - Widening requiring linear strip ROW of 10 acres or less per linear mile - Intersection improvement requiring ROW of three (3) acres or less | Yes |
| --- | --- |

| Undeveloped land being used for construction purposes is protected by a conservation easement | No |
| --- | --- |

| Undeveloped land being used for temporary ROW will be returned to equal or greater productive capability upon completion of the proposed project | Yes |
| --- | --- |

| Based on the scope and type of work, the proposed project is in compliance with the executed Farmland Memorandum of Understanding (MOU), the FPPA, and 7 CFR 658. Completion of the Farmland Conversion Impact Rating (FCIR) Form is not warranted. No further coordination is required. |  |
| --- | --- |

| OES Concurrence Date |  |
| --- | --- |

| NRCS Approval Date |  |
| --- | --- |

| Land being used for the proposed project is part of an Agricultural District | No |
| --- | --- |

Remarks:

Based on the scope and type of work, the proposed project is in compliance with the executed Farmland Memorandum of Understanding (MOU), the FPPA, and 7 CFR 658. Completion of the Farmland Conversion Impact Rating (FCIR) Form is not warranted. No further coordination is required.

| Are there any environmental commitments? | No |
| --- | --- |

Drinking Water

| The proposed project is wholly or partially located within the USEPA designated boundaries of a Sole Source Aquifer | No |
| --- | --- |

|  | Present |  |
| --- | --- | --- |
| The proposed project is wholly or partially located within the OEPA designated boundaries of a Source Water Protection Area | No |  |

| Coordination with the Local Public Water Administrator is required | No |
| --- | --- |

|  | Present |  |
| --- | --- | --- |
| Residential Wells are present | No |  |

Remarks:

The proposed project is not within the U.S. EPA Sole Source Aquifer area.  The project is located near two OEPA Source Water Protection Area.  However the project is over 300 and 400feet from two OEPA Source Water Protection Areas.  See the Drinking Water Resource Map in the Project File/Drinking Water/Water Source Protection Area Map.pdf

| Are there any environmental commitments? |  |
| --- | --- |

Section 4(f)

Section 4(f) Determination

| Section 4(f) properties are within and/or adjacent to the proposed project area | No |
| --- | --- |

|  | Present |  |
| --- | --- | --- |
| Publicly owned Park(s): |  |  |
| Publicly owned recreation facility(ies) and/or area(s): |  |  |
| Wildlife and Waterfowl refuge(s): |  |  |
| Historic Site(s): |  |  |

Identified Section 4(f) Properties

Remarks:

The closest Section 4(f) property is the ODNR public Hunting Area located over 1,000 feet west of the project intersection.  The  project will not impact the property nor restrict access to the site.

| Are there any environmental commitments? | No |
| --- | --- |

Section 6(f)

Section 6(f) Determination

|  | Present |  |
| --- | --- | --- |
| 6(f) Properties: | No |  |

Remarks:

No section 6(f) sites were identified on the NRCS LWCF List.  See the attached list in the Project File/Section6(f)/Project Information/LWCF Grant Listing.pdf

| Are there any environmental commitments? | No |
| --- | --- |

Community Impacts

Community Impacts

| Will the proposed action comply with the local/regional development patterns for the area? | Yes |
| --- | --- |

Remarks:

The proposed project is consistent with local plans and is not expected to change development patterns in the area. The project is listed in the Northeast Ohio Areawide Coordinating Agency (NOACA), the Metropolitan Planning Organization for the region, FY 2016-2019 Transportation Improvement Program.

| Will the proposed action result in substantial negative impacts to community cohesion? | No |
| --- | --- |

Remarks:

The proposed project will not negatively impact community cohesion.

| Will the proposed action result in indirect or cumulative impacts? | No |
| --- | --- |

Remarks:

The proposed project will not cause any indirect or cumulative impacts.

| Will the proposed action result in substantial impacts on health and educational facilities, public utilities, fire, police, emergency services, religious institutions, public transportation, pedestrian and bicycle facilities? | No |
| --- | --- |

Remarks:

Substantial impacts will not occur to health and educational facilities, public utilities, fire, police, emergency services, religious institutions, or public transportation facilities.  Emergency services and schools will be notified of any delays and closures associated with construction.  A Maintenance of Traffic Plan has been developed for this project that maintains interstate access during construction. Residents of the project area may experience short-term impacts during construction; however, safety for the public utilizing this facility will be improved in the long run.

THE CONTRACTOR WILL NOTIFY THE LOCAL PUBLIC SERVICES,SCHOOL DISTRICT, EMERGENCY SERVICES AND COMMUNITY A MINIMUM OF FOURTEEN (14) DAYS IN ADVANCE OF PROJECT CONSTRUCTION.  INCLUDED IN THE NOTIFICATION WILL BE THE PROJECTED DATES OF ANY LANE RESTRICTIONS, ROADWAY CLOSURE(S) AND DETOUR ROUTE(S).  Commitment is included in the General Commitments.

| Will the proposed action displace residents, businesses, institutions or farms? | No |
| --- | --- |

Remarks:

The project will not result in any displacements or relocation.

| Are there any Environmental Commitments? | No |
| --- | --- |

Environmental Justice

Environmental Justice

| Census Block Group # | % Minority | % Low Income |
| --- | --- | --- |
| 39055312000 | 2.19 | 15.64 |

| Are there block groups that have minority and/or low-income populations that are equal to or greater than 40%? | No |
| --- | --- |

Remarks:

The proposed project will haveno disproportionately high and adverse impacts to minority or low-incomepopulations based upon the table above, the attached mapping, and the answersto the questions above. No Environmental Justice issues were raised as a resultof public involvement activities conducted as part of the proposed project.Therefore, in accordance with the protections of Executive Order 12898 and FHWAOrder 6640.23A, no further Environmental Justice analysis is required.

| Are there any Environmental Commitments? | No |
| --- | --- |

Public Involvement

Public Involvement

Please provide a summary of the Public Involvement activities that have been conducted for this project. (For example press releases, letters to affected property owners and residents, meetings, special purpose meetings, newspaper articles, etc)

ODOT sent Public Notification Letters to residents on October 14, 2015 to inform them about field reviews.  ODOT requested to attend the Troy Township Trustee Meeting on 4/05/16 at the Troy Community House.  Meeting notification letters were sent on 3/21/16 to residents near the project area.  Brian Blayney and Thomas Sorge presented the project description and environmental impacts at the meeting.  See the Public information documentation in Project File/Public Involvement.

| Was Public Involvement conducted in compliance with Title VI requirements? | Yes |
| --- | --- |

| Is there any substantial environmental controversy on environmental grounds? | No |
| --- | --- |

Please summarize the Public Involvement responses received.

The Troy Township Trustees' Meeting was successful in informing the residents about the project.  Several residents asked about the right-of-way impacts to their property and ODOT personnel explained the right-of-way impacts to each resident after the meeting.  Most of the residents were concerned about the traffic movements through the intersection, specifically the left turning movements on US-422 onto SR-700.  One email comments were received on 9/26/16 and a response was sent 9/26/16.  ODOT Personnel distributed comment sheets at the meeting.  However, no comment sheets were received.  Please see the response letter in the Public Involvement/Project Information/Response to Public Comments.pdf.

| Are there any Environmental Commitments? | No |
| --- | --- |

Permits

Waterway Permits

| Are Waterway Permits required? | No |
| --- | --- |

ODNR

| Shore Structure Permit : | No |
| --- | --- |

Remarks:

The project does not require Waterway Permits.

| Are there any environmental commitments? | No |
| --- | --- |

Storm Water Permits

| NPDES Construction General Permit for Stormwater (NOI): | No |
| --- | --- |

| Watershed Specific NPDES Construction General Permit for Stormwater (NOI): | No |
| --- | --- |

Remarks:

The project does not require a NPDES Permit.

| Are there any environmental commitments? | No |
| --- | --- |

Floodplains

| The proposed project involves encroachment within a Special Flood Hazard Area (SFHA) | No |
| --- | --- |

Remarks:

The project is not located within a FEMA regulated Floodplain.

| Are there any environmental commitments? | No |
| --- | --- |

Landfills

| Is a 2713 Permit required? | No |
| --- | --- |

Remarks:

The ESA investigations did not identify any landfills within the project area.

| Are there any environmental commitments? | No |
| --- | --- |

Environmental Commitments

General, Maintenance of Traffic During Construction

Commitments

1. The contractor is responsible for notifying all local, county, state and federal emergency services and every effected school facility of any anticipated road/ramp closures at least fourteen (14) days before the closure. Specifically, the road closure notice must disclose 1) the exact portion of the road to be closed 2) the date of the anticipated closure 3) the duration of the closure 4) and the detour route(s). A copy of this notification must be presented at the preconstruction meeting. The contractor is also responsible for notifying the project engineer when, including date and time the notification is distributed. A copy of the notification must be sent to ODOT communications.

2. Under no circumstance are Temporary Right-of-way Easements to be used for storage of materials or equipment by the contractor unless noted otherwise.

Ecological, Agency Coordination

Commitments

1. THE PROJECT IS LOCATED WITHIN THE KNOWN HABITAT RANGES OF THE FEDERALLY LISTED AND PROTECTED INDIANA BAT AND NORTHERN LONG-EARED BAT.  NO TREES SHALL BE REMOVED UNDER THIS PROJECT FROM APRIL 1 THROUGH SEPTEMBER 30.  ALL NECESSARY TREE REMOVAL SHALL OCCUR FROM OCTOBER 1 THROUGH MARCH 31.  THIS REQUIREMENT IS NECESSARY TO AVOID AND MINIMIZE IMPACTS TO THESE SPECIES AS REQUIRED BY THE ENDANGERED SPECIES ACT.  FOR THE PURPOSES OF THIS NOTE, A TREE IS DEFINED AS A LIVE, DYING, OR DEAD WOODY PLANT, WITH A TRUNK THREE INCHES OR GREATER IN DIAMETER AT A HEIGHT OF 4.5 FEET ABOVE THE GROUND SURFACE, AND WITH A MINIMUM HEIGHT OF 13 FEET.

2. All consultation under Section 7(a) of the ESA will be concluded and waterway permits (if applicable) will be submitted to the appropriate agencies prior to any tree-clearing for the project.

3. Per ODOT CMS (104.04), all areas of disturbed ground occupied by the Contractor in connection with the work (with the exception of cultivated fields) will have suitable vegetative cover established by seeding and mulching according to CMS (Item 659).  When appropriate conditions apply, ODOT will consider the use of native grasses and wildflowers for revegetating disturbed areas according to CMS (Item 659.09) or similar seed mixes composed of native herbs and grasses.

Preparers  & Approvals

Form Preparer(s)

| Form Preparer | District 12 |
| --- | --- |

| Supporting Form Preparer(s) | Thomas Sorge |
| --- | --- |

Approvals & Electronic Signatures

| Approved & Electronically Signed By: | Approval Date: |
| --- | --- |

| Mark Carpenter TRANS ENGR 3 | December 20, 2016 |
| --- | --- |

Appendix

Uploaded Files by Report Section

Section 6(f)

LWCF Grant Listing.pdf

Purpose and Need

GEA-422-10.93 Corridor Study.pdf

2013 HSIP Priority Location Top 500 List.xlsx

Earmark Status Report.pdf

ESA

Screening.pdf

Phase I .pdf

OES Recommendations - Phase I.pdf

Monitoring Well Sampling Report.pdf

OES Recommendations - Phase II.pdf

OES Recommendations - Screening.pdf

Cultural Resources

Section 106 Request for Review.pdf

OES Approval - No Historic Properties Affected.pdf

Figure 1 CR IOC .pdf

Figure 2 CR IOC .pdf

Figure 3 CR IOC .pdf

Ecological

Coordination with USFWS.pdf

Coordination with ODNR.pdf

USFWS Comments.pdf

Level 1 Ecological Survey.pdf

OES Response to USFWS Comments.msg

ODNR Comments.pdf

Other Resources

Water Source Protection Area Map.pdf

Environmental Justice

Census Mapping.pdf

Public Involvement

Property Owner Notification Letter.pdf

Stakeholder Comments.msg

Public Meeting Notification.pdf

Public Meeting Handouts.pdf

Public Meeting Comment Sheet.docx

Public Meeting Exhibits.pdf

Response to Public Comments.pdf