Environmental Document

for

POR Liberty Street Bridge PID 101973

Environmental Document Level: C2

Approved: February 7, 2017

Prepared by: Brian Peck

ODOT DISTRICT 4

Phone:  330-786-4931

e-mail:

The environmental review, consultation, and other actions required by applicable Federal environmental laws for this project are being, or have been, carried-out by ODOT pursuant to 23 U.S.C. 327 and a Memorandum of Understanding dated December 11, 2015, and executed by FHWA and ODOT.

Contents

C2	3

Environmental Commitments, 	9

Environmental Commitments	12

C2	12

Preparers  & Approvals	14

Form Preparer(s)	14

Approvals & Electronic Signatures	14

Appendix	15

Uploaded Files by Report Section	15

General	15

ESA	15

Cultural Resources	15

Ecological	15

Environmental Justice	15

Public Involvement	15

Permits	16

C2

| PID | 101973 |
| --- | --- |

| Project Sponsor | PORTAGE COUNTY ENGINEER |
| --- | --- |

| ODOT District | 4 |
| --- | --- |

| Funding Source | Federal |
| --- | --- |

| Private Funding: | No |
| --- | --- |

Project Description:

The Portage County Engineer proposes to replace thestructurally deficient and functionally obsolete Liberty Street Bridge overCamp Creek in the village of Garrettsville, Portage County, Ohio.   The project includes roadway approach workand relocation of affected utilities in the vicinity of the structure.  Theproject will be constructed within the existing roadway right-of-way.  Theproposed new abutments will be placed behind the existing abutments that willbe cut to allow for new beam placement. No work is proposed below the Ordinary High Water Mark (OHWM) of CampCreek.

Excavation, grading or filling operations shall not beperformed in any wetlands, streams or other waters of the United States.

A total of 0.02 acre of potential suitable wooded habitat (SWH) was identified within the project construction limits and will be impacted by the project. SWH areas are characterized as forested areas >0.5 acre or areas <0.5 acre that had trees with suitable bat roosting characteristics and also within 1,000 feet of larger forested areas. This project meet the Non-Notifying Project type outlined in accordance with Section IIB2 of the Ecological MOA with regard to the Ohio Department of Natural Resources (ODNR), and does not require ODNR review. ODOT-OES provided project documentation to the U.S. Fish and Wildlife Service (USFWS) on December 30, 2016.  This project meets Consultation Category 1 (CC1) of the Federal Highway Administration (FHWA), Federal Railroad Administration (FRA), and Federal Transit Administration (FTA) Range-wide Programmatic Informal Consultation for Indiana Bat and Northern Long-eared Bat (referred to as the RW PC), and programmatic concurrence from the USFWS on a determination of May Effect, Not Likely To Adversely Affect (MANLAA).  The project will affect no other federally listed species.  On January 31, 2017, USFWS provided written concurrence for this project that meets CC1.

The Liberty Street Bridge will be closed for a maximum of approximately 90 consecutive days during construction.  A local detour route will be posted and access shall be maintained to all adjacent properties during construction.  The Portage County Engineer's Office released a news release for the project on July 15, 2016, and November 8, 2016.  Project maintenance of traffic information was submitted to local emergency and public services (public/private EMS, police, fire, schools, etc.) by electronic mail correspondence, dated July 6, 2016, and November 8, 2016.  Public comments were not received as a result of the public involvement activities conducted for the project.

| STIP Reference # | Statewide Line Item (SLI) Number 16 and is fiscally constrained. |
| --- | --- |

| Select the appropriate project type: | (28) Bridge rehabilitation, reconstruction, or replacement or the construction of grade separation to replace existing at-grade railroad crossings, if the actions meet the constraints in 23 CFR 771.117(e). Examples include: Railroad projects that close or relocate at-grade crossings |
| --- | --- |

In accordance with 23 CFR 771.117(e), the proposed project cannot be processed as a C2 CE, if it involves -

- a. Acquisition of more than a minor amount of right-of-way

- b. Residential or non-residential displacements

- c. A Coast Guard, Individual Section 404 and/or a Section 10 permit

- d. A Section 106 finding of Adverse Effect

- e. A Section 4(f) Programmatic or Individual Evaluation

- f. A finding of May Affect, Likely to Adversely Affect to Threatened and Endangered Species

- g. Construction of temporary access, or the closure of existing road, bridge, or ramps, that would result in major traffic disruptions

- h. Changes in access control

- i. Floodplain encroachment other than functionally dependent uses (e.g., bridges, wetlands) or actions that facilitate open space use (e.g., recreational trails, bicycle and pedestrian paths)

- j. Construction activities in, across or adjacent to a river component designated or proposed for inclusion in the National System of Wild and Scenic Rivers

- k. No minor public or agency controversy on environmental grounds (no opposition from any organized groups or agencies and no unresolved environmental coordination)

- l. If an EJ Analysis Report is required, the project must be processed as a D-level CE or higher level document

For certification purposes, documentation is required to illustrate no significant impacts will occur to the following environmental resources and that no unusual circumstances exist that would warrant a higher level of NEPA document. Upload all supporting documentation to the project file.

| Waterways: | Present; No Coast Guard, Individual 404, and/or Section 10 Permit required |
| --- | --- |

| Waterways Permit Type: |  |
| --- | --- |

| Isolated Wetland Permit | No |
| --- | --- |

| Endangered Species: | Present; No finding of May Affect, Likely to Adversely Affect |
| --- | --- |

| Endangered Species - Coordination | May Affect, Not Likely to Adversely Affect |
| --- | --- |

| Endangered Species - Coordination Date | 12/30/2016 |
| --- | --- |

| Endangered Species - Critical Habitat Present/Impacted | Indiana bat |
| --- | --- |
|  | Northern long-eared bat |

| Endangered Species - Other Critical Habitat Present/Impacted: | No |
| --- | --- |

| 100-Year Floodplain: | Encroachment Within the SFHA is a Functionally Dependent Use |
| --- | --- |

| EO 11988/NFIP Coordination and Documentation Completed: | Yes |
| --- | --- |

| NFIP Local Floodplain Coordinator Notification Date: | 12/14/2016 |
| --- | --- |

| Section 4(f): | Not present |
| --- | --- |

| Section 6(f): | Not present |
| --- | --- |

| Cultural Resources: | Present; No Finding of Adverse Effect |
| --- | --- |

| Cultural Resources Coordination: | No Historic Properties Affected |
| --- | --- |

| Cultural Resources Coordination - SHPO Concurrence Date: | 01/26/2016 |
| --- | --- |

Projects that meet C2 criteria are not anticipated to have impacts to the following environmental resources. If resources are present, documentation is only required if there is a potential for impacts.

| Air Quality: | Studies Not Required |
| --- | --- |

| Air Quality - Coordination with OES: | No |
| --- | --- |

| Noise: | Studies Not Required |
| --- | --- |

| Noise Coordination - OES Approval Date: | 01/01/1001 |
| --- | --- |

| Hazardous Materials - ESA Screening Conducted | Yes |
| --- | --- |

| Hazardous Materials - OES Approval Date: | 08/24/2016 |
| --- | --- |

| Phase I ESA Warranted Based on Coordination with OES: | No Further Studies Warranted |
| --- | --- |

| Farmland: | Urbanized Area; No Impacts in Accordance With the Farmland MOU and 7 CFR 658 |
| --- | --- |

| Scenic Rivers | No National Wild and Scenic River Within 1000 Feet of the Proposed Project Area |
| --- | --- |

Projects that meet C2 criteria must be in accordance with ODOT's EJ Guidance and activities conducted for Public Involvement are commensurate to the project's type and scope of work.

| Environmental Justice: | Does Not Exceed EJ Guidance Criteria; No EJ Analysis Report Required and No EJ Issues Raised During Public Involvement |
| --- | --- |

| Public Involvement: | Minimum PI Requirements Met; No Minor Public or Agency Controversy on Environmental Grounds |
| --- | --- |

| Environmental Commitments | Yes |
| --- | --- |

Environmental Commitments,

1.

The contractor will advise the Project Engineer a minimum of twenty-one (21) days prior to the start of construction activities.  The contractor must also provide notification to the Project Engineer a minimum of twenty-one (21) days prior to any lane restrictions or closures.  The Project Engineer will forward this information to the Portage County Engineer's Office for use to notify emergency services and the public a minimum of  fourteen (14) days prior to the start of project construction.  Included in this notification will be the proposed lane restrictions, road closures and detours required by the project.

2. No excavation, grading, or filling operations shall be performed in any wetlands, streams or other water of the United States.  Under no circumstances shall the contractor store equipment and/or materials in any wetlands, streams or other waters of the United States.  The contractor shall not perform any work below the ordinary high water mark of Camp Creek (elevation 966.8 feet), including construction of any temporary access fill or other activities that require equipment or personnel to encroach on the stream.  A quantity of Item 832 Perimeter Filter Fabric Fence and 832 Construction Fence shall be included in the plan to demarcate and protect Camp Creek.

3. THE PROJECT IS LOCATED WITHIN THE KNOWN HABITAT RANGES OF THE FEDERALLY LISTED AND PROTECTED INDIANA BAT AND NORTHERN LONG-EARED BAT.  NO TREES SHALL BE REMOVED UNDER THIS PROJECT FROM APRIL 1 THROUGH SEPTEMBER 30.  ALL NECESSARY TREE REMOVAL SHALL OCCUR FROM OCTOBER 1 THROUGH MARCH 31.  THIS REQUIREMENT IS NECESSARY TO AVOID AND MINIMIZE IMPACTS TO THESE SPECIES AS REQUIRED BY THE ENDANGERED SPECIES ACT.  FOR THE PURPOSES OF THIS NOTE, A TREE IS DEFINED AS A LIVE, DYING, OR DEAD WOODY PLANT, WITH A TRUNK THREE INCHES OR GREATER IN DIAMETER AT A HEIGHT OF 4.5 FEET ABOVE THE GROUND SURFACE, AND WITH A MINIMUM HEIGHT OF 13 FEET.

4. THE UNDERSIDE OF LIBERTY STREET BRIDGE SHALL BE CAREFULLY EXAMINED FOR THE PRESENCE OF BATS, ESPECIALLY FORM APRIL 1 TO SEPTEMBER 30.  IF ANY BATS ARE FOUND ROOSTING ON THE UNDERSIDE OF THE BRIDGE, THE USFWS, ECOLOGICAL SERVICES DIVISION (614-416-8993), ODOT OFFICE OF ENVIRONMENTAL SERVICES (614-466-7880) AND ODOT DISTRICT 4 ENVIRONMENTAL SECTION (330-786-4930) SHALL BE CONTACTED TO PROVIDE THIS INFORMATION.  SHOULD ADDITIONAL INFORMATION ON LISTED ENDANGERED/THREATENED/POTENTIALLY THREATENED SPECIES OR THEIR CRITICAL HABITAT BECOME AVAILABLE, OR IF NEW INFORMATION REVEALS EFFECTS OF THIS PROJECT THAT WERE NOT PREVIOUSLY CONSIDERED, ODOT WILL REINITIATE CONSULTATION WITH THE USFWS AND ODNR TO ASSESS WHETHER THE PROJECT DETERMINATIONS ARE STILL VALID.

5. Water column and sedimentation impacts shall be kept to a minimum through use of best management practices for soil erosion and sedimentation control.  All soil erosion and sediment control measures shall be in place prior to any excavation, grading or filling operations and installation of proposed structures or utilities. They shall remain in place until construction is completed and the area is stabilized as accepted by the engineer. These shall comply with ODOT's Handbook for Sediment and Erosion Control, which may be found at: http://www.dot.state.oh.us/drrc.

6. The contractor shall take all precautions to avoid and/or limit construction and demolition debris from entering the stream.  Any debris that does fall into the stream shall be removed as soon as possible.

7. The contractor shall take all precautions necessary to prevent epoxy-urethane sealer, paint or other materials used to repair, clean, paint, seal or treat any structure from entering any streams, wetlands or other waters of the United States and take the appropriate actions in the event of a release.

8. NATURAL BUFFERS ADJACENT TO STREAMS AND WETLANDS SHALL BE PRESERVED TO THE MAXIMUM EXTENT PRACTICABLE.  EXISTING RIPARIAN HABITATS SHOULD BE MAINTAINED TO THE MAXIMUM EXTENT PRACTICABLE AND EQUIPMENT STAGING AREAS SHALL BE KEPT WELL AWAY FROM STREAMS AND WETLANDS TO THE EXTENT PRACTICABLE.  ODOT CONSTRUCTION AND MATERIALS SPECIFICATIONS SECTION 107.10 (PROTECTION AND RESTORATION OF PROPERTY) PROHIBIT THE CONTRACTOR FROM CREATING STAGING AREAS NEAR STREAMS/WETLANDS.

Environmental Commitments

C2

Commitments

1.

The contractor will advise the Project Engineer a minimum of twenty-one (21) days prior to the start of construction activities.  The contractor must also provide notification to the Project Engineer a minimum of twenty-one (21) days prior to any lane restrictions or closures.  The Project Engineer will forward this information to the Portage County Engineer's Office for use to notify emergency services and the public a minimum of  fourteen (14) days prior to the start of project construction.  Included in this notification will be the proposed lane restrictions, road closures and detours required by the project.

2. No excavation, grading, or filling operations shall be performed in any wetlands, streams or other water of the United States.  Under no circumstances shall the contractor store equipment and/or materials in any wetlands, streams or other waters of the United States.  The contractor shall not perform any work below the ordinary high water mark of Camp Creek (elevation 966.8 feet), including construction of any temporary access fill or other activities that require equipment or personnel to encroach on the stream.  A quantity of Item 832 Perimeter Filter Fabric Fence and 832 Construction Fence shall be included in the plan to demarcate and protect Camp Creek.

3. THE PROJECT IS LOCATED WITHIN THE KNOWN HABITAT RANGES OF THE FEDERALLY LISTED AND PROTECTED INDIANA BAT AND NORTHERN LONG-EARED BAT.  NO TREES SHALL BE REMOVED UNDER THIS PROJECT FROM APRIL 1 THROUGH SEPTEMBER 30.  ALL NECESSARY TREE REMOVAL SHALL OCCUR FROM OCTOBER 1 THROUGH MARCH 31.  THIS REQUIREMENT IS NECESSARY TO AVOID AND MINIMIZE IMPACTS TO THESE SPECIES AS REQUIRED BY THE ENDANGERED SPECIES ACT.  FOR THE PURPOSES OF THIS NOTE, A TREE IS DEFINED AS A LIVE, DYING, OR DEAD WOODY PLANT, WITH A TRUNK THREE INCHES OR GREATER IN DIAMETER AT A HEIGHT OF 4.5 FEET ABOVE THE GROUND SURFACE, AND WITH A MINIMUM HEIGHT OF 13 FEET.

4. THE UNDERSIDE OF LIBERTY STREET BRIDGE SHALL BE CAREFULLY EXAMINED FOR THE PRESENCE OF BATS, ESPECIALLY FORM APRIL 1 TO SEPTEMBER 30.  IF ANY BATS ARE FOUND ROOSTING ON THE UNDERSIDE OF THE BRIDGE, THE USFWS, ECOLOGICAL SERVICES DIVISION (614-416-8993), ODOT OFFICE OF ENVIRONMENTAL SERVICES (614-466-7880) AND ODOT DISTRICT 4 ENVIRONMENTAL SECTION (330-786-4930) SHALL BE CONTACTED TO PROVIDE THIS INFORMATION.  SHOULD ADDITIONAL INFORMATION ON LISTED ENDANGERED/THREATENED/POTENTIALLY THREATENED SPECIES OR THEIR CRITICAL HABITAT BECOME AVAILABLE, OR IF NEW INFORMATION REVEALS EFFECTS OF THIS PROJECT THAT WERE NOT PREVIOUSLY CONSIDERED, ODOT WILL REINITIATE CONSULTATION WITH THE USFWS AND ODNR TO ASSESS WHETHER THE PROJECT DETERMINATIONS ARE STILL VALID.

5. Water column and sedimentation impacts shall be kept to a minimum through use of best management practices for soil erosion and sedimentation control.  All soil erosion and sediment control measures shall be in place prior to any excavation, grading or filling operations and installation of proposed structures or utilities. They shall remain in place until construction is completed and the area is stabilized as accepted by the engineer. These shall comply with ODOT's Handbook for Sediment and Erosion Control, which may be found at: http://www.dot.state.oh.us/drrc.

6. The contractor shall take all precautions to avoid and/or limit construction and demolition debris from entering the stream.  Any debris that does fall into the stream shall be removed as soon as possible.

7. The contractor shall take all precautions necessary to prevent epoxy-urethane sealer, paint or other materials used to repair, clean, paint, seal or treat any structure from entering any streams, wetlands or other waters of the United States and take the appropriate actions in the event of a release.

8. NATURAL BUFFERS ADJACENT TO STREAMS AND WETLANDS SHALL BE PRESERVED TO THE MAXIMUM EXTENT PRACTICABLE.  EXISTING RIPARIAN HABITATS SHOULD BE MAINTAINED TO THE MAXIMUM EXTENT PRACTICABLE AND EQUIPMENT STAGING AREAS SHALL BE KEPT WELL AWAY FROM STREAMS AND WETLANDS TO THE EXTENT PRACTICABLE.  ODOT CONSTRUCTION AND MATERIALS SPECIFICATIONS SECTION 107.10 (PROTECTION AND RESTORATION OF PROPERTY) PROHIBIT THE CONTRACTOR FROM CREATING STAGING AREAS NEAR STREAMS/WETLANDS.

Preparers  & Approvals

Form Preparer(s)

| Form Preparer | Brian Peck ODOT DISTRICT 4 330-786-4931 Brian.Peck@dot.ohio.gov |
| --- | --- |

| Supporting Form Preparer(s) | Brian Peck, Chad Root, Jon Hunt, Anthony Zumbo |
| --- | --- |

Approvals & Electronic Signatures

| Approved & Electronically Signed By: | Approval Date: |
| --- | --- |

| Edward Deley PROGRAM ADMIN 3 | February 7, 2017 |
| --- | --- |
| Edward Deley PROGRAM ADMIN 3 | February 7, 2017 |

Appendix

Uploaded Files by Report Section

General

Preliminary Site Plan.pdf

Preliminary Roadway Plan.pdf

ESA

Screening.pdf

Asbestos Survey.pdf

OES Recommendations - Screening.pdf

Cultural Resources

Section 106 Scoping Request.pdf

OES Approval - No Historic Properties Affected.pdf

Ecological

USFWS Comments and Response Information.pdf

Level 1 Ecological Survey.pdf

Coordination with USFWS.pdf

USFWS Comments.pdf

Environmental Justice

Census Mapping.pdf

Environmental Justice Form.pdf

Public Involvement

Property Owner Notification Letter.pdf

Press Release.pdf

EMS PARTA and Local School Notification.pdf

Press Release - Second Release.pdf

EMS and Public Services Contact List.pdf

Permits

FEMA FIRM.pdf

Coordination with Local Floodplain Administrator.pdf