Environmental Document

for

TRU SR 0005 00.00 PID 81641

Environmental Document Level: C2

Approved: September 26, 2017

Prepared by: District 4

The environmental review, consultation, and other actions required by applicable Federal environmental laws for this project are being, or have been, carried-out by ODOT pursuant to 23 U.S.C. 327 and a Memorandum of Understanding dated December 11, 2015, and executed by FHWA and ODOT.

Contents

C2	3

Environmental Commitments, 	11

Environmental Commitments	16

C2	16

Preparers  & Approvals	21

Form Preparer(s)	21

Approvals & Electronic Signatures	21

Appendix	22

Uploaded Files by Report Section	22

General	22

ESA	22

Cultural Resources	22

Ecological	22

Other Resources	22

Environmental Justice	22

Public Involvement	22

Permits	23

C2

| PID | 81641 |
| --- | --- |

| Project Sponsor | ODOT SPONSORING AGENCY |
| --- | --- |

| ODOT District | 4 |
| --- | --- |

| Funding Source | Federal |
| --- | --- |

| Private Funding: | No |
| --- | --- |

Project Description:

The OhioDepartment of Transportation (ODOT) plans to improve State Route (SR) 5 inBraceville and Warren Townships, Trumbull County, Ohio.  The project proposes to resurface 6.43 milesof SR 5, including partial-depth pavement repairs, from Straight Line Mile(SLM) 0.00 to SLM 6.43.  Additionally,the project proposes minor maintenance on three (3) structures, Structure No.TRU-5-2.71/Structure File Number (SFN) 7800045, Structure No. TRU-5-3.38L/SFN7800134 and Structure No. TRU-5-3.38R/SFN 7800150 and the replacement of one(1) structure, Structure No. TRU-5-4.27/Culvert File Number (CFN) 780050250along SR 5 as follows:

TRU-5-2.71(unnamed tributary to Mahoning River): The project proposes minor maintenance to the existing 16' concretefilled culvert conveying an unnamed tributary to the Mahoning River under SR 5.  Maintenance improvements include:  clearing/grubbing of vegetation within 15' ofthe structure and new structure identification signs.

TRU-5-3.38L(Mahoning River):  The project proposesminor maintenance to the existing 210' continuous steel beam bridge spanningthe Mahoning River.  Maintenanceimprovements include:  sealing concretewearing surface and approach slabs with gravity fed resin concretetreatment,  sealing concrete surfaces ofthe parapets and wing walls with epoxy-urethane, clearing/grubbing ofvegetation within 15' of the structure and new structure identification signs.

TRU-5-3.38R(Mahoning River):  The project proposesminor maintenance to the existing 210' continuous steel beam bridge spanningthe Mahoning River.  Maintenanceimprovements include:  sealing concretewearing surface and approach slabs with gravity fed resin concrete treatment,patching concrete substructure and parapets, removing all spalled areas fromthe bottom of the deck floor, sealingrepaired concrete surfaces with epoxy-urethane, repairing forward and rearapproach slab ends, clearing/grubbing of vegetation within 15' of the structureand new structure identification signs.

TRU-5-4.27(Ditch1):  The project proposes toreplace the existing dual 43" x 27" corrugated metal pipe culvertconveying Ditch 1 under SR 5.  The existingstructure will be removed and replaced with a dual 43" conduit, Type A,culvert.  Rock Channel Protection (RCP),Type C, will be placed on the inlet and outlet sides of the new structure.

Stage 3Plans can be found in the Project File/General/Project Information subsection asStage 3 Design.pdf.

Newpermanent right-of-way and/or temporary right-of-way is not required toconstruct this project.  Existingutilities within the project study area will not be impacted/relocated toconstruct the project.

The maximumimpact to Ditch 1 and Ditch 2 below the Ordinary High Water Mark (OHWM) isanticipated to be 0.06 ac.  Additionally,a total of 0.0025 ac. of suitable wooded habitat (SWH) for theFederally-listed Indiana Bat (Myotis sodalis) and Northern Long-Eared Bat(Myotis septentrionalis) will be impacted by the project. The project will notimpact any other environmentally sensitive resources within the project studyarea.  The explanation of SWH impacts canbe found in the Level 1 Ecological Survey Report (ESR) in the Project File/Ecological/Reports subsection as Level 1 Ecological Survey.pdf while theexplanation of the impacts to Ditch 1 and Ditch 2 can found in the PermitDetermination Request in the Project File/Permits/Waterways subsection as Permit DeterminationRequest.pdf.

On August 7,2017, and August 29, 2017,  ODOT-District4 conducted various public involvement activities including a news release,District website posting, and emergency and public service notifications.  One (1) comment was received concerning theproject schedule.  ODOT-District 4responded to this comment on September 6, 2017. All public involvementdocumentation for the project can be found in the Project File/Public Involvement/Project Information subsection.

| STIP Reference # | Statewide Line Item (SLI) Number 32 and is fiscally constrained. |
| --- | --- |

| Select the appropriate project type: | (26) Modernization of a highway by resurfacing, restoration, rehabilitation, reconstruction, adding shoulders, or adding auxiliary lanes (including parking, weaving, turning, and climbing lanes), if the action meets the constraints in paragraph (e) of this section. Examples include: Joint or limited use of right-of-way where the proposed use would have minimal or no adverse social (including highway safety), economic or environmental impacts; Installation of new noise walls and other new noise mitigation projects; Construction of highway safety and truck escape ramps; Construction of bicycle lanes and pedestrian walkways, sidewalks, shared-use paths, or facilities and trailhead parking that do not otherwise qualify for a C1 designation; Beautification or facility improvement projects (i.e. landscaping, curb and gutter installation and replacement, ADA ramps/curb ramps, installation of park benches, decorative lighting, etc.); Construction of alternative energy facilities (fuel tank farms, wind turbines, etc.) |
| --- | --- |
|  | (28) Bridge rehabilitation, reconstruction, or replacement or the construction of grade separation to replace existing at-grade railroad crossings, if the actions meet the constraints in 23 CFR 771.117(e). Examples include: Railroad projects that close or relocate at-grade crossings |

In accordance with 23 CFR 771.117(e), the proposed project cannot be processed as a C2 CE, if it involves -

- a. Acquisition of more than a minor amount of right-of-way

- b. Residential or non-residential displacements

- c. A Coast Guard, Individual Section 404 and/or a Section 10 permit

- d. A Section 106 finding of Adverse Effect

- e. A Section 4(f) Programmatic or Individual Evaluation

- f. A finding of May Affect, Likely to Adversely Affect to Threatened and Endangered Species

- g. Construction of temporary access, or the closure of existing road, bridge, or ramps, that would result in major traffic disruptions

- h. Changes in access control

- i. Floodplain encroachment other than functionally dependent uses (e.g., bridges, wetlands) or actions that facilitate open space use (e.g., recreational trails, bicycle and pedestrian paths)

- j. Construction activities in, across or adjacent to a river component designated or proposed for inclusion in the National System of Wild and Scenic Rivers

- k. No minor public or agency controversy on environmental grounds (no opposition from any organized groups or agencies and no unresolved environmental coordination)

- l. If an EJ Analysis Report is required, the project must be processed as a D-level CE or higher level document

For certification purposes, documentation is required to illustrate no significant impacts will occur to the following environmental resources and that no unusual circumstances exist that would warrant a higher level of NEPA document. Upload all supporting documentation to the project file.

| Waterways: | Present; No Coast Guard, Individual 404, and/or Section 10 Permit required |
| --- | --- |

| Waterways Permit Type: | Regional General Permit |
| --- | --- |

| Waterways Permit Approval Date | 09/11/2017 |
| --- | --- |

| Isolated Wetland Permit | No |
| --- | --- |

| Endangered Species: | Present; No finding of May Affect, Likely to Adversely Affect |
| --- | --- |

| Endangered Species - Coordination | May Affect, Not Likely to Adversely Affect |
| --- | --- |

| Endangered Species - Coordination Date | 06/30/2017 |
| --- | --- |

| Endangered Species - Critical Habitat Present/Impacted | Indiana bat |
| --- | --- |
|  | Northern long-eared bat |

| Endangered Species - Other Critical Habitat Present/Impacted: | No |
| --- | --- |

| 100-Year Floodplain: | Encroachment Within the SFHA is a Functionally Dependent Use |
| --- | --- |

| EO 11988/NFIP Coordination and Documentation Completed: | Yes |
| --- | --- |

| NFIP Local Floodplain Coordinator Notification Date: | 09/14/2017 |
| --- | --- |

| Section 4(f): | Not present |
| --- | --- |

| Section 6(f): | Not present |
| --- | --- |

| Cultural Resources: | Present; No Finding of Adverse Effect |
| --- | --- |

| Cultural Resources Coordination: | Minimum Potential to Cause Effect - Appendix A |
| --- | --- |

| Cultural Resources Coordination - ODOT Approval Date: | 08/07/2017 |
| --- | --- |

Projects that meet C2 criteria are not anticipated to have impacts to the following environmental resources. If resources are present, documentation is only required if there is a potential for impacts.

| Air Quality: | Studies Not Required |
| --- | --- |

| Air Quality - Coordination with OES: | No |
| --- | --- |

| Noise: | Studies Not Required |
| --- | --- |

| Noise Coordination - OES Approval Date: | 09/14/2017 |
| --- | --- |

| Hazardous Materials - ESA Screening Conducted | Yes |
| --- | --- |

| Hazardous Materials - OES Approval Date: | 08/04/2017 |
| --- | --- |

| Phase I ESA Warranted Based on Coordination with OES: | No Further Studies Warranted |
| --- | --- |

| Farmland: | Non-Urbanized Area; No Impacts in Accordance With the Farmland MOU, 7 CFR 658, and ORC 929.05 |
| --- | --- |

| Scenic Rivers | No National Wild and Scenic River Within 1000 Feet of the Proposed Project Area |
| --- | --- |

Projects that meet C2 criteria must be in accordance with ODOT's EJ Guidance and activities conducted for Public Involvement are commensurate to the project's type and scope of work.

| Environmental Justice: | Does Not Exceed EJ Guidance Criteria; No EJ Analysis Report Required and No EJ Issues Raised During Public Involvement |
| --- | --- |

| Public Involvement: | Minimum PI Requirements Met; No Minor Public or Agency Controversy on Environmental Grounds |
| --- | --- |

| Environmental Commitments | Yes |
| --- | --- |

Environmental Commitments,

1. The project designer shall incorporate the following note into the plans:

ENDANGERED SPECIES HABITAT-INDIANA BAT/NORTHERN LONG-EARED BAT:

THE TRU-5-4.27 STRUCTURE IS LOCATED WITHIN THE KNOWN HABITAT RANGES OF THE FEDERALLY LISTED AND PROTECTED INDIANA BAT AND NORTHERN LONG-EARED BAT.  NO TREES SHALL BE REMOVED AT THIS LOCATION FROM APRIL 1 THROUGH SEPTEMBER 30.  ALL NECESSARY TREE REMOVAL SHALL OCCUR FROM OCTOBER 1 THROUGH MARCH 31.  THIS REQUIREMENT IS NECESSARY TO AVOID AND MINIMIZE IMPACTS TO THESE SPECIES AS REQUIRED BY THE ENDANGERED SPECIES ACT.  FOR THE PURPOSES OF THIS NOTE, A TREE IS DEFINED AS A LIVE, DYING, OR DEAD WOODY PLANT, WITH A TRUNK THREE INCHES OR GREATER IN DIAMETER AT A HEIGHT OF 4.5 FEET ABOVE THE GROUND SURFACE, AND WITH A MINIMUM HEIGHT OF 13 FEET.

2. The project designer shall incorporate the following note into the plans:

MATERIALS REMOVED FROM DITCHES, STREAMS AND/OR WETLANDS: THE CONTRACTOR SHALL TAKE ALL PRECAUTIONS NECESSARY TO PREVENT ANY INCIDENTAL DISCHARGES ASSOCIATED WITH THE EXCAVATION AND HAULING OF MATERIAL FROM STREAMS AND DITCHES.  THIS PERTAINS TO ANY EXCAVATION OPERATION SUCH AS, FOUNDATION PIER OR ABUTMENT EXCAVATION, CHANNEL CLEAN OUT, EXCAVATION FOR ROCK CHANNEL PROTECTION AND REMOVAL OF ANY TEMPORARY FILL ASSOCIATED WITH CONSTRUCTION OPERATIONS.

ALL MATERIALS REMOVED FROM THE DITCHES, STREAMS AND/OR WETLANDS MUST BE IMMEDIATELY REMOVED TO AN UPLAND SITE AND STABILIZED (I.E., SEEDED) TO PREVENT REDISTRIBUTION INTO ANY WATERS OF THE UNITED STATES.  IMMEDIATE REMOVAL IS DEFINED BY THE UNITED STATES ARMY CORPS OF ENGINEERS AS DEPOSITING THE REMOVED MATERIALS DIRECTLY INTO A TRUCK AND REMOVING THE MATERIAL FROM THE SITE; PLACEMENT OF REMOVED MATERIALS INTO A WETLAND OR ON THE BANKS OF A STREAM EVEN TEMPORARILY IS CONSIDERED A FILL AND REQUIRES A PERMIT ACTION.  ANY AREAS DISTURBED BY EQUIPMENT ACTIVITIES MUST BE SEEDED WITH NATIVE SPECIES TO PREVENT EROSION OF SEDIMENTS INTO WATERS OF THE UNITED STATES.

3. The project designer shall incorporate the following note into the plans:

STRUCTURE PAINTING/CONCRETE SEALING OPERATIONS:

THE CONTRACTOR SHALL TAKE ALL PRECAUTIONS NECESSARY TO PREVENT EPOXY-URETHANE SEALER, PAINT OR OTHER MATERIALS USED TO REPAIR, CLEAN, PAINT, SEAL OR TREAT ANY STRUCTURE FROM ENTERING ANY STREAMS, WETLANDS OR OTHER WATERS OF THE UNITED STATES AND TAKE THE APPROPRIATE ACTIONS IN THE EVENT OF A RELEASE.

4. The project designer shall incorporate the following note into the plans:

DITCH IMPACTS/AVOIDANCE:

THE PROJECT WILL IMPACT AN ESTIMATED 0.06 ACRE OF JURISDICTIONAL DITCH.  THE FOLLOWING DITCHES WILL BE IMPACTED BY THIS PROJECT:

DITCH 1 - 0.052 ACRE

DITCH 2 - 0.008 ACRE

UNDER NO CIRCUMSTANCES SHALL THE CONTRACTOR IMPACT THE REMAINING AREA OF DITCH 1 AND DITCH 2 DELINEATED BEYOND THE PROJECT CONSTRUCTION LIMITS AND DEPICTED IN THE ECOLOGICAL RESOURCES MAP AND THE TRU-5-4.27 PLAN AND PROFILE.  THE ECOLOGICAL RESOURCES MAP IS AVAILABLE FOR INSPECTION AT THE ODOT DISTRICT 4 OFFICE, 2088 SOUTH ARLINGTON, AKRON, OHIO 44306.  UNDER NO CIRCUMSTANCES SHALL THE CONTRACTOR STORE EQUIPMENT AND/OR MATERIALS WITHIN THESE DITCH AREAS. TO PROTECT AND DELINEATE THE BOUNDARY OF THE EXISTING REMAINING AREAS OF DITCH 1 AND DITCH 2, A FILTER FABRIC FENCE AND TEMPORARY CONSTRUCTION FENCE, PER SUPPLEMENTAL SPECIFICATION 832, SHALL BE INSTALLED ADJACENT TO THE DITCH AREAS BY THE CONTRACTOR, MAINTAINING A ONE-FOOT BUFFER BETWEEN THE FENCE AND THE WETLAND BOUNDARIES (WHEN PRACTICABLE), PRIOR TO THE START OF ANY CONSTRUCTION ACTIVITIES, INCLUDING ANY NECESSARY CLEARING AND GRUBBING ACTIVITIES, AND MAINTAINED BY THE CONTRACTOR THROUGHOUT PROJECT CONSTRUCTION.  UNDER NO CIRCUMSTANCES SHALL THE CONTRACTOR STORE CONSTRUCTION EQUIPMENT AND/OR MATERIALS IN ANY DITCHES, ETC. ODOT CONSTRUCTION AND MATERIALS SPECIFICATIONS SECTION 107.10 (PROTECTION AND RESTORATION OF PROPERTY) PROHIBIT THE CONTRACTOR FROM CREATING STAGING AREAS NEAR STREAMS AND/OR WETLANDS.

5. The project designer shall incorporate the following note into the plans:

MECHANICAL EQUIPMENT OPERATION AT STREAM CHANNEL:

THE MECHANICAL EQUIPMENT USED TO EXECUTE THE WORK AUTHORIZED HEREIN SHALL BE OPERATED IN SUCH A WAY AS TO MINIMIZE TURBIDITY THAT COULD DEGRADE WATER QUALITY AND ADVERSELY AFFECT AQUATIC PLANT AND ANIMAL LIFE.

6. The project designer shall incorporate the following note into the plans:

WETLAND AVOIDANCE:

NO EXCAVATION, GRADING OR FILLING OPERATIONS SHALL BE PERFORMED IN THE WETLANDS DELINEATED BEYOND THE PROJECT CONSTRUCTION LIMITS AND DEPICTED IN THE ECOLOGICAL RESOURCES MAP AND THE TRU-5-4.27 PLAN AND PROFILE.  THE ECOLOGICAL RESOURCES MAP IS AVAILABLE FOR INSPECTION AT THE ODOT DISTRICT 4 OFFICE, 2088 SOUTH ARLINGTON, AKRON, OHIO 44306.  TO PROTECT AND DELINEATE THE BOUNDARIES OF THE WETLANDS, A FILTER FABRIC FENCE AND TEMPORARY CONSTRUCTION FENCE, PER SUPPLEMENTAL SPECIFICATION 832, SHALL BE INSTALLED AT THE PROPOSED CONSTRUCTION LIMITS BY THE CONTRACTOR, MAINTAINING A ONE-FOOT BUFFER BETWEEN THE FENCE AND THE WETLAND BOUNDARIES (WHEN PRACTICABLE), PRIOR TO THE START OF ANY CONSTRUCTION ACTIVITIES, INCLUDING ANY NECESSARY CLEARING AND GRUBBING ACTIVITIES, AND MAINTAINED BY THE CONTRACTOR THROUGHOUT PROJECT CONSTRUCTION. UNDER NO CIRCUMSTANCES SHALL THE CONTRACTOR STORE CONSTRUCTION EQUIPMENT AND/OR MATERIALS IN ANY WETLANDS, ETC. ODOT CONSTRUCTION AND MATERIALS SPECIFICATIONS SECTION 107.10 (PROTECTION AND RESTORATION OF PROPERTY) PROHIBIT THE CONTRACTOR FROM CREATING STAGING AREAS NEAR STREAMS AND/OR WETLANDS.  ADDITIONALLY, THE WETLANDS HAVE BEEN LABELED "DO NOT DISTURB" TO ENSURE COMPLIANCE WITH ALL APPLICABLE ENVIRONMENTAL LAWS AND REGULATIONS.

7. The project designer shall incorporate the following note into the plans:

CONSTRUCTION AND DEMOLITION DEBRIS:

THE CONTRACTOR SHALL TAKE PRECAUTIONS TO AVOID AND/OR LIMIT CONSTRUCTION AND DEMOLITION DEBRIS FROM ENTERING DITCHES, WETLANDS AND/OR STREAM(S).  ANY DEBRIS THAT DOES FALL INTO DITCHES, WETLANDS AND/OR STREAM(S) SHALL BE REMOVED AS SOON AS POSSIBLE WITHIN 72 HOURS.

8. The project designer shall incorporate the following note into the plans:

STREAM/RIVER AVOIDANCE:

NO EXCAVATION, GRADING OR FILLING OPERATIONS SHALL BE PERFORMED IN THE STREAM/RIVER LOCATED AT TRU-5-2.71 AND TRU-5-3.38L/R.  UNDER NO CIRCUMSTANCES SHALL THE CONTRACTOR STORE CONSTRUCTION EQUIPMENT AND/OR MATERIALS IN THESE WATERS.  ODOT CONSTRUCTION AND MATERIALS SPECIFICATIONS SECTION 107.10 (PROTECTION AND RESTORATION OF PROPERTY) PROHIBIT THE CONTRACTOR FROM CREATING STAGING AREAS NEAR STREAMS AND/OR WETLANDS.

9. The project designer shall incorporate the following note into the plans:

TREE CUTTING/REMOVAL PROHIBITED:

THE TRU-5-2.71 AND TRU-5-3.38L/R STRUCTURES ARE LOCATED WITHIN THE KNOWN HABITAT RANGES OF THE FEDERALLY LISTED AND PROTECTED INDIANA BAT AND NORTHERN LONG-EARED BAT.  TREE TRIMMING IS PERMITTED FOR THESE LOCATIONS AS DIRECTED BY THE PROJECT ENGINEER, HOWEVER, NO TREES SHALL BE REMOVED AT THESE LOCATIONS.  A TREE IS DEFINED AS A LIVE, DYING, OR DEAD WOODY PLANT, WITH A TRUNK THREE INCHES OR GREATER IN DIAMETER AT A HEIGHT OF 4.5 FEET ABOVE THE GROUND SURFACE, AND WITH A MINIMUM HEIGHT OF 13 FEET.

Environmental Commitments

C2

Commitments

1. The project designer shall incorporate the following note into the plans:

ENDANGERED SPECIES HABITAT-INDIANA BAT/NORTHERN LONG-EARED BAT:

THE TRU-5-4.27 STRUCTURE IS LOCATED WITHIN THE KNOWN HABITAT RANGES OF THE FEDERALLY LISTED AND PROTECTED INDIANA BAT AND NORTHERN LONG-EARED BAT.  NO TREES SHALL BE REMOVED AT THIS LOCATION FROM APRIL 1 THROUGH SEPTEMBER 30.  ALL NECESSARY TREE REMOVAL SHALL OCCUR FROM OCTOBER 1 THROUGH MARCH 31.  THIS REQUIREMENT IS NECESSARY TO AVOID AND MINIMIZE IMPACTS TO THESE SPECIES AS REQUIRED BY THE ENDANGERED SPECIES ACT.  FOR THE PURPOSES OF THIS NOTE, A TREE IS DEFINED AS A LIVE, DYING, OR DEAD WOODY PLANT, WITH A TRUNK THREE INCHES OR GREATER IN DIAMETER AT A HEIGHT OF 4.5 FEET ABOVE THE GROUND SURFACE, AND WITH A MINIMUM HEIGHT OF 13 FEET.

2. The project designer shall incorporate the following note into the plans:

MATERIALS REMOVED FROM DITCHES, STREAMS AND/OR WETLANDS: THE CONTRACTOR SHALL TAKE ALL PRECAUTIONS NECESSARY TO PREVENT ANY INCIDENTAL DISCHARGES ASSOCIATED WITH THE EXCAVATION AND HAULING OF MATERIAL FROM STREAMS AND DITCHES.  THIS PERTAINS TO ANY EXCAVATION OPERATION SUCH AS, FOUNDATION PIER OR ABUTMENT EXCAVATION, CHANNEL CLEAN OUT, EXCAVATION FOR ROCK CHANNEL PROTECTION AND REMOVAL OF ANY TEMPORARY FILL ASSOCIATED WITH CONSTRUCTION OPERATIONS.

ALL MATERIALS REMOVED FROM THE DITCHES, STREAMS AND/OR WETLANDS MUST BE IMMEDIATELY REMOVED TO AN UPLAND SITE AND STABILIZED (I.E., SEEDED) TO PREVENT REDISTRIBUTION INTO ANY WATERS OF THE UNITED STATES.  IMMEDIATE REMOVAL IS DEFINED BY THE UNITED STATES ARMY CORPS OF ENGINEERS AS DEPOSITING THE REMOVED MATERIALS DIRECTLY INTO A TRUCK AND REMOVING THE MATERIAL FROM THE SITE; PLACEMENT OF REMOVED MATERIALS INTO A WETLAND OR ON THE BANKS OF A STREAM EVEN TEMPORARILY IS CONSIDERED A FILL AND REQUIRES A PERMIT ACTION.  ANY AREAS DISTURBED BY EQUIPMENT ACTIVITIES MUST BE SEEDED WITH NATIVE SPECIES TO PREVENT EROSION OF SEDIMENTS INTO WATERS OF THE UNITED STATES.

3. The project designer shall incorporate the following note into the plans:

STRUCTURE PAINTING/CONCRETE SEALING OPERATIONS:

THE CONTRACTOR SHALL TAKE ALL PRECAUTIONS NECESSARY TO PREVENT EPOXY-URETHANE SEALER, PAINT OR OTHER MATERIALS USED TO REPAIR, CLEAN, PAINT, SEAL OR TREAT ANY STRUCTURE FROM ENTERING ANY STREAMS, WETLANDS OR OTHER WATERS OF THE UNITED STATES AND TAKE THE APPROPRIATE ACTIONS IN THE EVENT OF A RELEASE.

4. The project designer shall incorporate the following note into the plans:

DITCH IMPACTS/AVOIDANCE:

THE PROJECT WILL IMPACT AN ESTIMATED 0.06 ACRE OF JURISDICTIONAL DITCH.  THE FOLLOWING DITCHES WILL BE IMPACTED BY THIS PROJECT:

DITCH 1 - 0.052 ACRE

DITCH 2 - 0.008 ACRE

UNDER NO CIRCUMSTANCES SHALL THE CONTRACTOR IMPACT THE REMAINING AREA OF DITCH 1 AND DITCH 2 DELINEATED BEYOND THE PROJECT CONSTRUCTION LIMITS AND DEPICTED IN THE ECOLOGICAL RESOURCES MAP AND THE TRU-5-4.27 PLAN AND PROFILE.  THE ECOLOGICAL RESOURCES MAP IS AVAILABLE FOR INSPECTION AT THE ODOT DISTRICT 4 OFFICE, 2088 SOUTH ARLINGTON, AKRON, OHIO 44306.  UNDER NO CIRCUMSTANCES SHALL THE CONTRACTOR STORE EQUIPMENT AND/OR MATERIALS WITHIN THESE DITCH AREAS. TO PROTECT AND DELINEATE THE BOUNDARY OF THE EXISTING REMAINING AREAS OF DITCH 1 AND DITCH 2, A FILTER FABRIC FENCE AND TEMPORARY CONSTRUCTION FENCE, PER SUPPLEMENTAL SPECIFICATION 832, SHALL BE INSTALLED ADJACENT TO THE DITCH AREAS BY THE CONTRACTOR, MAINTAINING A ONE-FOOT BUFFER BETWEEN THE FENCE AND THE WETLAND BOUNDARIES (WHEN PRACTICABLE), PRIOR TO THE START OF ANY CONSTRUCTION ACTIVITIES, INCLUDING ANY NECESSARY CLEARING AND GRUBBING ACTIVITIES, AND MAINTAINED BY THE CONTRACTOR THROUGHOUT PROJECT CONSTRUCTION.  UNDER NO CIRCUMSTANCES SHALL THE CONTRACTOR STORE CONSTRUCTION EQUIPMENT AND/OR MATERIALS IN ANY DITCHES, ETC. ODOT CONSTRUCTION AND MATERIALS SPECIFICATIONS SECTION 107.10 (PROTECTION AND RESTORATION OF PROPERTY) PROHIBIT THE CONTRACTOR FROM CREATING STAGING AREAS NEAR STREAMS AND/OR WETLANDS.

5. The project designer shall incorporate the following note into the plans:

MECHANICAL EQUIPMENT OPERATION AT STREAM CHANNEL:

THE MECHANICAL EQUIPMENT USED TO EXECUTE THE WORK AUTHORIZED HEREIN SHALL BE OPERATED IN SUCH A WAY AS TO MINIMIZE TURBIDITY THAT COULD DEGRADE WATER QUALITY AND ADVERSELY AFFECT AQUATIC PLANT AND ANIMAL LIFE.

6. The project designer shall incorporate the following note into the plans:

WETLAND AVOIDANCE:

NO EXCAVATION, GRADING OR FILLING OPERATIONS SHALL BE PERFORMED IN THE WETLANDS DELINEATED BEYOND THE PROJECT CONSTRUCTION LIMITS AND DEPICTED IN THE ECOLOGICAL RESOURCES MAP AND THE TRU-5-4.27 PLAN AND PROFILE.  THE ECOLOGICAL RESOURCES MAP IS AVAILABLE FOR INSPECTION AT THE ODOT DISTRICT 4 OFFICE, 2088 SOUTH ARLINGTON, AKRON, OHIO 44306.  TO PROTECT AND DELINEATE THE BOUNDARIES OF THE WETLANDS, A FILTER FABRIC FENCE AND TEMPORARY CONSTRUCTION FENCE, PER SUPPLEMENTAL SPECIFICATION 832, SHALL BE INSTALLED AT THE PROPOSED CONSTRUCTION LIMITS BY THE CONTRACTOR, MAINTAINING A ONE-FOOT BUFFER BETWEEN THE FENCE AND THE WETLAND BOUNDARIES (WHEN PRACTICABLE), PRIOR TO THE START OF ANY CONSTRUCTION ACTIVITIES, INCLUDING ANY NECESSARY CLEARING AND GRUBBING ACTIVITIES, AND MAINTAINED BY THE CONTRACTOR THROUGHOUT PROJECT CONSTRUCTION. UNDER NO CIRCUMSTANCES SHALL THE CONTRACTOR STORE CONSTRUCTION EQUIPMENT AND/OR MATERIALS IN ANY WETLANDS, ETC. ODOT CONSTRUCTION AND MATERIALS SPECIFICATIONS SECTION 107.10 (PROTECTION AND RESTORATION OF PROPERTY) PROHIBIT THE CONTRACTOR FROM CREATING STAGING AREAS NEAR STREAMS AND/OR WETLANDS.  ADDITIONALLY, THE WETLANDS HAVE BEEN LABELED "DO NOT DISTURB" TO ENSURE COMPLIANCE WITH ALL APPLICABLE ENVIRONMENTAL LAWS AND REGULATIONS.

7. The project designer shall incorporate the following note into the plans:

CONSTRUCTION AND DEMOLITION DEBRIS:

THE CONTRACTOR SHALL TAKE PRECAUTIONS TO AVOID AND/OR LIMIT CONSTRUCTION AND DEMOLITION DEBRIS FROM ENTERING DITCHES, WETLANDS AND/OR STREAM(S).  ANY DEBRIS THAT DOES FALL INTO DITCHES, WETLANDS AND/OR STREAM(S) SHALL BE REMOVED AS SOON AS POSSIBLE WITHIN 72 HOURS.

8. The project designer shall incorporate the following note into the plans:

STREAM/RIVER AVOIDANCE:

NO EXCAVATION, GRADING OR FILLING OPERATIONS SHALL BE PERFORMED IN THE STREAM/RIVER LOCATED AT TRU-5-2.71 AND TRU-5-3.38L/R.  UNDER NO CIRCUMSTANCES SHALL THE CONTRACTOR STORE CONSTRUCTION EQUIPMENT AND/OR MATERIALS IN THESE WATERS.  ODOT CONSTRUCTION AND MATERIALS SPECIFICATIONS SECTION 107.10 (PROTECTION AND RESTORATION OF PROPERTY) PROHIBIT THE CONTRACTOR FROM CREATING STAGING AREAS NEAR STREAMS AND/OR WETLANDS.

9. The project designer shall incorporate the following note into the plans:

TREE CUTTING/REMOVAL PROHIBITED:

THE TRU-5-2.71 AND TRU-5-3.38L/R STRUCTURES ARE LOCATED WITHIN THE KNOWN HABITAT RANGES OF THE FEDERALLY LISTED AND PROTECTED INDIANA BAT AND NORTHERN LONG-EARED BAT.  TREE TRIMMING IS PERMITTED FOR THESE LOCATIONS AS DIRECTED BY THE PROJECT ENGINEER, HOWEVER, NO TREES SHALL BE REMOVED AT THESE LOCATIONS.  A TREE IS DEFINED AS A LIVE, DYING, OR DEAD WOODY PLANT, WITH A TRUNK THREE INCHES OR GREATER IN DIAMETER AT A HEIGHT OF 4.5 FEET ABOVE THE GROUND SURFACE, AND WITH A MINIMUM HEIGHT OF 13 FEET.

Preparers  & Approvals

Form Preparer(s)

| Form Preparer | District 4 |
| --- | --- |

| Supporting Form Preparer(s) | Brian Ross, Matthew Chaney |
| --- | --- |

Approvals & Electronic Signatures

| Approved & Electronically Signed By: | Approval Date: |
| --- | --- |

| Edward Deley PROGRAM ADMIN 3 | September 26, 2017 |
| --- | --- |
| Edward Deley PROGRAM ADMIN 3 | September 26, 2017 |

Appendix

Uploaded Files by Report Section

General

Stage 3 Design.pdf

USGS Quadrangle Topographical Map.pdf

ESA

Regulated Materials Review Form.pdf

District Determination - No ESA Studies Required.pdf

Cultural Resources

Records Check.pdf

Buckeye Assets Mapping.pdf

Ecological

Level 1 ESR TRU-5-0.00 PID 81641.pdf

Coordination with USFWS.pdf

USFWS Comments.pdf

Ecological Review Form - Ecologically Exempt Project.pdf

Other Resources

Water Source Protection Area Map.pdf

Environmental Justice

Underserved Populations Documentation.pdf

Census Mapping.pdf

Project Related OES Decision - Environmental Justice.pdf

Public Involvement

Press Release.pdf

Emergency and Public Services Contact List.pdf

Emergency and Public Services Notification.pdf

Public Comments Received.pdf

Response to Public Comments.pdf

District Website Posting.pdf

Permits

Reginal General Permit.pdf

OES Permit Determination.pdf

Special Provisions.pdf

FEMA FIRM.pdf

Project Manager Floodplain Notification.pdf

Public Explanation of Floodplain Impacts.pdf

Permit Determination Request.pdf

District Determination - No Floodplain Impacts.pdf