Environmental Document

for

TRU TR 261 Corinth Court Rd PID 97634

Environmental Document Level: C2

Approved: May 19, 2016

Prepared by: Jennifer Walker

Burgess  AND  Niple Inc.

Phone:  614-459-2050

e-mail:

The environmental review, consultation, and other actions required by applicable Federal environmental laws for this project are being, or have been, carried-out by ODOT pursuant to 23 U.S.C. 327 and a Memorandum of Understanding dated December 11, 2015, and executed by FHWA and ODOT.

Contents

C2	3

Environmental Commitments, 	8

Environmental Commitments	12

C2	12

Preparers  & Approvals	16

Form Preparer(s)	16

Approvals & Electronic Signatures	16

Appendix	17

Uploaded Files by Report Section	17

General	17

Alternatives	17

ESA	17

Cultural Resources	17

Ecological	17

Other Resources	17

Environmental Justice	18

Public Involvement	18

Permits	18

C2

| PID | 97634 |
| --- | --- |

| Project Sponsor | TRUMBULL COUNTY ENGINEER |
| --- | --- |

| ODOT District | 4 |
| --- | --- |

| Funding Source | Federal, State, Other |
| --- | --- |

| Private Funding: | No |
| --- | --- |

Project Description:

The Trumbull County Engineer proposes to replace theexisting Corinth Court Road bridge (TR-261, Structure File Number (SFN)7832818) over Sugar Creek in Johnston and Vernon Townships, Trumbull County,Ohio.The existing structure, a singlespan steel pony truss bridge with concrete/stone abutments, will be replacedwith a 60-foot single span steel beam bridge with a reinforced concrete deck onsemi-integral abutments on spread footings. The project includes reconstruction of approximately 320 feet ofapproach roadway (135 feet on the south side and 185 feet on the north side ofthe bridge) to transition to the new structure. Rock channel protection (RCP) will be required at the northwest andsouthwest quadrants of the new bridge crossing over Sugar Creek to protect theslopes from erosion.  Temporaryright-of-way (R/W) will be required to construct the project.

During construction, 86 linear feet of Sugar Creek,  0.01 ac. of Category 2 wetlands (Wetland B) and one (1) potential roost tree for the Indiana Bat (Myotis sodalis) and Northern Long Eared Bat (Myotis septentrionalis) are proposed to be impacted by the project.

| STIP Reference # | 2016stipID0219FDPE/2016stipID0219FDRW/2016stipID0219FDCO and is fiscally constrained. |
| --- | --- |

| Select the appropriate project type: | (28) Bridge rehabilitation, reconstruction, or replacement or the construction of grade separation to replace existing at-grade railroad crossings, if the actions meet the constraints in 23 CFR 771.117(e). Examples include: Railroad projects that close or relocate at-grade crossings |
| --- | --- |

In accordance with 23 CFR 771.117(e), the proposed project cannot be processed as a C2 CE, if it involves -

- a. Acquisition of more than a minor amount of right-of-way

- b. Residential or non-residential displacements

- c. A Coast Guard, Individual Section 404 and/or a Section 10 permit

- d. A Section 106 finding of Adverse Effect

- e. A Section 4(f) Programmatic or Individual Evaluation

- f. A finding of May Affect, Likely to Adversely Affect to Threatened and Endangered Species

- g. Construction of temporary access, or the closure of existing road, bridge, or ramps, that would result in major traffic disruptions

- h. Changes in access control

- i. Floodplain encroachment other than functionally dependent uses (e.g., bridges, wetlands) or actions that facilitate open space use (e.g., recreational trails, bicycle and pedestrian paths)

- j. Construction activities in, across or adjacent to a river component designated or proposed for inclusion in the National System of Wild and Scenic Rivers

- k. No minor public or agency controversy on environmental grounds (no opposition from any organized groups or agencies and no unresolved environmental coordination)

- l. If an EJ Analysis Report is required, the project must be processed as a D-level CE or higher level document

For certification purposes, documentation is required to illustrate no significant impacts will occur to the following environmental resources and that no unusual circumstances exist that would warrant a higher level of NEPA document. Upload all supporting documentation to the project file.

| Waterways: | Present; No Coast Guard, Individual 404, and/or Section 10 Permit required |
| --- | --- |

| Waterways Permit Type: | Permit Determination and/or Permit Application Approval Pending |
| --- | --- |

| Isolated Wetland Permit | No |
| --- | --- |

| Endangered Species: | Present; No finding of May Affect, Likely to Adversely Affect |
| --- | --- |

| Endangered Species - Coordination | May Affect, Not Likely to Adversely Affect |
| --- | --- |

| Endangered Species - Coordination Date | 11/30/2015 |
| --- | --- |

| Endangered Species - Critical Habitat Present/Impacted | Indiana bat |
| --- | --- |
|  | Northern long-eared bat |

| Endangered Species - Other Critical Habitat Present/Impacted: | No |
| --- | --- |

| 100-Year Floodplain: | No Encroachment Within a Special Flood Hazard Area (SFHA) |
| --- | --- |

| Section 4(f): | Not present |
| --- | --- |

| Section 6(f): | Not present |
| --- | --- |

| Cultural Resources: | Present; No Finding of Adverse Effect |
| --- | --- |

| Cultural Resources Coordination: | No Historic Properties Affected |
| --- | --- |

| Cultural Resources Coordination - SHPO Concurrence Date: | 12/16/2015 |
| --- | --- |

Projects that meet C2 criteria are not anticipated to have impacts to the following environmental resources. If resources are present, documentation is only required if there is a potential for impacts.

| Air Quality: | Studies Not Required |
| --- | --- |

| Air Quality - Coordination with OES: | No |
| --- | --- |

| Noise: | Studies Not Required |
| --- | --- |

| Noise Coordination - OES Approval Date: | 12/04/2014 |
| --- | --- |

| Hazardous Materials - ESA Screening Conducted | Yes |
| --- | --- |

| Hazardous Materials - OES Approval Date: | 11/05/2015 |
| --- | --- |

| Phase I ESA Warranted Based on Coordination with OES: | No Further Studies Warranted |
| --- | --- |

| Farmland: | Non-Urbanized Area; No Impacts in Accordance With the Farmland MOU, 7 CFR 658, and ORC 929.05 |
| --- | --- |

| Scenic Rivers | No National Wild and Scenic River Within 1000 Feet of the Proposed Project Area |
| --- | --- |

Projects that meet C2 criteria must be in accordance with ODOT's EJ Guidance and activities conducted for Public Involvement are commensurate to the project's type and scope of work.

| Environmental Justice: | Does Not Exceed EJ Guidance Criteria; No EJ Analysis Report Required and No EJ Issues Raised During Public Involvement |
| --- | --- |

| Public Involvement: | Minimum PI Requirements Met; No Minor Public or Agency Controversy on Environmental Grounds |
| --- | --- |

| Environmental Commitments | Yes |
| --- | --- |

Environmental Commitments,

1. COMMUNITY NOTIFICATION:

THE CONTRACTOR WILL BE REQUIRED TO COORDINATE THE PROJECT SCHEDULE WITH THE TRUMBULL COUNTY ENGINEERS OFFICE.  THE TRUMBULL COUNTY ENGINEERS OFFICE WILL THEN NOTIFY THE LOCAL EMERGENCY SERVICES AND COMMUNITY FOURTEEN (14) DAYS IN ADVANCE OF PROJECT CONSTRUCTION.  INCLUDED IN THE NOTIFICATION WILL BE THE PROJECTED DATES OF THE LANE RESTRICTIONS, ROADWAY CLOSURE AND DETOUR ROUTE.

2. RIPARIAN HABITAT:  EXISTING RIPARIAN HABITAT ZONES (NATURAL BUFFERS) ALONG THE STREAM CHANNEL(S) SHALL BE MAINTAINED TO THE MAXIMUM EXTENT PRACTICABLE DURING PROJECT CONSTRUCTION.

3. CONSTRUCTION AND DEMOLITION DEBRIS:  THE CONTRACTOR SHALL TAKE PRECAUTIONS TO AVOID AND/OR LIMIT CONSTRUCTION AND DEMOLITION DEBRIS FROM ENTERING WETLANDS AND STREAM(S).  ANY DEBRIS THAT DOES FALL INTO WETLANDS AND/OR STREAM(S) SHALL BE REMOVED AS SOON AS POSSIBLE WITHIN 72 HOURS.

4. MECHANICAL EQUIPMENT OPERATION AT STREAM CHANNEL:  THE MECHANICAL EQUIPMENT USED TO EXECUTE THE WORK AUTHORIZED HEREIN SHALL BE OPERATED IN SUCH A WAY AS TO MINIMIZE TURBIDITY THAT COULD DEGRADE WATER QUALITY AND ADVERSELY AFFECT AQUATIC PLANT AND ANIMAL LIFE.

5. STRUCTURE PAINTING/CONCRETE SEALING OPERATIONS:  THE CONTRACTOR SHALL TAKE ALL PRECAUTIONS NECESSARY TO PREVENT EPOXY-URETHANE SEALER, PAINT OR OTHER MATERIALS USED TO REPAIR, CLEAN, PAINT, SEAL OR TREAT ANY STRUCTURE FROM ENTERING ANY STREAMS, WETLANDS OR OTHER WATERS OF THE UNITED STATES AND TAKE THE APPROPRIATE ACTIONS IN THE EVENT OF A RELEASE.

6. STREAM CHANNEL EXCAVATION/IN STREAM WORK:  THE CONTRACTOR SHALL TAKE ALL PRECAUTIONS NECESSARY TO PREVENT ANY INCIDENTAL DISCHARGES ASSOCIATED WITH THE EXCAVATION AND HAULING OF MATERIAL FROM THE STREAM CHANNEL.  THIS PERTAINS TO ANY EXCAVATION OPERATION SUCH AS, FOUNDATION PIER OR ABUTMENT EXCAVATION, CHANNEL CLEAN OUT, EXCAVATION FOR ROCK CHANNEL PROTECTION AND REMOVAL OF ANY TEMPORARY FILL ASSOCIATED WITH CONSTRUCTION OPERATIONS.

7. MATERIALS REMOVED FROM DITCHES, STREAMS AND/OR WETLANDS:  ALL MATERIALS REMOVED FROM THE DITCHES, STREAMS AND/OR WETLANDS MUST BE IMMEDIATELY REMOVED TO AN UPLAND SITE AND STABILIZED (I.E., SEEDED) TO PREVENT REDISTRIBUTION INTO ANY WATERS OF THE UNITED STATES.  IMMEDIATE REMOVAL IS DEFINED BY THE UNITED STATES ARMY CORPS OF ENGINEERS AS DEPOSITING THE REMOVED MATERIALS DIRECTLY INTO A TRUCK AND REMOVING THE MATERIAL FROM THE SITE; PLACEMENT OF REMOVED MATERIALS INTO A WETLAND OR ON THE BANKS OF A STREAM EVEN TEMPORARILY IS CONSIDERED A FILL AND REQUIRES A PERMIT ACTION.  ANY AREAS DISTURBED BY EQUIPMENT ACTIVITIES MUST BE SEEDED WITH NATIVE SPECIES TO PREVENT EROSION OF SEDIMENTS INTO WATERS OF THE UNITED STATES.

8. WETLANDS IMPACTS/AVOIDANCE - UNDER THE DIRECTION OF THE PROJECT ENGINEER, THE CONTRACTOR MAY IMPACT/PLACE PERMANENT FILLS IN THE FOLLOWING WETLANDS:

WETLAND B

STATION 104+10 LEFT TO STATION 105+05 RIGHT - 0.01 ACRE IMPACT;

UNDER NO CIRCUMSTANCES SHALL THE CONTRACTOR IMPACT THE REMAINING WETLANDS DELINEATED BEYOND THE PROJECT CONSTRUCTION LIMITS AND DEPICTED IN THE WETLANDS LOCATION MAPPING AND THE PROJECT PLANS.  THE WETLAND LOCATION MAPPING IS AVAILABLE FOR INSPECTION AT THE ODOT DISTRICT 4 OFFICE, 2088 SOUTH ARLINGTON, AKRON, OHIO 44306.  UNDER NO CIRCUMSTANCES SHALL THE CONTRACTOR STORE EQUIPMENT AND/OR MATERIALS WITHIN THESE WETLAND AREAS. TO PROTECT AND DELINEATE THE BOUNDARY OF THE EXISTING REMAINING WETLANDS, A FILTER FABRIC FENCE AND TEMPORARY CONSTRUCTION FENCE, PER SUPPLEMENTAL SPECIFICATION 832, SHALL BE INSTALLED AT THE PROPOSED CONSTRUCTION LIMITS WITHIN THE WETLANDS AREA BY THE CONTRACTOR PRIOR TO THE START OF ANY CONSTRUCTION ACTIVITIES WITHIN THESE LIMITS AND ADJACENT AREA, INCLUDING ANY NECESSARY CLEARING AND GRUBBING ACTIVITIES, AND MAINTAINED BY THE CONTRACTOR THROUGHOUT PROJECT CONSTRUCTION AND SHALL BE REMOVED BY THE CONTRACTOR UPON PROJECT COMPLETION.

9. AREAS DISTURBED BY EQUIPMENT ACTIVITIES:

ANY AREAS DISTURBED BY EQUIPMENT ACTIVITIES MUST BE SEEDED WITH AN APPROPRIATE PRAIRE SEED MIX AND MULCHED DURING CONSTRUCTION TO ENCOURAGE ESTABLISHMENT OF BENEFICIAL VEGETATIVE COVER FOR POLLINATORS AND DECREASE OR PREVENT EROSION OF SEDIMENTS INTO WATERS OF THE UNITED STATES.

10. CONSTRUCTION EQUIPMENT AND MATERIALS STAGING AREAS:

CONSTRUCTION EQUIPMENT AND MATERIAL STAGING AREAS SHALL BE KEPT AWAY FROM STREAMS AND WETLANDS TO THE MAXIMUM EXTENT PRACTICABLE.  ODOT CONSTRUCTION AND MATERIALS SPECIFICATIONS SECTION 107.10 (PROTECTION AND RESTORATION OF PROPERTY) PROHIBIT THE CONTRACTOR FROM CREATING STAGING AREAS NEAR STREAMS/WETLANDS.

11. BEST MANAGEMENT PRACTICES/SOIL EROSION AND SEDIMENTATION CONTROL:

WATER COLUMN AND SEDIMENTATION IMPACTS SHALL BE KEPT TO A MINIMUM THROUGH THE USE OF BEST MANAGEMENT PRACTICES FOR SOIL EROSION AND SEDIMENTATION CONTROL. ALL SOIL EROSION CONTROL MEASURES SHALL BE IN PLACE PRIOR TO ANY EXCAVATION, GRADING OR FILLING OPERATIONS AND INSTALLATION OF PROPOSED STRUCTURES OR UTILITIES. THEY SHALL REMAIN IN PLACE UNTIL CONSTRUCTION IS COMPLETED AND THE AREA IS STABILIZED AS ACCEPTED BY THE ENGINEER. THESE SHALL COMPLY WITH ODOT'S HANDBOOK FOR SEDIMENT AND EROSION CONTROL, WHICH MAY BE FOUND AT: HTTP://WWW.DOT.STATE.OH.US/DRRC/.

12. TREE CLEARING:

NO TREE CLEARING ON ANY PART OF THE PROJECT SHALL COMMENCE UNTIL ALL NECESSARY SECTION 404/401 PERMITS AUTHORIZING THE PROJECT HAVE BEEN OBTAINED FROM THE U. S. ARMY CORPS OF ENGINEERS FOR THE PROJECT.

13. ENDANGERED SPECIES HABITAT-INDIANA BAT/NORTHERN LONG-EARED BAT:

THE PROJECT IS LOCATED WITHIN THE KNOWN HABITAT RANGES OF THE FEDERALLY LISTED AND PROTECTED INDIANA BAT AND NORTHERN LONG-EARED BAT.  NO TREES SHALL BE REMOVED UNDER THIS PROJECT FROM APRIL 1 THROUGH SEPTEMBER 30.  ALL NECESSARY TREE REMOVAL SHALL OCCUR FROM OCTOBER 1 THROUGH MARCH 31.  THIS REQUIREMENT IS NECESSARY TO AVOID AND MINIMIZE IMPACTS TO THESE SPECIES AS REQUIRED BY THE ENDANGERED SPECIES ACT.  FOR THE PURPOSES OF THIS NOTE, A TREE IS DEFINED AS A LIVE, DYING, OR DEAD WOODY PLANT, WITH A TRUNK THREE INCHES OR GREATER IN DIAMETER AT A HEIGHT OF 4.5 FEET ABOVE THE GROUND SURFACE, AND WITH A MINIMUM HEIGHT OF 13 FEET.

PRIOR TO REMOVAL OF THE EXISTING BRIDGE DECK, THE UNDERSIDE OF THE TRU-CORINTH COURT ROAD (TR261) BRIDGE OVER SUGAR CREEK SHALL BE CAREFULLY EXAMINED FOR THE PRESENCE OF BATS, ESPECIALLY FROM APRIL 1 TO SEPTEMBER 30.  IF ANY BATS ARE FOUND ROOSTING ON THE UNDERSIDE OF A BRIDGE, THE UNITED STATES FISH AND WILDLIFE SERVICE, ECOLOGICAL SERVICES DIVISION (614-416-8993), ODOT OFFICE OF ENVIRONMENTAL SERVICES (614-466-7880) AND ODOT DISTRICT 4 ENVIRONMENTAL SECTION (330-786-4930) SHALL BE CONTACTED TO PROVIDE THIS INFORMATION.

14. DRINKING WATER RESOURCES:

CAUTION WILL BE EXERCISED DURING REFUELING OPERATIONS AND DURING CONSTRUCTION EQUIPMENT MAINTENANCE ACTIVITIES.  IN CASE OF AN INCIDENT AND/OR SPILL, THE CONTRACTOR SHALL, AS SOON AS POSSIBLE, NOTIFY THE ENGINEER/SUPERVISOR AND CONTACT THE OHIO EPA: OHIO EPA SPILL REPORTING 24 HOUR EMERGENCY SERVICE CALL: 1-800-282-9378 OR NEDO (330) 963-1200 AND PROVIDE AS MUCH OF THE FOLLOWING INFORMATION AS POSSIBLE: 1. TIME OBSERVED 2. LOCATION 3. MATERIAL RELEASED 4. PROBABLE SOURCE 5. VOLUME & DURATION 6. PRESENT & ANTICIPATED MOVEMENT OF CONTAMINANT 7. PERSONNEL ON SCENE 8. ACTIONS ALREADY INITIATED 9. PERSON(S) ON THE SCENE TO CONTACT.

Environmental Commitments

C2

Commitments

1. COMMUNITY NOTIFICATION:

THE CONTRACTOR WILL BE REQUIRED TO COORDINATE THE PROJECT SCHEDULE WITH THE TRUMBULL COUNTY ENGINEERS OFFICE.  THE TRUMBULL COUNTY ENGINEERS OFFICE WILL THEN NOTIFY THE LOCAL EMERGENCY SERVICES AND COMMUNITY FOURTEEN (14) DAYS IN ADVANCE OF PROJECT CONSTRUCTION.  INCLUDED IN THE NOTIFICATION WILL BE THE PROJECTED DATES OF THE LANE RESTRICTIONS, ROADWAY CLOSURE AND DETOUR ROUTE.

2. RIPARIAN HABITAT:  EXISTING RIPARIAN HABITAT ZONES (NATURAL BUFFERS) ALONG THE STREAM CHANNEL(S) SHALL BE MAINTAINED TO THE MAXIMUM EXTENT PRACTICABLE DURING PROJECT CONSTRUCTION.

3. CONSTRUCTION AND DEMOLITION DEBRIS:  THE CONTRACTOR SHALL TAKE PRECAUTIONS TO AVOID AND/OR LIMIT CONSTRUCTION AND DEMOLITION DEBRIS FROM ENTERING WETLANDS AND STREAM(S).  ANY DEBRIS THAT DOES FALL INTO WETLANDS AND/OR STREAM(S) SHALL BE REMOVED AS SOON AS POSSIBLE WITHIN 72 HOURS.

4. MECHANICAL EQUIPMENT OPERATION AT STREAM CHANNEL:  THE MECHANICAL EQUIPMENT USED TO EXECUTE THE WORK AUTHORIZED HEREIN SHALL BE OPERATED IN SUCH A WAY AS TO MINIMIZE TURBIDITY THAT COULD DEGRADE WATER QUALITY AND ADVERSELY AFFECT AQUATIC PLANT AND ANIMAL LIFE.

5. STRUCTURE PAINTING/CONCRETE SEALING OPERATIONS:  THE CONTRACTOR SHALL TAKE ALL PRECAUTIONS NECESSARY TO PREVENT EPOXY-URETHANE SEALER, PAINT OR OTHER MATERIALS USED TO REPAIR, CLEAN, PAINT, SEAL OR TREAT ANY STRUCTURE FROM ENTERING ANY STREAMS, WETLANDS OR OTHER WATERS OF THE UNITED STATES AND TAKE THE APPROPRIATE ACTIONS IN THE EVENT OF A RELEASE.

6. STREAM CHANNEL EXCAVATION/IN STREAM WORK:  THE CONTRACTOR SHALL TAKE ALL PRECAUTIONS NECESSARY TO PREVENT ANY INCIDENTAL DISCHARGES ASSOCIATED WITH THE EXCAVATION AND HAULING OF MATERIAL FROM THE STREAM CHANNEL.  THIS PERTAINS TO ANY EXCAVATION OPERATION SUCH AS, FOUNDATION PIER OR ABUTMENT EXCAVATION, CHANNEL CLEAN OUT, EXCAVATION FOR ROCK CHANNEL PROTECTION AND REMOVAL OF ANY TEMPORARY FILL ASSOCIATED WITH CONSTRUCTION OPERATIONS.

7. MATERIALS REMOVED FROM DITCHES, STREAMS AND/OR WETLANDS:  ALL MATERIALS REMOVED FROM THE DITCHES, STREAMS AND/OR WETLANDS MUST BE IMMEDIATELY REMOVED TO AN UPLAND SITE AND STABILIZED (I.E., SEEDED) TO PREVENT REDISTRIBUTION INTO ANY WATERS OF THE UNITED STATES.  IMMEDIATE REMOVAL IS DEFINED BY THE UNITED STATES ARMY CORPS OF ENGINEERS AS DEPOSITING THE REMOVED MATERIALS DIRECTLY INTO A TRUCK AND REMOVING THE MATERIAL FROM THE SITE; PLACEMENT OF REMOVED MATERIALS INTO A WETLAND OR ON THE BANKS OF A STREAM EVEN TEMPORARILY IS CONSIDERED A FILL AND REQUIRES A PERMIT ACTION.  ANY AREAS DISTURBED BY EQUIPMENT ACTIVITIES MUST BE SEEDED WITH NATIVE SPECIES TO PREVENT EROSION OF SEDIMENTS INTO WATERS OF THE UNITED STATES.

8. WETLANDS IMPACTS/AVOIDANCE - UNDER THE DIRECTION OF THE PROJECT ENGINEER, THE CONTRACTOR MAY IMPACT/PLACE PERMANENT FILLS IN THE FOLLOWING WETLANDS:

WETLAND B

STATION 104+10 LEFT TO STATION 105+05 RIGHT - 0.01 ACRE IMPACT;

UNDER NO CIRCUMSTANCES SHALL THE CONTRACTOR IMPACT THE REMAINING WETLANDS DELINEATED BEYOND THE PROJECT CONSTRUCTION LIMITS AND DEPICTED IN THE WETLANDS LOCATION MAPPING AND THE PROJECT PLANS.  THE WETLAND LOCATION MAPPING IS AVAILABLE FOR INSPECTION AT THE ODOT DISTRICT 4 OFFICE, 2088 SOUTH ARLINGTON, AKRON, OHIO 44306.  UNDER NO CIRCUMSTANCES SHALL THE CONTRACTOR STORE EQUIPMENT AND/OR MATERIALS WITHIN THESE WETLAND AREAS. TO PROTECT AND DELINEATE THE BOUNDARY OF THE EXISTING REMAINING WETLANDS, A FILTER FABRIC FENCE AND TEMPORARY CONSTRUCTION FENCE, PER SUPPLEMENTAL SPECIFICATION 832, SHALL BE INSTALLED AT THE PROPOSED CONSTRUCTION LIMITS WITHIN THE WETLANDS AREA BY THE CONTRACTOR PRIOR TO THE START OF ANY CONSTRUCTION ACTIVITIES WITHIN THESE LIMITS AND ADJACENT AREA, INCLUDING ANY NECESSARY CLEARING AND GRUBBING ACTIVITIES, AND MAINTAINED BY THE CONTRACTOR THROUGHOUT PROJECT CONSTRUCTION AND SHALL BE REMOVED BY THE CONTRACTOR UPON PROJECT COMPLETION.

9. AREAS DISTURBED BY EQUIPMENT ACTIVITIES:

ANY AREAS DISTURBED BY EQUIPMENT ACTIVITIES MUST BE SEEDED WITH AN APPROPRIATE PRAIRE SEED MIX AND MULCHED DURING CONSTRUCTION TO ENCOURAGE ESTABLISHMENT OF BENEFICIAL VEGETATIVE COVER FOR POLLINATORS AND DECREASE OR PREVENT EROSION OF SEDIMENTS INTO WATERS OF THE UNITED STATES.

10. CONSTRUCTION EQUIPMENT AND MATERIALS STAGING AREAS:

CONSTRUCTION EQUIPMENT AND MATERIAL STAGING AREAS SHALL BE KEPT AWAY FROM STREAMS AND WETLANDS TO THE MAXIMUM EXTENT PRACTICABLE.  ODOT CONSTRUCTION AND MATERIALS SPECIFICATIONS SECTION 107.10 (PROTECTION AND RESTORATION OF PROPERTY) PROHIBIT THE CONTRACTOR FROM CREATING STAGING AREAS NEAR STREAMS/WETLANDS.

11. BEST MANAGEMENT PRACTICES/SOIL EROSION AND SEDIMENTATION CONTROL:

WATER COLUMN AND SEDIMENTATION IMPACTS SHALL BE KEPT TO A MINIMUM THROUGH THE USE OF BEST MANAGEMENT PRACTICES FOR SOIL EROSION AND SEDIMENTATION CONTROL. ALL SOIL EROSION CONTROL MEASURES SHALL BE IN PLACE PRIOR TO ANY EXCAVATION, GRADING OR FILLING OPERATIONS AND INSTALLATION OF PROPOSED STRUCTURES OR UTILITIES. THEY SHALL REMAIN IN PLACE UNTIL CONSTRUCTION IS COMPLETED AND THE AREA IS STABILIZED AS ACCEPTED BY THE ENGINEER. THESE SHALL COMPLY WITH ODOT'S HANDBOOK FOR SEDIMENT AND EROSION CONTROL, WHICH MAY BE FOUND AT: HTTP://WWW.DOT.STATE.OH.US/DRRC/.

12. TREE CLEARING:

NO TREE CLEARING ON ANY PART OF THE PROJECT SHALL COMMENCE UNTIL ALL NECESSARY SECTION 404/401 PERMITS AUTHORIZING THE PROJECT HAVE BEEN OBTAINED FROM THE U. S. ARMY CORPS OF ENGINEERS FOR THE PROJECT.

13. ENDANGERED SPECIES HABITAT-INDIANA BAT/NORTHERN LONG-EARED BAT:

THE PROJECT IS LOCATED WITHIN THE KNOWN HABITAT RANGES OF THE FEDERALLY LISTED AND PROTECTED INDIANA BAT AND NORTHERN LONG-EARED BAT.  NO TREES SHALL BE REMOVED UNDER THIS PROJECT FROM APRIL 1 THROUGH SEPTEMBER 30.  ALL NECESSARY TREE REMOVAL SHALL OCCUR FROM OCTOBER 1 THROUGH MARCH 31.  THIS REQUIREMENT IS NECESSARY TO AVOID AND MINIMIZE IMPACTS TO THESE SPECIES AS REQUIRED BY THE ENDANGERED SPECIES ACT.  FOR THE PURPOSES OF THIS NOTE, A TREE IS DEFINED AS A LIVE, DYING, OR DEAD WOODY PLANT, WITH A TRUNK THREE INCHES OR GREATER IN DIAMETER AT A HEIGHT OF 4.5 FEET ABOVE THE GROUND SURFACE, AND WITH A MINIMUM HEIGHT OF 13 FEET.

PRIOR TO REMOVAL OF THE EXISTING BRIDGE DECK, THE UNDERSIDE OF THE TRU-CORINTH COURT ROAD (TR261) BRIDGE OVER SUGAR CREEK SHALL BE CAREFULLY EXAMINED FOR THE PRESENCE OF BATS, ESPECIALLY FROM APRIL 1 TO SEPTEMBER 30.  IF ANY BATS ARE FOUND ROOSTING ON THE UNDERSIDE OF A BRIDGE, THE UNITED STATES FISH AND WILDLIFE SERVICE, ECOLOGICAL SERVICES DIVISION (614-416-8993), ODOT OFFICE OF ENVIRONMENTAL SERVICES (614-466-7880) AND ODOT DISTRICT 4 ENVIRONMENTAL SECTION (330-786-4930) SHALL BE CONTACTED TO PROVIDE THIS INFORMATION.

14. DRINKING WATER RESOURCES:

CAUTION WILL BE EXERCISED DURING REFUELING OPERATIONS AND DURING CONSTRUCTION EQUIPMENT MAINTENANCE ACTIVITIES.  IN CASE OF AN INCIDENT AND/OR SPILL, THE CONTRACTOR SHALL, AS SOON AS POSSIBLE, NOTIFY THE ENGINEER/SUPERVISOR AND CONTACT THE OHIO EPA: OHIO EPA SPILL REPORTING 24 HOUR EMERGENCY SERVICE CALL: 1-800-282-9378 OR NEDO (330) 963-1200 AND PROVIDE AS MUCH OF THE FOLLOWING INFORMATION AS POSSIBLE: 1. TIME OBSERVED 2. LOCATION 3. MATERIAL RELEASED 4. PROBABLE SOURCE 5. VOLUME & DURATION 6. PRESENT & ANTICIPATED MOVEMENT OF CONTAMINANT 7. PERSONNEL ON SCENE 8. ACTIONS ALREADY INITIATED 9. PERSON(S) ON THE SCENE TO CONTACT.

Preparers  & Approvals

Form Preparer(s)

| Form Preparer | Jennifer Walker Burgess AND Niple Inc. 5085 Reed Road, Columbus OH 43220 614-459-2050 jennifer.walker@burgessniple.com |
| --- | --- |

| Supporting Form Preparer(s) | Mary Christoff, Sean Carpenter, Gary Shaffer, Krista Carter |
| --- | --- |

Approvals & Electronic Signatures

| Approved & Electronically Signed By: | Approval Date: |
| --- | --- |

| Edward Deley PROGRAM ADMIN 3 | May 19, 2016 |
| --- | --- |
| Edward Deley PROGRAM ADMIN 3 | May 19, 2016 |

Appendix

Uploaded Files by Report Section

General

Aerial Map.pdf

County Map.pdf

Photographs.pdf

Stage 2 Design.pdf

Alternatives

Feasibility Study.pdf

ESA

OES Recommendations - Screening.pdf

Asbestos Survey.pdf

Screening.pdf

Cultural Resources

OES Approval - No Historic Properties Affected.pdf

Section 106 Request for Review.pdf

Ecological

Level 1 MOA Coordination.pdf

USFWS Comments.pdf

Level 1 Ecological Survey.pdf

ODNR Comments.pdf

ODOT Response to USFWS and ODNR Comments.pdf

Wetland Documentation.pdf

Other Resources

Water Source Protection Area Map.pdf

Drinking Water Correspondence.pdf

Water Source Protection Area Map.PNG

Environmental Justice

Census Mapping.pdf

Answers to EJ Questions.pdf

Public Involvement

Press Release.pdf

Property Owner Notification Letter.pdf

Public Comments Received.pdf

Response to Public Comments.pdf

Permits

FEMA FIRM.pdf