Environmental Document

for

GEA SR 087 19.75 Safety PID 23733

Environmental Document Level: C2

Approved: October 6, 2017

Prepared by: Libby Rushley

Lawhon  AND  Associates

e-mail:

The environmental review, consultation, and other actions required by applicable Federal environmental laws for this project are being, or have been, carried-out by ODOT pursuant to 23 U.S.C. 327 and a Memorandum of Understanding dated December 11, 2015, and executed by FHWA and ODOT.

Contents

C2	3

Environmental Commitments, 	8

Environmental Commitments	9

C2	9

Preparers  & Approvals	10

Form Preparer(s)	10

Approvals & Electronic Signatures	10

Appendix	11

Uploaded Files by Report Section	11

Noise	11

General	11

ESA	11

Cultural Resources	11

Ecological	11

Other Resources	12

Environmental Justice	12

Public Involvement	12

Permits	12

C2

| PID | 23733 |
| --- | --- |

| Project Sponsor | ODOT SPONSORING AGENCY |
| --- | --- |

| ODOT District | 12 |
| --- | --- |

| Funding Source | Federal |
| --- | --- |

| Private Funding: | No |
| --- | --- |

Project Description:

ODOT proposes to improve SR 87 (Kinsman Road) between Hayes Road and Bundysburg Road in Middlefield Township, Geauga County. The improvements include reconstruction of approximately 800-feet of the hill near Hayes Road to improve vertical sight distance, shoulder reconstruction and widening for Amish buggy lane on both sides of SR 87 for approximately 0.9-miles, and replacement of three culverts. Work will be completed using part-width construction. No detour will be needed.

Based upon review of appropriate mapping, the proposed project is located in a non-urbanized area. Based on the scope and type of work, the proposed project meets the terms and conditions of the Memorandum of Understanding between the Natural Resource Conservation Service and the Ohio Department of Transportation for Projects Involving Farmlands (Agreement No. 19552)executed on March 15,2016. Therefore, completion of the Farmland Conversion Impact Rating (FCIR) Form was not warranted and no further coordination is required.

Some pavement improvements were made in 2014 under PID 92571.

SR 87 is functionally classified as a Major Collector in the project area.

This project will require temporary and permanent right-of-way.

| STIP Reference # | FY 2018-2021 STIP Statewide Line Item #1, 15 |
| --- | --- |

| Select the appropriate project type: | (27) Highway safety or traffic operations improvement projects, including the installation of ramp metering control devices and lighting, if the project meets the constraints in paragraph (e) of this section. Examples include: Lane reduction ("road diet") changes provided traffic analysis is completed; Railroad projects that close at-grade crossings to improve safety or traffic operations. |
| --- | --- |

In accordance with 23 CFR 771.117(e), the proposed project cannot be processed as a C2 CE, if it involves -

- a. Acquisition of more than a minor amount of right-of-way

- b. Residential or non-residential displacements

- c. A Coast Guard, Individual Section 404 and/or a Section 10 permit

- d. A Section 106 finding of Adverse Effect

- e. A Section 4(f) Programmatic or Individual Evaluation

- f. A finding of May Affect, Likely to Adversely Affect to Threatened and Endangered Species

- g. Construction of temporary access, or the closure of existing road, bridge, or ramps, that would result in major traffic disruptions

- h. Changes in access control

- i. Floodplain encroachment other than functionally dependent uses (e.g., bridges, wetlands) or actions that facilitate open space use (e.g., recreational trails, bicycle and pedestrian paths)

- j. Construction activities in, across or adjacent to a river component designated or proposed for inclusion in the National System of Wild and Scenic Rivers

- k. No minor public or agency controversy on environmental grounds (no opposition from any organized groups or agencies and no unresolved environmental coordination)

- l. If an EJ Analysis Report is required, the project must be processed as a D-level CE or higher level document

For certification purposes, documentation is required to illustrate no significant impacts will occur to the following environmental resources and that no unusual circumstances exist that would warrant a higher level of NEPA document. Upload all supporting documentation to the project file.

| Waterways: | Present; No Coast Guard, Individual 404, and/or Section 10 Permit required |
| --- | --- |

| Waterways Permit Type: | Permit Determination and/or Permit Application Approval Pending |
| --- | --- |

| Isolated Wetland Permit | No |
| --- | --- |

| Endangered Species: | Present; No finding of May Affect, Likely to Adversely Affect |
| --- | --- |

| Endangered Species - Coordination | May Affect, Not Likely to Adversely Affect |
| --- | --- |

| Endangered Species - Coordination Date | 07/26/2016 |
| --- | --- |

| Endangered Species - Critical Habitat Present/Impacted | Indiana bat |
| --- | --- |
|  | Northern long-eared bat |

| Endangered Species - Other Critical Habitat Present/Impacted: | No |
| --- | --- |

| 100-Year Floodplain: | No Encroachment Within a Special Flood Hazard Area (SFHA) |
| --- | --- |

| Section 4(f): | Not present |
| --- | --- |

| Section 6(f): | Not present |
| --- | --- |

| Cultural Resources: | Present; No Finding of Adverse Effect |
| --- | --- |

| Cultural Resources Coordination: | No Historic Properties Affected |
| --- | --- |

| Cultural Resources Coordination - SHPO Concurrence Date: | 09/26/2016 |
| --- | --- |

Projects that meet C2 criteria are not anticipated to have impacts to the following environmental resources. If resources are present, documentation is only required if there is a potential for impacts.

| Air Quality: | Studies Not Required |
| --- | --- |

| Air Quality - Coordination with OES: | No |
| --- | --- |

| Noise: | Studies Not Required |
| --- | --- |

| Noise Coordination - OES Approval Date: | 10/05/2017 |
| --- | --- |

| Hazardous Materials - ESA Screening Conducted | Yes |
| --- | --- |

| Hazardous Materials - OES Approval Date: | 04/07/2016 |
| --- | --- |

| Phase I ESA Warranted Based on Coordination with OES: | Yes |
| --- | --- |

| Phase II ESA Warranted Based on Coordination with OES: | Yes |
| --- | --- |

| Hazardous Materials - Coordination Results | No Further Studies Warranted |
| --- | --- |

| Hazardous Materials - OES Approval Date - Phase I: | 11/29/2016 |
| --- | --- |

| Hazardous Materials - OES Approval Date - Phase II: | 04/11/2017 |
| --- | --- |

| Farmland: | Non-Urbanized Area; No Impacts in Accordance With the Farmland MOU, 7 CFR 658, and ORC 929.05 |
| --- | --- |

| Scenic Rivers | No National Wild and Scenic River Within 1000 Feet of the Proposed Project Area |
| --- | --- |

Projects that meet C2 criteria must be in accordance with ODOT's EJ Guidance and activities conducted for Public Involvement are commensurate to the project's type and scope of work.

| Environmental Justice: | Does Not Exceed EJ Guidance Criteria; No EJ Analysis Report Required and No EJ Issues Raised During Public Involvement |
| --- | --- |

| Public Involvement: | Minimum PI Requirements Met; No Minor Public or Agency Controversy on Environmental Grounds |
| --- | --- |

| Environmental Commitments | Yes |
| --- | --- |

Environmental Commitments,

1. This project is located within the known habitat ranges of the federally listed and protected Indiana Bat and Northern Long-Eared Bat. No trees shall be removed under this project from April 1 through September 30. All necessary tree removal shall occur from October 1 through March 31. This requirement is necessary to avoid and minimize impacts to these species as required by the Endangered Species Act. For the purpose of this note, a tree is defined as a live, dying, or dead woody plant with a truck three inches or greater in diameter at a height of 4.5 feet above the ground surface, and with a minimum height of 13 feet.

2. Modify all phases/aspects of the project (e.g. temporary work areas, alignments) to avoid tree removal in excess of what is required to implement the project safely.

3. Ensure tree removal is limited to that specified in project plans by clearly marking clearing limits. Ensure that contractors understand clearing limits and how they are marked in the field.

Environmental Commitments

C2

Commitments

1. This project is located within the known habitat ranges of the federally listed and protected Indiana Bat and Northern Long-Eared Bat. No trees shall be removed under this project from April 1 through September 30. All necessary tree removal shall occur from October 1 through March 31. This requirement is necessary to avoid and minimize impacts to these species as required by the Endangered Species Act. For the purpose of this note, a tree is defined as a live, dying, or dead woody plant with a truck three inches or greater in diameter at a height of 4.5 feet above the ground surface, and with a minimum height of 13 feet.

2. Modify all phases/aspects of the project (e.g. temporary work areas, alignments) to avoid tree removal in excess of what is required to implement the project safely.

3. Ensure tree removal is limited to that specified in project plans by clearly marking clearing limits. Ensure that contractors understand clearing limits and how they are marked in the field.

Preparers  & Approvals

Form Preparer(s)

| Form Preparer | Libby Rushley Lawhon AND Associates 1441 King Avenue, Columbus OH 43212 lrushley@lawhon-assoc.com |
| --- | --- |

| Supporting Form Preparer(s) | Daniel BENDER, Thomas Powell |
| --- | --- |

Approvals & Electronic Signatures

| Approved & Electronically Signed By: | Approval Date: |
| --- | --- |

| Mark Carpenter TRANS ENGR 3 | October 6, 2017 |
| --- | --- |

Appendix

Uploaded Files by Report Section

Noise

District Determination - No Noise Analysis Required.pdf

General

County Map.pdf

Aerial Map.pdf

USGS Quadrangle Topographical Map.pdf

Stage 1 Design.pdf

ESA

Screening.pdf

OES Recommendations - Screening.pdf

Phase I .pdf

OES Recommendations - Phase I.pdf

Phase II .pdf

OES Recommendations - Phase II.pdf

Cultural Resources

Section 106 Request for Review.pdf

Phase I Archaeology .pdf

OES Approval - No Historic Properties Affected.pdf

Section 106 Eligibility Determination.pdf

Records Check.pdf

Scoping Guidance_Archaeology.pdf

Ecological

Level 1 ESR GEA-87-19.75 PID 23733.pdf

Coordination with USFWS.pdf

Consult Form GEA-87-19.75 PID 23733.pdf

Wetland Finding.docx

Other Resources

Water Source Protection Area Map.pdf

Site Specific Resource Mapping.pdf

Well Log Data.pdf

Hydric Soil Mapping.pdf

ArcGIS Shapefile.shp

FCIR Form.pdf

Coordination with NRCS.pdf

Environmental Justice

Census Mapping.pdf

Underserved Populations Documentation.pdf

Public Involvement

Property Owner Notification Letter.pdf

Public Meeting Notification.pdf

Public Comments Received.pdf

Public Meeting Sign In Sheet.pdf

Meeting Notice in Geauga Maple Leaf.pdf

Public Meeting Handouts.pdf

Phone call Response to Puclic comment 6-27-16.pdf.pdf

Response to Public Comments.pdf

Permits

FEMA FIRM.pdf