Environmental Document

for

POR Center Road (CR-293) PID 93078

Environmental Document Level: C2

Approved: February 8, 2017

Prepared by: Brian Peck

ODOT DISTRICT 4

Phone:  330-786-4931

e-mail:

The environmental review, consultation, and other actions required by applicable Federal environmental laws for this project are being, or have been, carried-out by ODOT pursuant to 23 U.S.C. 327 and a Memorandum of Understanding dated December 11, 2015, and executed by FHWA and ODOT.

Contents

C2	3

Environmental Commitments, 	11

Environmental Commitments	19

C2	19

Preparers  & Approvals	27

Form Preparer(s)	27

Approvals & Electronic Signatures	27

Appendix	28

Uploaded Files by Report Section	28

Environmental Commitments	28

Section 4(f)	28

General	28

ESA	28

Cultural Resources	28

Ecological	28

Environmental Justice	29

Public Involvement	29

Permits	29

C2

| PID | 93078 |
| --- | --- |

| Project Sponsor | PORTAGE COUNTY ENGINEER |
| --- | --- |

| ODOT District | 4 |
| --- | --- |

| Funding Source | Federal |
| --- | --- |

| Private Funding: | No |
| --- | --- |

Project Description:

The project, proposed and sponsored by the Portage CountyEngineer, is located in Nelson Township, northeastern Portage County, Ohio.The project proposes to reconstruct 1.98miles of Center Road (County Road 293) from BrosiusRoad to Parkman Road.  Center Road is classified as a Rural MinorCollector and the average existing pavement width is 19 feetwith 1-foot graded shoulders.  The projectproposes to reconstruct the pavement edges and shoulders to accommodate two (2)11-foot lanes with 2-foot asphalt shoulders and 4-foot graded shoulders and also reconstructthe existing open storm drainage system, including replacement of several smalldiameter vitrified clay, plastic, concrete and corrugated metal cross culverts and drivewayculverts.  Roadway geometric improvements additionally require replacement of Bridge No.POR-C293A-NL092; Structure File Number (SFN) 6733360.  Theexisting bridge, spanning Tinker Creek, is a 21-foot single span steel beam bridge with a 24-foot wide corrugated steel deck face-to-face of guardrails, an asphalt wearing surface and stone/concreteabutments.  Thisbridge, located approximately 65 feet northeast of the Center Road/Hopkins Roadintersection, will be replaced with a 22-foot clear span precast reinforced three-sided culvert on cast-in-place concrete spread footings and a 28-foot wide asphalt wearing surface face-to-face of guardrails.  All project construction activities will becompleted within the existing roadway right-of-way.

A Level 1 Ecological Survey Report (ESR) was prepared forthe project by LJB, Inc. personnel in August 2015.Based on the field review to complete theAugust 2015, Level 1 ESR, six streams and six wetlands were identified withinor abutting the project study area.  Theproject will impact all six streams and four of the six identified wetlandsareas with a total stream impact of approximately 238 linear feet and a total wetlandsimpact of approximately 0.017 acre.

The project is located within the known habitat ranges ofthe Federally endangered Indiana Bat and Northern Long-eared Bat and will removesix potential roosting habitat trees.OtherFederally listed species and/or state listed species will not be impacted bythe project.

The Ohio Department of Natural Resources (ODNR) NaturalDatabase Heritage Program identified one management area, the Eagle Creek StateNature Preserve, located within a one-mile radius of the project study area.The Eagle Creek State Nature Preserve is 472acres total and its northern edge is located within the southern section of theproject study area.The project will notpermanently incorporate land from the Eagle Creek State Nature Preserve into atransportation facility.  Moreover, theproject will not restrict access to or temporarily occupy the nature preserve arealocated adjacent to the project study area.Therefore, Section 4(f) coordination is notrequired for the project in accordance with 23 CFR 774.

Special flood hazard areas exist in proximity tothe project.  The Portage County Engineer coordinated the project construction plan with the Portage County Floodplain Administrator on December 19, 2016, and January 27, 2017.  Upon review of the submitted project documents, the Portage County Floodplain Administrator, in correspondence dated February 2, 2017 (corrected year), stated the proposed bridge and road work project will not impact the existing floodplain in the area (Nelson Township) of Portage County and that it is acceptable relative to floodplain management requirements.

Traffic will be maintained during construction with theexception of a complete closure required for the POR-C293A-NL092 bridge replacement.Center Road will be closed at thePOR-C293A-NL092 bridge for approximately 90 consecutive days duringconstruction.  A detour route will beposted for the project. The Portage County Engineer's Office released a newsrelease for the project on November 21, 2016, and maintenance of trafficinformation was submitted to local emergency and public services(public/private EMS, police, fire, schools, etc.) by electronic mailcorrespondence on November 21, 2016.Publiccomments were not received as a result of the public involvement activitiesconducted for the project.

| STIP Reference # | Statewide Line Item (SLI) Number 16 and is fiscally constrained. |
| --- | --- |

| Select the appropriate project type: | (26) Modernization of a highway by resurfacing, restoration, rehabilitation, reconstruction, adding shoulders, or adding auxiliary lanes (including parking, weaving, turning, and climbing lanes), if the action meets the constraints in paragraph (e) of this section. Examples include: Joint or limited use of right-of-way where the proposed use would have minimal or no adverse social (including highway safety), economic or environmental impacts; Installation of new noise walls and other new noise mitigation projects; Construction of highway safety and truck escape ramps; Construction of bicycle lanes and pedestrian walkways, sidewalks, shared-use paths, or facilities and trailhead parking that do not otherwise qualify for a C1 designation; Beautification or facility improvement projects (i.e. landscaping, curb and gutter installation and replacement, ADA ramps/curb ramps, installation of park benches, decorative lighting, etc.); Construction of alternative energy facilities (fuel tank farms, wind turbines, etc.) |
| --- | --- |
|  | (28) Bridge rehabilitation, reconstruction, or replacement or the construction of grade separation to replace existing at-grade railroad crossings, if the actions meet the constraints in 23 CFR 771.117(e). Examples include: Railroad projects that close or relocate at-grade crossings |

In accordance with 23 CFR 771.117(e), the proposed project cannot be processed as a C2 CE, if it involves -

- a. Acquisition of more than a minor amount of right-of-way

- b. Residential or non-residential displacements

- c. A Coast Guard, Individual Section 404 and/or a Section 10 permit

- d. A Section 106 finding of Adverse Effect

- e. A Section 4(f) Programmatic or Individual Evaluation

- f. A finding of May Affect, Likely to Adversely Affect to Threatened and Endangered Species

- g. Construction of temporary access, or the closure of existing road, bridge, or ramps, that would result in major traffic disruptions

- h. Changes in access control

- i. Floodplain encroachment other than functionally dependent uses (e.g., bridges, wetlands) or actions that facilitate open space use (e.g., recreational trails, bicycle and pedestrian paths)

- j. Construction activities in, across or adjacent to a river component designated or proposed for inclusion in the National System of Wild and Scenic Rivers

- k. No minor public or agency controversy on environmental grounds (no opposition from any organized groups or agencies and no unresolved environmental coordination)

- l. If an EJ Analysis Report is required, the project must be processed as a D-level CE or higher level document

For certification purposes, documentation is required to illustrate no significant impacts will occur to the following environmental resources and that no unusual circumstances exist that would warrant a higher level of NEPA document. Upload all supporting documentation to the project file.

| Waterways: | Present; No Coast Guard, Individual 404, and/or Section 10 Permit required |
| --- | --- |

| Waterways Permit Type: | Permit Determination and/or Permit Application Approval Pending |
| --- | --- |

| Isolated Wetland Permit | No |
| --- | --- |

| Endangered Species: | Present; No finding of May Affect, Likely to Adversely Affect |
| --- | --- |

| Endangered Species - Coordination | May Affect, Not Likely to Adversely Affect |
| --- | --- |

| Endangered Species - Coordination Date | 09/28/2015 |
| --- | --- |

| Endangered Species - Critical Habitat Present/Impacted | Indiana bat |
| --- | --- |
|  | Northern long-eared bat |

| Endangered Species - Other Critical Habitat Present/Impacted: | No |
| --- | --- |

| 100-Year Floodplain: | Encroachment Within the SFHA is a Functionally Dependent Use |
| --- | --- |

| EO 11988/NFIP Coordination and Documentation Completed: | Yes |
| --- | --- |

| NFIP Local Floodplain Coordinator Notification Date: | 01/27/2017 |
| --- | --- |

| Section 4(f): | Present; No Impacts and/or Access Restrictions |
| --- | --- |

| Section 6(f): | Not present |
| --- | --- |

| Cultural Resources: | Present; No Finding of Adverse Effect |
| --- | --- |

| Cultural Resources Coordination: | Minimum Potential to Cause Effect - Appendix B |
| --- | --- |

| Cultural Resources Coordination - ODOT Approval Date: | 09/28/2015 |
| --- | --- |

Projects that meet C2 criteria are not anticipated to have impacts to the following environmental resources. If resources are present, documentation is only required if there is a potential for impacts.

| Air Quality: | Studies Not Required |
| --- | --- |

| Air Quality - Coordination with OES: | No |
| --- | --- |

| Noise: | Studies Not Required |
| --- | --- |

| Noise Coordination - OES Approval Date: |  |
| --- | --- |

| Hazardous Materials - ESA Screening Conducted | Yes |
| --- | --- |

| Hazardous Materials - OES Approval Date: | 01/20/2016 |
| --- | --- |

| Phase I ESA Warranted Based on Coordination with OES: | No Further Studies Warranted |
| --- | --- |

| Farmland: | Non-Urbanized Area; No Impacts in Accordance With the Farmland MOU, 7 CFR 658, and ORC 929.05 |
| --- | --- |

| Scenic Rivers | No National Wild and Scenic River Within 1000 Feet of the Proposed Project Area |
| --- | --- |

Projects that meet C2 criteria must be in accordance with ODOT's EJ Guidance and activities conducted for Public Involvement are commensurate to the project's type and scope of work.

| Environmental Justice: | Does Not Exceed EJ Guidance Criteria; No EJ Analysis Report Required and No EJ Issues Raised During Public Involvement |
| --- | --- |

| Public Involvement: | Minimum PI Requirements Met; No Minor Public or Agency Controversy on Environmental Grounds |
| --- | --- |

| Environmental Commitments | Yes |
| --- | --- |

Environmental Commitments,

1. THE CONTRACTOR WILL ADVISE THE PROJECT ENGINEER A MINIMUM OF TWENTY-ONE (21) DAYS PRIOR TO THE START OF CONSTRUCTION ACTIVITIES.  THE CONTRACTOR MUST ALSO PROVIDE NOTIFICATION TO THE PROJECT ENGINEER A MINIMUM OF FOURTEEN (14) DAYS PRIOR TO ANY LANE RESTRICTIONS OR CLOSURES.  THE PROJECT ENGINEER WILL FORWARD THIS INFORMATION TO THE PORTAGE COUNTY ENGINEER'S OFFICE FOR USE TO NOTIFY EMERGENCY SERVICES AND THE PUBLIC A MINIMUM OF FOURTEEN (14) DAYS PRIOR TO THE START OF PROJECT CONSTRUCTION.  INCLUDED IN THIS NOTIFICATION WILL BE THE PROPOSED LANE RESTRICTIONS, ROAD CLOSURES AND DETOURS REQUIRED BY THE PROJECT.

2. ALL PROJECTS INVOLVING JURISDICTIONAL WATERS OF THE UNITED STATES (STREAMS, RIVERS, NON-ISOLATED WETLANDS) AND/OR ISOLATED WETLANDS ARE SUBJECT TO REGULATION UNDER SECTIONS 404 AND 401 OF THE CLEAN WATER ACT, AND POSSIBLY OHIO EPA ISOLATED WETLAND LAW.  THE WATERWAY PERMITS CONDITIONS VERIFIED FOR THE PROJECT HAVE BEEN INCORPORATED INTO THE PROJECT CONSTRUCTION PLAN AS SPECIAL PROVISIONS OR WILL BE PROVIDED TO THE PROJECT CONTRACTOR PRIOR TO PROJECT CONSTRUCTION. THE PROJECT CONTRACTOR SHALL ADHERE TO ALL WATERWAY PERMIT TERMS AND CONDITIONS THROUGHOUT PROJECT CONSTRUCTION.

3. UNDER THE DIRECTION OF THE PROJECT ENGINEER, THE CONTRACTOR MAY PLACE PERMANENT FILL IN WETLANDS AS INDICATED IN THE PLAN. PLACEMENT OF TEMPORARY FILLS IN WETLANDS IS PROHIBITED.

UNDER NO CIRCUMSTANCES SHALL CONSTRUCTION EQUIPMENT AND/OR MATERIALS BE OPERATED OR STORED WITHIN WETLAND A, WETLAND E AND/OR THE UN-IMPACTED AREAS OF WETLAND B, WETLAND C, WETLAND D AND WETLAND F AS INDICATED IN THE PLAN.

TO PROTECT AND DELINEATE THE BOUNDARY OF THE EXISTING REMAINING WETLANDS, CONSTRUCTION FENCE AND PERIMETER FILTER FABRIC FENCE SHALL BE INSTALLED ALONG THE PROPOSED CONSTRUCTION LIMITS WITHIN THE WETLANDS AREAS BY THE CONTRACTOR PRIOR TO THE START OF ANY CONSTRUCTION ACTIVITIES WITHIN THE LIMITS AND ADJACENT AREA, INCLUDING ANY NECESSARY CLEARING AND GRUBBING ACTIVITIES.  THE CONSTRUCTION FENCE AND PERIMETER FILTER FABRIC FENCE SHALL BE MAINTAINED BY THE CONTRACTOR THROUGHOUT PROJECT CONSTRUCTION AND SHALL BE REMOVED BY THE CONTRACTOR UPON PROJECT COMPLETION.  UNDER NO CIRCUMSTANCES SHALL THE CONTRACTOR IMPACT THE REMAINING WETLANDS AREAS BEYOND THE CONSTRUCTION LIMITS.  UNDER NO CIRCUMSTANCES SHALL THE CONTRACTOR STORE EQUIPMENT AND/OR MATERIALS WITHIN WETLANDS AREAS.

4. WATER COLUMN AND SEDIMENTATION IMPACTS SHALL BE KEPT TO A MINIMUM THROUGH THE USE OF BEST MANAGEMENT PRACTICES FOR SOIL EROSION AND SEDIMENTATION CONTROL.  ALL SOIL EROSION AND SEDIMENT CONTROL MEASURES SHALL BE IN PLACE PRIOR TO ANY EXCAVATION, GRADING OR FILLING OPERATIONS AND INSTALLATION OF PROPOSED STRUCTURES OR UTILITIES.  THEY SHALL REMAIN IN PLACE UNTIL CONSTRUCTION IS COMPLETED AND THE AREA IS STABILIZED AS ACCEPTED BY THE ENGINEER.  SPECIFICATIONS SET FORTH IN THE MOST CURRENT VERSION OF ODOT'S CONSTRUCTION AND MATERIAL SPECIFICATIONS, LOCATION AND DESIGN MANUAL AND STANDARD DRAWINGS SHALL BE USED TO ENSURE ADEQUATE EROSION AND SEDIMENT CONTROL DURING CONSTRUCTION. ALL SEDIMENT AND EROSION CONTROLS SHALL BE REMOVED UPON STABILIZATION OF THE PROJECT AREA WITH VEGETATION AND ACCPTANCE OF THE PROJECT ENGINEER.

5. NATURAL BUFFERS ADJACENT TO STREAMS AND WETLANDS SHALL BE PRESERVED TO THE MAXIMUM EXTENT PRACTICABLE.  EXISTING RIPARIAN HABITATS SHOULD BE MAINTAINED TO THE MAXIMUM EXTENT PRACTICABLE AND EQUIPMENT STAGING AREAS SHALL BE KEPT WELL AWAY FROM STREAMS AND WETLANDS TO THE EXTENT PRACTICABLE.  ODOT CONSTRUCTION AND MATERIALS SPECIFICATIONS SECTION 107.10 (PROTECTION AND RESTORATION OF PROPERTY) PROHIBIT THE CONTRACTOR FROM CREATING STAGING AREAS NEAR STREAMS/WETLANDS.

6. AREAS DISTURBED BY THE PROJECT SHALL BE SEEDED/REVEGETATED AND MULCHED DURING CONSTRUCTION TO ENCOURAGE ESTABLISHMENT OF VEGETATION COVER, DECREASE EROSION AND PREVENT EROSION OF SEDIMENTS INTO WATERS OF THE UNITED STATES.

7. THE CONTRACTOR SHALL TAKE ALL PRECAUTIONS NECESSARY TO PREVENT ANY INCIDENTAL DISCHARGES ASSOCIATED WITH THE EXCAVATION AND HAULING OF MATERIAL FROM STREAMS AND DITCHES.  THIS PERTAINS TO ANY EXCAVATION OPERATION SUCH AS, FOUNDATION PIER OR ABUTMENT EXCAVATION, CHANNEL CLEAN OUT, EXCAVATION FOR ROCK CHANNEL PROTECTION.

8. ALL MATERIALS REMOVED FROM STREAMS AND/OR DITCHES MUST BE IMMEDIATELY REMOVED TO AN UPLAND SITE AND STABILIZED (I.E., SEEDED) TO PREVENT REDISTRIBUTION INTO ANY WATERS OF THE UNITED STATES. IMMEDIATE REMOVAL IS DEFINED BY THE UNITED STATES ARMY CORPS OF ENGINEERS AS DEPOSITING THE REMOVED MATERIALS DIRECTLY INTO A TRUCK AND REMOVING THE MATERIAL FROM THE SITE. PLACEMENT OF REMOVED MATERIALS INTO A WETLANDS OR ON THE BANKS OF A STREAM EVEN TEMPORARILY IS CONSIDERED A FILL AND REQUIRES A PERMIT ACTION.  THE CONTRACTOR SHALL UTILIZE AND APPROVED DISPOSAL SITE FOR THE DISPOSAL OF ALL WASTE SOILS AND OTHER MATERIALS.  THE APPROVED DISPOSAL SITE SHALL NOT INCLUDE ANY WETLANDS, STREAMBANKS OR 100-YEAR FLOOD PLAINS.

9. THE MECHANICAL EQUIPMENT USED TO EXECUTE THE WORK AUTHORIZED HEREIN SHALL BE OPERATED IN SUCH A WAY AS TO MINIMIZE TURBIDITY THAT COULD DEGRADE WATER QUALITY AND ADVERSELY AFFECT AQUATIC PLANT AND ANIMAL LIFE.

10. THE CONTRACTOR SHALL TAKE ALL PRECAUTIONS NECESSARY TO PREVENT EPOXY-URETHANE SEALER, PAINT OR OTHER MATERIALS USED TO REPAIR, CLEAN, PAINT, SEAL OR TREAT ANY STRUCTURE FROM ENTERING ANY STREAMS, WETLANDS OR OTHER WATERS OF THE UNITED STATES AND TAKE THE APPROPRIATE ACTIONS IN THE EVENT OF A RELEASE.

11. THE PROJECT IS LOCATED WITHIN THE KNOWN HABITAT RANGES OF THE FEDERALLY LISTED AND PROTECTED INDIANA BAT AND NORTHERN LONG-EARED BAT.  NO TREES SHALL BE REMOVED UNDER THIS PROJECT FROM APRIL 1 THROUGH SEPTEMBER 30.  ALL NECESSARY TREE REMOVAL SHALL OCCUR FROM OCTOBER 1 THROUGH MARCH 31.  THIS REQUIREMENT IS NECESSARY TO AVOID AND MINIMIZE IMPACTS TO THESE SPECIES AS REQUIRED BY THE ENDANGERED SPECIES ACT.  FOR THE PURPOSES OF THIS NOTE, A TREE IS DEFINED AS A LIVE, DYING, OR DEAD WOODY PLANT, WITH A TRUNK THREE INCHES OR GREATER IN DIAMETER AT A HEIGHT OF 4.5 FEET ABOVE THE GROUND SURFACE, AND WITH A MINIMUM HEIGHT OF 13 FEET.

PRIOR TO DEMOLITION AND REMOVAL OF THE POR-C293A-NL092 BRIDGE (SFN: 6733360), THE UNDERSIDE OF EACH BRIDGE SHALL BE CAREFULLY EXAMINED FOR THE PRESENCE OF BATS, ESPECIALLY FROM APRIL 15 TO SEPTEMBER 15.  IF ANY BATS ARE FOUND ROOSTING ON THE UNDERSIDE OF THE BRIDGE, ODOT-OES (614-466-7100) SHALL BE NOTIFIED AND THE US FISH AND WILDLIFE SERVICE, ECOLOGICAL SERVICES DIVISION SHALL BE CONTACTED OR PROVIDED WITH INFORMATION.

12. THE OHIO DEPARTMENT OF NATURAL RESOURCES, DIVISION OF NATURAL AREA & PRESERVES, MUST RECEIVE PROJECT NOTIFICATION. THE PORTAGE COUNTY ENGINEER'S OFFICE SHALL NOTIFY THE OHIO DEPARTMENT OF NATURAL RESOURCES, DIVISION OF NATURAL AREA & PRESERVES IN WRITING OF THE PROJECT PRECONSTRUCTION MEETING AT LEAST FIFTEEN (15) CALENDAR DAYS PRIOR TO THE MEETING.  FURTHER, AT LEAST FIFTEEN (15) CALENDAR DAYS PRIOR TO THE BEGINNING OF ANY CONSTRUCTION WORK, INCLUDING INSTALLATION OF MAINTENANCE OF TRAFFIC SIGNAGE, STAGING OF EQUIPMENT AND/OR MATERIALS, ETC., WITHIN 1000 FEET OF THE EAGLE CREEK STATE NATURE PRESERVE, THE CONTRACTOR SHALL PROVIDE WRITTEN NOTIFICATION TO THE ODNR DIVISION OF NATURAL AREAS AND PRESERVES, NORTHEAST REGIONAL PRESERVE MANAGER, ADAM WOHLEVER.

Adam.Wohlever@dnr.state.oh.us or (440) 476-2511

INFORMATION REQUIRED AS PART OF THE PRECONSTRUCTION NOTIFICATION SHALL INCLUDE:

1) THE CONTRACTORS NAME AND ADDRESS

2) CONTRACTOR, PORTAGE COUNTY ENGINEER AND ODOT, DISTRICT 4 CONSTRUCTION REPRESENTATIVE CONTACT INFORMATION

3) THE SCHEDULED DATES FOR THE START AND COMPLETION OF CONSTRUCTION OPERATIONS

4) ONE COPY OF THE CONSTRUCTION PLANS.

MR. WOHLEVER SHALL ALSO BE IMMEDIATELY NOTIFIED UPON ANY POTENTIAL NEGATIVE IMPACTS AND IMMEDIATELY FOLLOWING THE COMPLETION OF WORK IN THE AREAS OF CONCERN (AREAS IN PROXIMITY OF THE EAGLE CREEK STATE NATURE PRESERVE) FOR INSPECTION.

13. UNDER NO CIRCUMSTANCES SHALL ANY EQUIPMENT (BACKHOE, EARTH MOVING EQUIPMENT, ETC.) AND/OR MATERIALS ENTER THE EAGLE CREEK STATE NATURE PRESERVE.  THE CONTRACTOR SHALL TAKE ALL PRECAUTIONS NECESSARY TO PREVENT ALL CONSTRUCTION MATERIALS, WASTE MATERIALS, WATER, CHEMICALS OR OTHER SUBSTANCES USED TO CONSTRUCT THE PROJECT FROM ENTERING DITCHES, STREAMS, WETLANDS OR ANY OTHER SURFACE WATER OR STORM WATER CONVEYANCE TO THE EAGLE CREEK STATE NATURE PRESERVE.  TO PROTECT AND DELINEATE THE BOUNDARY OF THE EAGLE CREEK STATE NATURE PRESERVE, A QUANTITY OF ITEM 832- EROSION CONTROL: FILTER FABRIC FENCE AND ITEM 607 FENCE MISC.: CONSTRUCTION FENCE WILL BE USED.  THE FILTER FABRIC FENCE (THIS ITEM WILL BE PAID FOR UNDER ITEM 832 - EROSION CONTROL) AND CONSTRUCTION FENCE SHALL BE INSTALLED AROUND THE PERIMETER OF THE CONSTRUCTION LIMITS IDENTIFIED IN THE PLANS WITHIN THE CENTER ROAD RIGHT OF WAY MAINTAINING A BUFFER BETWEEN THE FILTER FABRIC FENCE AND CONSTRUCTION FENCE AND THE CONSTRUCTION LIMITS IDENTIFIED IN THE PLAN.  ALL FABRIC FENCE AND CONSTRUCTION FENCE SHALL BE INSTALLED PRIOR TO CONSTRUCTION ACTIVITIES.  THIS INCLUDES ANY NECESSARY CLEARING AND GRUBBING ACTIVITIES.  THE FENCE SHALL BE MAINTAINED THROUGHOUT THE DURATION OF THE PROJECT AND REMOVED BY THE CONTRACTOR AT THE COMPLETION OF THE PROJECT.

APPROPRIATELY DESIGNED EROSION CONTROLS SHALL BE IN PLACE PRIOR TO ANY EXCAVATION, GRADING OR FILLING OPERATIONS AND INSTALLATION OF PROPOSED STRUCTURES AND/OR UTILITIES.  ALL SOIL EROSION AND SEDIMENT CONTROL MEASURES SHALL BE PROPERLY MAINTAINED UNTIL FINAL PROJECT SITE STABILIZATION IS ACHIEVED AND ACCEPTED BY THE ENGINEER.  EQUIPMENT AND MATERIAL STAGING AREAS SHALL BE KEPT AWAY FROM THE EAGLE CREEK STATE NATURE PRESERVE TO THE EXTENT PRACTICABLE. IDLE EQUIPMENT, PETROCHEMICALS AND TOXIC/HAZARDOUS MATERIALS SHOULD NOT BE STORED IN PROXIMITY OF THE EAGLE CREEK STATE NATURE PRESERVE, STREAMS, WETLANDS AND OTHER WATERS OF THE UNITED STATES.  ALL PROJECT RELATED REFUELING AND MAINTENANCE ACTIVITIES SHALL BE PERFORMED IN AN ENVIRONMENTALLY RESPONSIBLE MANNER AND UNDER NO CIRCUMSTANCES SHALL THE CONTRACTOR DISCHARGE ANY PETROCHEMICALS AND/OR TOXIC AND HAZARDOUS MATERIALS.

ALL WASTE MATERIALS GENERATED DURING CONSTRUCTION ACTIVITIES SHALL BE IMMEDIATELY REMOVED FROM THE CONSTRUCTION SITE.  IMMEDIATE REMOVAL IS DEFINED AS DEPOSITING THE REMOVED MATERIAL DIRECTLY INTO A TRUCK AND REMOVING THE MATERIALS FROM THE SITE; PLACEMENT OF REMOVED MATERIALS WITHIN 1,000 FEET OF THE EAGLE CREEK STATE NATURE PRESERVE IS PROHIBITED. PLACEMENT OF REMOVED MATERIAL INTO A WETLAND OR ON THE BANKS OF A STREAM OR RIVER, OR WITHIN A 100-YEAR FLOOD PLAIN EVEN TEMPORARILY IS CONSIDERED A FILL AND IS PROHIBITED.

THE CONTRACTOR SHALL PROVIDE WRITTEN NOTIFICATION TO THE PROJECT ENGINEER FOLLOWING COMPLETION OF CONSTRUCTION IN PROXIMITY OF THE EAGLE CREEK STATE NATURE PRESERVE.  THE PROJECT ENGINEER SHALL INSPECT THE AREA AND COORDINATE WITH THE ODNR DIVISION OF NATURAL AREAS AND PRESERVES, NORTHEAST REGIONAL PRESERVE MANAGER, ADAM WOHLEVER TO ENSURE AREAS DISTURBED DURING CONSTRUCTION ARE PROPERLY STABILIZED AND REVEGETATED.

14. FEDERALLY THREATENED SPECIES - NORTHERN MONKSHOOD:

THIS PROJECT LIES WITHIN THE RANGE OF NORTHERN MONKSHOOD (ACONITUM NOVEBORACENSE), A FEDERALLY LISTED THREATENED SPECIES. THE PLANT IS KNOWN TO OCCUR 0.09-MILE SOUTH OF THE PROJECT AREA.  THERE IS A SWPPP PLAN (SWPPP TO BE DEVELOPED BY THE PROJECT CONTRACTOR) IN PLACE FOR THE PROTECTION OF THIS SPECIES, PLEASE REFER TO THE SWPPP FOR FURTHER GUIDANCE OR CONTACT ODOT-OFFICE OF ENVIRONMENTAL SERVICES - ECOLOGICAL SECTION (614-466-7100), RICK GARDNER WITH THE OHIO DEPARTMENT OF NATURAL RESOURCES DIVISION OF NATURAL AREAS AND PRESERVES (614-265-6419) OR THE USFWS COLUMBUS OHIO FIELD OFFICE (614-416-8993) WITH QUESTIONS.

15. THE CONTRACTOR SHALL UTILIZE AN APPROVED DISPOSAL SITE FOR THE DISPOSAL OF ALL WASTE SOILS AND OTHER MATERIALS.  THE APPROVED DISPOSAL SITE SHALL NOT INCLUDE ANY WETLANDS, STREAMBANKS OR 100-YEAR FLOOD PLAINS.

16. The contractor is prohibiting from entering onto the Eagle Creek State Nature Preserve property and/or otherwise impacting the Eagle Creek State Nature Preserve. The contractor shall take all precautions necessary to prevent all construction materials, waste materials, water, chemicals or other substances used to construct the project from entering ditches, streams, wetlands or any other surface water or storm water conveyance to the Eagle Creek State Nature Preserve. Existing natural buffers shall be maintained to maximum extent practicable and equipment and staging areas shall be kept well away Eagle Creek State Nature Preserve property. Culverts and structures that match stream characteristics of capacity, slope and orientation shall be designed for the project and placed to allow free movement of aquatic fauna.  Incorporation of SWPPP and BMPs from ODOT's Construction and materials Specification Handbook is required for the project.  Plan notes shall be incorporated into the construction plan prohibiting the contractor from entering onto the Eagle Creek State Nature Preserve property and/or otherwise impacting the Eagle Creek State Nature Preserve.

17. Impacts to streams and wetlands will be permitted and mitigated under the current Clean Water Act requirements as regulated by the US Army Corps of Engineers and Ohio EPA regulations (33 CFR parts 325 and 332 and 40 CFR part 230, and OAC 3745-01-54), and Ohio Isolated Wetland Law (ORC 6111.027).

18. Equipment and material staging areas shall be kept away from the Eagle Creek State Nature Preserve to the extent practicable. Idle equipment. Petrochemicals and toxic/hazardous materials should not be stored in proximity of the Eagle Creek State Nature Preserve.  All project related refueling and maintenance activities shall be performed in an environmentally responsible manner.

19. Waste materials generated during construction shall be immediately removed.  Placement of removed materials within 1,000 feet of the Eagle Creek State Nature Preserve shall be prohibited.

20. All aquatic resources indicated on the plans shall be demarcated in the field as per ODOT Supplemental Specification 832 and the applicable waterway permits special provisions prior to site disturbance. Materials utilized in, or adjacent to, aquatic resources for temporary or permanent fill or bank protection shall consist of suitable material free from toxic contaminants in other than trace quantities.  Broken asphalt is specifically excluded.  Chromated Copper Arsenate (CCA), and other pressure treated lumber shall not be used in structures that are placed in aquatic resources. Furthermore, an oil spill kit will be located within 150 feet of any equipment working in a stream or wetland.  The oil spill kit shall be maintained for the life of the construction contract.

21. Soil erosion and sediment control measures will be in place prior to construction operations and shall remain in place until construction is completed and the area is accepted by the engineer. All sediment and erosion controls shall be removed upon stabilization of the project area with vegetation and acceptance of the project engineer. The contractor shall utilize and approved disposal site for the disposal of all waste soils and other materials.  The approved disposal site shall not include any wetlands, streambanks or 100-year flood plains.

22. Reconstructed and disturbed earthen areas shall be seeded/revegetated and mulched during construction to encourage establishment of vegetation cover, decrease erosion and prevent erosion of sediments into waters of the United States.

23. The contractor shall contact the Northeast Regional Preserve Manager, Adam Wohlever, at least one week prior to project commencement. Mr. Wohlever shall also be immediately notified upon any potential negative impacts and immediately following the completion of work in the areas of concern for inspection.

24. Waterway permits shall be submitted to the appropriate agencies and accepted prior to any tree-clearing for the project.

Environmental Commitments

C2

Commitments

1. THE CONTRACTOR WILL ADVISE THE PROJECT ENGINEER A MINIMUM OF TWENTY-ONE (21) DAYS PRIOR TO THE START OF CONSTRUCTION ACTIVITIES.  THE CONTRACTOR MUST ALSO PROVIDE NOTIFICATION TO THE PROJECT ENGINEER A MINIMUM OF FOURTEEN (14) DAYS PRIOR TO ANY LANE RESTRICTIONS OR CLOSURES.  THE PROJECT ENGINEER WILL FORWARD THIS INFORMATION TO THE PORTAGE COUNTY ENGINEER'S OFFICE FOR USE TO NOTIFY EMERGENCY SERVICES AND THE PUBLIC A MINIMUM OF FOURTEEN (14) DAYS PRIOR TO THE START OF PROJECT CONSTRUCTION.  INCLUDED IN THIS NOTIFICATION WILL BE THE PROPOSED LANE RESTRICTIONS, ROAD CLOSURES AND DETOURS REQUIRED BY THE PROJECT.

2. ALL PROJECTS INVOLVING JURISDICTIONAL WATERS OF THE UNITED STATES (STREAMS, RIVERS, NON-ISOLATED WETLANDS) AND/OR ISOLATED WETLANDS ARE SUBJECT TO REGULATION UNDER SECTIONS 404 AND 401 OF THE CLEAN WATER ACT, AND POSSIBLY OHIO EPA ISOLATED WETLAND LAW.  THE WATERWAY PERMITS CONDITIONS VERIFIED FOR THE PROJECT HAVE BEEN INCORPORATED INTO THE PROJECT CONSTRUCTION PLAN AS SPECIAL PROVISIONS OR WILL BE PROVIDED TO THE PROJECT CONTRACTOR PRIOR TO PROJECT CONSTRUCTION. THE PROJECT CONTRACTOR SHALL ADHERE TO ALL WATERWAY PERMIT TERMS AND CONDITIONS THROUGHOUT PROJECT CONSTRUCTION.

3. UNDER THE DIRECTION OF THE PROJECT ENGINEER, THE CONTRACTOR MAY PLACE PERMANENT FILL IN WETLANDS AS INDICATED IN THE PLAN. PLACEMENT OF TEMPORARY FILLS IN WETLANDS IS PROHIBITED.

UNDER NO CIRCUMSTANCES SHALL CONSTRUCTION EQUIPMENT AND/OR MATERIALS BE OPERATED OR STORED WITHIN WETLAND A, WETLAND E AND/OR THE UN-IMPACTED AREAS OF WETLAND B, WETLAND C, WETLAND D AND WETLAND F AS INDICATED IN THE PLAN.

TO PROTECT AND DELINEATE THE BOUNDARY OF THE EXISTING REMAINING WETLANDS, CONSTRUCTION FENCE AND PERIMETER FILTER FABRIC FENCE SHALL BE INSTALLED ALONG THE PROPOSED CONSTRUCTION LIMITS WITHIN THE WETLANDS AREAS BY THE CONTRACTOR PRIOR TO THE START OF ANY CONSTRUCTION ACTIVITIES WITHIN THE LIMITS AND ADJACENT AREA, INCLUDING ANY NECESSARY CLEARING AND GRUBBING ACTIVITIES.  THE CONSTRUCTION FENCE AND PERIMETER FILTER FABRIC FENCE SHALL BE MAINTAINED BY THE CONTRACTOR THROUGHOUT PROJECT CONSTRUCTION AND SHALL BE REMOVED BY THE CONTRACTOR UPON PROJECT COMPLETION.  UNDER NO CIRCUMSTANCES SHALL THE CONTRACTOR IMPACT THE REMAINING WETLANDS AREAS BEYOND THE CONSTRUCTION LIMITS.  UNDER NO CIRCUMSTANCES SHALL THE CONTRACTOR STORE EQUIPMENT AND/OR MATERIALS WITHIN WETLANDS AREAS.

4. WATER COLUMN AND SEDIMENTATION IMPACTS SHALL BE KEPT TO A MINIMUM THROUGH THE USE OF BEST MANAGEMENT PRACTICES FOR SOIL EROSION AND SEDIMENTATION CONTROL.  ALL SOIL EROSION AND SEDIMENT CONTROL MEASURES SHALL BE IN PLACE PRIOR TO ANY EXCAVATION, GRADING OR FILLING OPERATIONS AND INSTALLATION OF PROPOSED STRUCTURES OR UTILITIES.  THEY SHALL REMAIN IN PLACE UNTIL CONSTRUCTION IS COMPLETED AND THE AREA IS STABILIZED AS ACCEPTED BY THE ENGINEER.  SPECIFICATIONS SET FORTH IN THE MOST CURRENT VERSION OF ODOT'S CONSTRUCTION AND MATERIAL SPECIFICATIONS, LOCATION AND DESIGN MANUAL AND STANDARD DRAWINGS SHALL BE USED TO ENSURE ADEQUATE EROSION AND SEDIMENT CONTROL DURING CONSTRUCTION. ALL SEDIMENT AND EROSION CONTROLS SHALL BE REMOVED UPON STABILIZATION OF THE PROJECT AREA WITH VEGETATION AND ACCPTANCE OF THE PROJECT ENGINEER.

5. NATURAL BUFFERS ADJACENT TO STREAMS AND WETLANDS SHALL BE PRESERVED TO THE MAXIMUM EXTENT PRACTICABLE.  EXISTING RIPARIAN HABITATS SHOULD BE MAINTAINED TO THE MAXIMUM EXTENT PRACTICABLE AND EQUIPMENT STAGING AREAS SHALL BE KEPT WELL AWAY FROM STREAMS AND WETLANDS TO THE EXTENT PRACTICABLE.  ODOT CONSTRUCTION AND MATERIALS SPECIFICATIONS SECTION 107.10 (PROTECTION AND RESTORATION OF PROPERTY) PROHIBIT THE CONTRACTOR FROM CREATING STAGING AREAS NEAR STREAMS/WETLANDS.

6. AREAS DISTURBED BY THE PROJECT SHALL BE SEEDED/REVEGETATED AND MULCHED DURING CONSTRUCTION TO ENCOURAGE ESTABLISHMENT OF VEGETATION COVER, DECREASE EROSION AND PREVENT EROSION OF SEDIMENTS INTO WATERS OF THE UNITED STATES.

7. THE CONTRACTOR SHALL TAKE ALL PRECAUTIONS NECESSARY TO PREVENT ANY INCIDENTAL DISCHARGES ASSOCIATED WITH THE EXCAVATION AND HAULING OF MATERIAL FROM STREAMS AND DITCHES.  THIS PERTAINS TO ANY EXCAVATION OPERATION SUCH AS, FOUNDATION PIER OR ABUTMENT EXCAVATION, CHANNEL CLEAN OUT, EXCAVATION FOR ROCK CHANNEL PROTECTION.

8. ALL MATERIALS REMOVED FROM STREAMS AND/OR DITCHES MUST BE IMMEDIATELY REMOVED TO AN UPLAND SITE AND STABILIZED (I.E., SEEDED) TO PREVENT REDISTRIBUTION INTO ANY WATERS OF THE UNITED STATES. IMMEDIATE REMOVAL IS DEFINED BY THE UNITED STATES ARMY CORPS OF ENGINEERS AS DEPOSITING THE REMOVED MATERIALS DIRECTLY INTO A TRUCK AND REMOVING THE MATERIAL FROM THE SITE. PLACEMENT OF REMOVED MATERIALS INTO A WETLANDS OR ON THE BANKS OF A STREAM EVEN TEMPORARILY IS CONSIDERED A FILL AND REQUIRES A PERMIT ACTION.  THE CONTRACTOR SHALL UTILIZE AND APPROVED DISPOSAL SITE FOR THE DISPOSAL OF ALL WASTE SOILS AND OTHER MATERIALS.  THE APPROVED DISPOSAL SITE SHALL NOT INCLUDE ANY WETLANDS, STREAMBANKS OR 100-YEAR FLOOD PLAINS.

9. THE MECHANICAL EQUIPMENT USED TO EXECUTE THE WORK AUTHORIZED HEREIN SHALL BE OPERATED IN SUCH A WAY AS TO MINIMIZE TURBIDITY THAT COULD DEGRADE WATER QUALITY AND ADVERSELY AFFECT AQUATIC PLANT AND ANIMAL LIFE.

10. THE CONTRACTOR SHALL TAKE ALL PRECAUTIONS NECESSARY TO PREVENT EPOXY-URETHANE SEALER, PAINT OR OTHER MATERIALS USED TO REPAIR, CLEAN, PAINT, SEAL OR TREAT ANY STRUCTURE FROM ENTERING ANY STREAMS, WETLANDS OR OTHER WATERS OF THE UNITED STATES AND TAKE THE APPROPRIATE ACTIONS IN THE EVENT OF A RELEASE.

11. THE PROJECT IS LOCATED WITHIN THE KNOWN HABITAT RANGES OF THE FEDERALLY LISTED AND PROTECTED INDIANA BAT AND NORTHERN LONG-EARED BAT.  NO TREES SHALL BE REMOVED UNDER THIS PROJECT FROM APRIL 1 THROUGH SEPTEMBER 30.  ALL NECESSARY TREE REMOVAL SHALL OCCUR FROM OCTOBER 1 THROUGH MARCH 31.  THIS REQUIREMENT IS NECESSARY TO AVOID AND MINIMIZE IMPACTS TO THESE SPECIES AS REQUIRED BY THE ENDANGERED SPECIES ACT.  FOR THE PURPOSES OF THIS NOTE, A TREE IS DEFINED AS A LIVE, DYING, OR DEAD WOODY PLANT, WITH A TRUNK THREE INCHES OR GREATER IN DIAMETER AT A HEIGHT OF 4.5 FEET ABOVE THE GROUND SURFACE, AND WITH A MINIMUM HEIGHT OF 13 FEET.

PRIOR TO DEMOLITION AND REMOVAL OF THE POR-C293A-NL092 BRIDGE (SFN: 6733360), THE UNDERSIDE OF EACH BRIDGE SHALL BE CAREFULLY EXAMINED FOR THE PRESENCE OF BATS, ESPECIALLY FROM APRIL 15 TO SEPTEMBER 15.  IF ANY BATS ARE FOUND ROOSTING ON THE UNDERSIDE OF THE BRIDGE, ODOT-OES (614-466-7100) SHALL BE NOTIFIED AND THE US FISH AND WILDLIFE SERVICE, ECOLOGICAL SERVICES DIVISION SHALL BE CONTACTED OR PROVIDED WITH INFORMATION.

12. THE OHIO DEPARTMENT OF NATURAL RESOURCES, DIVISION OF NATURAL AREA & PRESERVES, MUST RECEIVE PROJECT NOTIFICATION. THE PORTAGE COUNTY ENGINEER'S OFFICE SHALL NOTIFY THE OHIO DEPARTMENT OF NATURAL RESOURCES, DIVISION OF NATURAL AREA & PRESERVES IN WRITING OF THE PROJECT PRECONSTRUCTION MEETING AT LEAST FIFTEEN (15) CALENDAR DAYS PRIOR TO THE MEETING.  FURTHER, AT LEAST FIFTEEN (15) CALENDAR DAYS PRIOR TO THE BEGINNING OF ANY CONSTRUCTION WORK, INCLUDING INSTALLATION OF MAINTENANCE OF TRAFFIC SIGNAGE, STAGING OF EQUIPMENT AND/OR MATERIALS, ETC., WITHIN 1000 FEET OF THE EAGLE CREEK STATE NATURE PRESERVE, THE CONTRACTOR SHALL PROVIDE WRITTEN NOTIFICATION TO THE ODNR DIVISION OF NATURAL AREAS AND PRESERVES, NORTHEAST REGIONAL PRESERVE MANAGER, ADAM WOHLEVER.

Adam.Wohlever@dnr.state.oh.us or (440) 476-2511

INFORMATION REQUIRED AS PART OF THE PRECONSTRUCTION NOTIFICATION SHALL INCLUDE:

1) THE CONTRACTORS NAME AND ADDRESS

2) CONTRACTOR, PORTAGE COUNTY ENGINEER AND ODOT, DISTRICT 4 CONSTRUCTION REPRESENTATIVE CONTACT INFORMATION

3) THE SCHEDULED DATES FOR THE START AND COMPLETION OF CONSTRUCTION OPERATIONS

4) ONE COPY OF THE CONSTRUCTION PLANS.

MR. WOHLEVER SHALL ALSO BE IMMEDIATELY NOTIFIED UPON ANY POTENTIAL NEGATIVE IMPACTS AND IMMEDIATELY FOLLOWING THE COMPLETION OF WORK IN THE AREAS OF CONCERN (AREAS IN PROXIMITY OF THE EAGLE CREEK STATE NATURE PRESERVE) FOR INSPECTION.

13. UNDER NO CIRCUMSTANCES SHALL ANY EQUIPMENT (BACKHOE, EARTH MOVING EQUIPMENT, ETC.) AND/OR MATERIALS ENTER THE EAGLE CREEK STATE NATURE PRESERVE.  THE CONTRACTOR SHALL TAKE ALL PRECAUTIONS NECESSARY TO PREVENT ALL CONSTRUCTION MATERIALS, WASTE MATERIALS, WATER, CHEMICALS OR OTHER SUBSTANCES USED TO CONSTRUCT THE PROJECT FROM ENTERING DITCHES, STREAMS, WETLANDS OR ANY OTHER SURFACE WATER OR STORM WATER CONVEYANCE TO THE EAGLE CREEK STATE NATURE PRESERVE.  TO PROTECT AND DELINEATE THE BOUNDARY OF THE EAGLE CREEK STATE NATURE PRESERVE, A QUANTITY OF ITEM 832- EROSION CONTROL: FILTER FABRIC FENCE AND ITEM 607 FENCE MISC.: CONSTRUCTION FENCE WILL BE USED.  THE FILTER FABRIC FENCE (THIS ITEM WILL BE PAID FOR UNDER ITEM 832 - EROSION CONTROL) AND CONSTRUCTION FENCE SHALL BE INSTALLED AROUND THE PERIMETER OF THE CONSTRUCTION LIMITS IDENTIFIED IN THE PLANS WITHIN THE CENTER ROAD RIGHT OF WAY MAINTAINING A BUFFER BETWEEN THE FILTER FABRIC FENCE AND CONSTRUCTION FENCE AND THE CONSTRUCTION LIMITS IDENTIFIED IN THE PLAN.  ALL FABRIC FENCE AND CONSTRUCTION FENCE SHALL BE INSTALLED PRIOR TO CONSTRUCTION ACTIVITIES.  THIS INCLUDES ANY NECESSARY CLEARING AND GRUBBING ACTIVITIES.  THE FENCE SHALL BE MAINTAINED THROUGHOUT THE DURATION OF THE PROJECT AND REMOVED BY THE CONTRACTOR AT THE COMPLETION OF THE PROJECT.

APPROPRIATELY DESIGNED EROSION CONTROLS SHALL BE IN PLACE PRIOR TO ANY EXCAVATION, GRADING OR FILLING OPERATIONS AND INSTALLATION OF PROPOSED STRUCTURES AND/OR UTILITIES.  ALL SOIL EROSION AND SEDIMENT CONTROL MEASURES SHALL BE PROPERLY MAINTAINED UNTIL FINAL PROJECT SITE STABILIZATION IS ACHIEVED AND ACCEPTED BY THE ENGINEER.  EQUIPMENT AND MATERIAL STAGING AREAS SHALL BE KEPT AWAY FROM THE EAGLE CREEK STATE NATURE PRESERVE TO THE EXTENT PRACTICABLE. IDLE EQUIPMENT, PETROCHEMICALS AND TOXIC/HAZARDOUS MATERIALS SHOULD NOT BE STORED IN PROXIMITY OF THE EAGLE CREEK STATE NATURE PRESERVE, STREAMS, WETLANDS AND OTHER WATERS OF THE UNITED STATES.  ALL PROJECT RELATED REFUELING AND MAINTENANCE ACTIVITIES SHALL BE PERFORMED IN AN ENVIRONMENTALLY RESPONSIBLE MANNER AND UNDER NO CIRCUMSTANCES SHALL THE CONTRACTOR DISCHARGE ANY PETROCHEMICALS AND/OR TOXIC AND HAZARDOUS MATERIALS.

ALL WASTE MATERIALS GENERATED DURING CONSTRUCTION ACTIVITIES SHALL BE IMMEDIATELY REMOVED FROM THE CONSTRUCTION SITE.  IMMEDIATE REMOVAL IS DEFINED AS DEPOSITING THE REMOVED MATERIAL DIRECTLY INTO A TRUCK AND REMOVING THE MATERIALS FROM THE SITE; PLACEMENT OF REMOVED MATERIALS WITHIN 1,000 FEET OF THE EAGLE CREEK STATE NATURE PRESERVE IS PROHIBITED. PLACEMENT OF REMOVED MATERIAL INTO A WETLAND OR ON THE BANKS OF A STREAM OR RIVER, OR WITHIN A 100-YEAR FLOOD PLAIN EVEN TEMPORARILY IS CONSIDERED A FILL AND IS PROHIBITED.

THE CONTRACTOR SHALL PROVIDE WRITTEN NOTIFICATION TO THE PROJECT ENGINEER FOLLOWING COMPLETION OF CONSTRUCTION IN PROXIMITY OF THE EAGLE CREEK STATE NATURE PRESERVE.  THE PROJECT ENGINEER SHALL INSPECT THE AREA AND COORDINATE WITH THE ODNR DIVISION OF NATURAL AREAS AND PRESERVES, NORTHEAST REGIONAL PRESERVE MANAGER, ADAM WOHLEVER TO ENSURE AREAS DISTURBED DURING CONSTRUCTION ARE PROPERLY STABILIZED AND REVEGETATED.

14. FEDERALLY THREATENED SPECIES - NORTHERN MONKSHOOD:

THIS PROJECT LIES WITHIN THE RANGE OF NORTHERN MONKSHOOD (ACONITUM NOVEBORACENSE), A FEDERALLY LISTED THREATENED SPECIES. THE PLANT IS KNOWN TO OCCUR 0.09-MILE SOUTH OF THE PROJECT AREA.  THERE IS A SWPPP PLAN (SWPPP TO BE DEVELOPED BY THE PROJECT CONTRACTOR) IN PLACE FOR THE PROTECTION OF THIS SPECIES, PLEASE REFER TO THE SWPPP FOR FURTHER GUIDANCE OR CONTACT ODOT-OFFICE OF ENVIRONMENTAL SERVICES - ECOLOGICAL SECTION (614-466-7100), RICK GARDNER WITH THE OHIO DEPARTMENT OF NATURAL RESOURCES DIVISION OF NATURAL AREAS AND PRESERVES (614-265-6419) OR THE USFWS COLUMBUS OHIO FIELD OFFICE (614-416-8993) WITH QUESTIONS.

15. THE CONTRACTOR SHALL UTILIZE AN APPROVED DISPOSAL SITE FOR THE DISPOSAL OF ALL WASTE SOILS AND OTHER MATERIALS.  THE APPROVED DISPOSAL SITE SHALL NOT INCLUDE ANY WETLANDS, STREAMBANKS OR 100-YEAR FLOOD PLAINS.

16. The contractor is prohibiting from entering onto the Eagle Creek State Nature Preserve property and/or otherwise impacting the Eagle Creek State Nature Preserve. The contractor shall take all precautions necessary to prevent all construction materials, waste materials, water, chemicals or other substances used to construct the project from entering ditches, streams, wetlands or any other surface water or storm water conveyance to the Eagle Creek State Nature Preserve. Existing natural buffers shall be maintained to maximum extent practicable and equipment and staging areas shall be kept well away Eagle Creek State Nature Preserve property. Culverts and structures that match stream characteristics of capacity, slope and orientation shall be designed for the project and placed to allow free movement of aquatic fauna.  Incorporation of SWPPP and BMPs from ODOT's Construction and materials Specification Handbook is required for the project.  Plan notes shall be incorporated into the construction plan prohibiting the contractor from entering onto the Eagle Creek State Nature Preserve property and/or otherwise impacting the Eagle Creek State Nature Preserve.

17. Impacts to streams and wetlands will be permitted and mitigated under the current Clean Water Act requirements as regulated by the US Army Corps of Engineers and Ohio EPA regulations (33 CFR parts 325 and 332 and 40 CFR part 230, and OAC 3745-01-54), and Ohio Isolated Wetland Law (ORC 6111.027).

18. Equipment and material staging areas shall be kept away from the Eagle Creek State Nature Preserve to the extent practicable. Idle equipment. Petrochemicals and toxic/hazardous materials should not be stored in proximity of the Eagle Creek State Nature Preserve.  All project related refueling and maintenance activities shall be performed in an environmentally responsible manner.

19. Waste materials generated during construction shall be immediately removed.  Placement of removed materials within 1,000 feet of the Eagle Creek State Nature Preserve shall be prohibited.

20. All aquatic resources indicated on the plans shall be demarcated in the field as per ODOT Supplemental Specification 832 and the applicable waterway permits special provisions prior to site disturbance. Materials utilized in, or adjacent to, aquatic resources for temporary or permanent fill or bank protection shall consist of suitable material free from toxic contaminants in other than trace quantities.  Broken asphalt is specifically excluded.  Chromated Copper Arsenate (CCA), and other pressure treated lumber shall not be used in structures that are placed in aquatic resources. Furthermore, an oil spill kit will be located within 150 feet of any equipment working in a stream or wetland.  The oil spill kit shall be maintained for the life of the construction contract.

21. Soil erosion and sediment control measures will be in place prior to construction operations and shall remain in place until construction is completed and the area is accepted by the engineer. All sediment and erosion controls shall be removed upon stabilization of the project area with vegetation and acceptance of the project engineer. The contractor shall utilize and approved disposal site for the disposal of all waste soils and other materials.  The approved disposal site shall not include any wetlands, streambanks or 100-year flood plains.

22. Reconstructed and disturbed earthen areas shall be seeded/revegetated and mulched during construction to encourage establishment of vegetation cover, decrease erosion and prevent erosion of sediments into waters of the United States.

23. The contractor shall contact the Northeast Regional Preserve Manager, Adam Wohlever, at least one week prior to project commencement. Mr. Wohlever shall also be immediately notified upon any potential negative impacts and immediately following the completion of work in the areas of concern for inspection.

24. Waterway permits shall be submitted to the appropriate agencies and accepted prior to any tree-clearing for the project.

Preparers  & Approvals

Form Preparer(s)

| Form Preparer | Brian Peck ODOT DISTRICT 4 330-786-4931 Brian.Peck@dot.ohio.gov |
| --- | --- |

| Supporting Form Preparer(s) | Brian Peck, Chad Root, Jon Hunt, Anthony Zumbo, Dan Prevost |
| --- | --- |

Approvals & Electronic Signatures

| Approved & Electronically Signed By: | Approval Date: |
| --- | --- |

| Edward Deley PROGRAM ADMIN 3 | February 8, 2017 |
| --- | --- |
| Edward Deley PROGRAM ADMIN 3 | February 8, 2017 |

Appendix

Uploaded Files by Report Section

Environmental Commitments

Environmental Consultation Form.docx

Section 4(f)

Section 4(f) Coordination Flow Chart.pdf

Site Specific Property Map.jpg

General

Stage 2 Design.pdf

POR-293-0092 Bridge Plan.pdf

ESA

Screening.pdf

OES Recommendations - Screening.pdf

Cultural Resources

Minimum Potential to Cause Effects - B.pdf

Section 106 Request for Review.pdf

Ecological

Coordination with ODNR.pdf

Coordination with USFWS.pdf

ODNR Comments.pdf

Coordination with Project Sponsor.pdf

Project Related Correspondence.pdf

USFWS Comments.pdf

Coordination Field Meeting with ODNR .pdf

Wetlands Finding.pdf

Response to USFWS and ODNR Comments.pdf

Level 1 Ecological Survey.pdf

Field Investigation Notes.pdf

PCEO Culvert Maintenance Information.pdf

Environmental Justice

Census Mapping.pdf

Environmental Justice Form.pdf

Public Involvement

Press Release.pdf

EMS and Public Services Coordination.pdf

EMS and Public Services Contact List.pdf

Permits

FEMA FIRM.pdf

Pre-Construction Notification.pdf

Coordination with Local Floodplain Administrator.pdf

Local Floodplain Administrator Approval1.pdf

Local Floodplain Administrator Approval2.pdf