Categorical Exclusion Document

for

LAK IR 090 23.42 L&R Repair PID 101048

CE Level: D1

Approved: January 15, 2016

Prepared by: District 12

Contents

Project Type	5

General Project Information	6

Project, Cost Schedule and Work Limits	6

No roadway data for PIDs associated with this CE Form	8

Design Criteria For Bridges	8

Maintenance of Traffic During Construction	10

Environmental Commitments, Maintenance of Traffic During Construction	11

Right of Way and Utility Involvement	11

Purpose & Need	12

Purpose & Need	12

Alternatives	14

Alternatives	14

Air & Noise	15

Noise	15

Mobile Source Air Toxics (MSATs)	15

Particulate Matter 2.5 (PM2.5)	16

Carbon (CO)	16

Ozone	16

Environmental Commitments	17

ESA	18

ESA	18

Cultural Resources	19

Cultural Properties Present	19

Archaeological Resource Adverse Effect	20

History/Architecture Adverse Effect	20

Environmental Commitments	20

Ecological	21

ESR	21

Wetlands 	21

Streams & Rivers 	22

Other Surface Waters 	23

Terrestrial Habitat 	23

Threatened or Endangered Species 	24

Agency Coordination 	25

Environmental Commitments, Agency Coordination 	26

Other Resources	28

Farmlands	28

Drinking Water	29

Section 4(f) & 6(f)	30

Section 4(f) Determination	30

Identified Section 4(f) Properties	30

Section 6(f) Determination	31

Community Impacts	32

Community Impacts	32

Environmental Justice	34

Environmental Justice	34

Public Involvement	35

Public Involvement	35

Permits	36

Waterway Permits	36

Storm Water Permits	36

Floodplains	37

Landfills	37

Environmental Commitments	38

General, Maintenance of Traffic During Construction	38

Ecological, Agency Coordination 	38

Preparers  & Approvals	40

Form Preparer(s)	40

Approvals & Electronic Signatures	40

Appendix	41

Uploaded Files by Report Section	41

General	41

ESA	41

Cultural Resources	41

Ecological	41

Permits	41

Project Type

| Please check all of the following actions that apply (Must check at least one): | (13) Actions described in 23 CFR 771.117 (c)(26), (c)(27), and (c)(28) that do not meet the constraints listed in 23 CFR 771.117(e). (a) Project types that exceed thresholds in Appendix A (b) Project types that exceed thresholds in Appendix B |
| --- | --- |

General Project Information

Project, Cost Schedule and Work Limits

| CE Level | D1 |
| --- | --- |

| PID | 101048 |
| --- | --- |

| Project Name | LAK IR 090 23.42 L&R Repair |
| --- | --- |

| Project Sponsor | ODOT SPONSORING AGENCY |
| --- | --- |

| ODOT District | 12 |
| --- | --- |

| Funding Source | State, Other |
| --- | --- |

| Private Funding: | No |
| --- | --- |

| Local Public Funding: | No |
| --- | --- |

| The proposed project is listed on the STIP | Yes |
| --- | --- |

| The next phase of the proposed project is listed on the STIP | Yes |
| --- | --- |

| The current cost estimate is in line with existing federal procedures for Ohio STIP Amendments and Administration Modifications | Yes |
| --- | --- |

| Planning and Engineering | $61,991.00 |
| --- | --- |

| Right of Way | $0.00 |
| --- | --- |

| Construction | $871,000.00 |
| --- | --- |

| Other | $0.00 |
| --- | --- |

| An Interchange Modification / Interchange Justification Study (IMS/IJS) was completed | No |
| --- | --- |

Project Description:

Replace the forward expansion joints on the twin I-90 bridges over the Grand River. This work was removed from PID 100078 where the rear joints will be replaced.

The project also involves removing steel debris from the Grand River that was originally part of PID 95309.  The work was removed from PID 95309 because of scheduling conflicts cause by ecological coordination.

USGS Map and Aerial Map are included in Project File/General/Project Information/USGS Map.JPG and Aerial Map.pdf.

| Limits of Proposed Work | Over the Grand River |
| --- | --- |

| Start (SLM): | 23.40 |
| --- | --- |

| End (SLM): | 23.51 |
| --- | --- |

| Total Work Length (Miles): | 0.11 |
| --- | --- |

No roadway data for PIDs associated with this CE Form

Design Criteria For Bridges

Tab Number: 4304969

| Sufficiency Rating: | 085.0 |
| --- | --- |

| General Rating: | 8 |
| --- | --- |

| Date Built: | 09/01/2008 |
| --- | --- |

| Bridge Location: | 3.45 MI W OF SR 528 |
| --- | --- |

|  | Existing | Proposed |
| --- | --- | --- |
| Bridge Type: | 322 | 322 |
| Bridge Length (ft): | 909 | 909 |
| Number of Main Spans: | 5 | 5 |
| Max Span Length (ft): | 220 | 220 |
| Load Restrictions (TON): | 150 | 150 |
| Curb to Curb Width (ft): | 52 | 52 |
| Shoulder Width(ft): | 10 | 10 |
| Under Clearance (ft): | N/A | N/A |

| Bridge Type Description: | The project will repair the bridge expansion joints. |
| --- | --- |

| Load Restrictions Description: | The bridge does not have load restrictions. |
| --- | --- |

| Will the structure be rehabilitated or replaced as part of the project? | No |
| --- | --- |

| If this bridge is a historic bridge, what type is it? | N/A |
| --- | --- |

| Remarks: | The bridge was constructed in 2010 and is not eligible for the National Register of Historic Places. |
| --- | --- |

Tab Number: 4304993

| Sufficiency Rating: | 096.2 |
| --- | --- |

| General Rating: | 8 |
| --- | --- |

| Date Built: | 08/01/2008 |
| --- | --- |

| Bridge Location: | 3.45 MI W OF SR-528 |
| --- | --- |

|  | Existing | Proposed |
| --- | --- | --- |
| Bridge Type: | 322 | 322 |
| Bridge Length (ft): | 909 | 909 |
| Number of Main Spans: | 5 | 5 |
| Max Span Length (ft): | 220 | 220 |
| Load Restrictions (TON): | 150 | 150 |
| Curb to Curb Width (ft): | 52 | 52 |
| Shoulder Width(ft): | 10 | 10 |
| Under Clearance (ft): | N/A | N/A |

| Bridge Type Description: | The project will replace the forward expansion joints on the twin I-90 bridges over the Grand River. This work was removed from PID 100078 where the rear joints will be replaced. Work includes removing steel debris from the Grand River. |
| --- | --- |

| Load Restrictions Description: | The project will not effect the load restrictions for the bridges. |
| --- | --- |

| Will the structure be rehabilitated or replaced as part of the project? | Yes |
| --- | --- |

| If this bridge is a historic bridge, what type is it? | N/A |
| --- | --- |

| Remarks: | The bridge does not meet the critera for the National Register of Historic Places. |
| --- | --- |

Maintenance of Traffic During Construction

| A roadway, bridge or ramp closure is required | Yes |
| --- | --- |

| A temporary bridge or roadway is proposed | No |
| --- | --- |

| A detour is required for the proposed project | No |
| --- | --- |

| The proposed MOT substantially impacts sensitive environmental resources | No |
| --- | --- |

| Substantial controversy is associated with the proposed MOT | No |
| --- | --- |

| Coordination has been initiated and/or completed with local emergency services, schools, public institutions/facilities, etc. | No |
| --- | --- |

Remarks:

A Plan Note Will be added for the contractor to notify Emergency Agencies and  Schools Districts two weeks prior to lane closures.

| Are there any Environmental Commitments? | Yes |
| --- | --- |

Environmental Commitments, Maintenance of Traffic During Construction

1. A Plan Note Will be added for the contractor to notify Emergency Agencies and  Schools Districts two weeks prior to lane closures.

Right of Way and Utility Involvement

| The project requires Permanent Right-of-Way | No |
| --- | --- |

| The project requires Permanent easement(s) | No |
| --- | --- |

| The project requires Temporary Right-of-Way | No |
| --- | --- |

| Number of parcels impacted by Permanent Right-of-Way: | 0 |
| --- | --- |

| Number of parcels impacted by Temporary Right-of-Way: | 0 |
| --- | --- |

| Approximate acreage of Permanent Right-of-Way needed: | 0 |
| --- | --- |

| Approximate acreage of Temporary Right-of-Way needed: | 0 |
| --- | --- |

| Electrical lines, gas lines, water lines, sewer lines, phone lines or other utilities exist in the project area | No |
| --- | --- |

Remarks:

The project does not require additional right-of-way or utility relocations.

Purpose & Need

Purpose & Need

Project History:

The twin LAK-90-23.40 Bridges were reconstructed in 2010.

Purpose Statement:

The purpose of the project is to address the deficeint expansion joint and the steel in the Grand River.

Need Element(s):

During demolition of the old bridges, the contractor left a few pieces of steel in the State Wild and Scenic Grand River.  ODNR Scenic River Coordinator requested action because it is a hazard to canoe traffic.   The Expansion joints are not operating properly and are not stable.

Goals and Objectives:

ODNR Scenic River Coordinator requested action regarding the steel beam in the river because it is a hazard to canoe traffic.

Summary Statement:

The purpose of the project is to address the deficient expansion joint and the steel in the Grand River.

Logical Termini and Independent Utility:

The deficient Expansion Joints are located on the east and west approach pavement .  The Steel is located under the eastbound bridge in the Grand River.

Alternatives

Alternatives

Discuss No Build Alternative:

The No Build Alternative does not address the deficient expansion joint or the steel in the Grand River.

| Was a Feasibility Study completed? | No |
| --- | --- |

| Was an Alternative Evaluation Report (AER) completed? | No |
| --- | --- |

| Name | Description | Reason Dismissed | Preferred Alternative |
| --- | --- | --- | --- |
| Build Alternative | This Alterntive replaces the expansion joints and removes the steel from the river. | The alternative was not dimissed | Yes |
| No Build Alternative | Leaves the deficient joints in place and the steel in the Grand River. | Did not address the Purpose and Need Statement | No |

Discuss Reasons Alternative Identified as Preferred was selected:

The Build Alternative was chosen to address the ODNR concern for canoe traffic and the alternative replaces the deficient expansion joint.

Air & Noise

Noise

| Is noise analysis required for the project? | No |
| --- | --- |

| Are there noise sensitive land uses within 500' of the project area? | No |
| --- | --- |

Remarks:

The proposed project will not cause an increase in traffic volumes, will not substantially change the vehicle mix or speed, will not involve new roadways or substantially change the alignments or shielding effects of the existing roadway. Therefore, this project is deemed unrelated to increased traffic noise.

Mobile Source Air Toxics (MSATs)

| The proposed project is exempt in accordance with 40 CFR 93.126 | Yes |
| --- | --- |

Remarks:

The project is in the following category listed under 40 CFR 93.126 and is exempt from MSAT's:  Pavement resurfacing and/or rehabilitation.

Particulate Matter 2.5 (PM2.5)

| Is the project in an air quality non-attainment or maintenance area for PM2.5? | Yes |
| --- | --- |

| Does this project fall in a category under 40CFR93.126, which means this project exempt from conformity analysis? | Yes |
| --- | --- |

Remarks:

The project is in the following category listed under 40 CFR 93.126 and is exempt from MSAT's: Pavement resurfacing and/or rehabilitation.

Carbon (CO)

| The proposed project is exempt in accordance with 40 CFR 93.126 | Yes |
| --- | --- |

Remarks:

The project is in the following category listed under 40 CFR 93.126 and is exempt from MSAT's: Pavement resurfacing and/or rehabilitation.

Ozone

| Is the project in an Ozone maintenance area? | Yes |
| --- | --- |

| Is the project on the STIP/TIP? | Yes |
| --- | --- |

Remarks:

The project is listed on Statewide Line Item #32 and is exempt from ozone analysis.

Environmental Commitments

| Are there any environmental commitments? | No |
| --- | --- |

ESA

ESA

| Does the project require any Permanent ROW or easement? | No |
| --- | --- |

| Does the project require any temporary ROW? | No |
| --- | --- |

| ESA Screening Report was completed by District Staff | No |
| --- | --- |

Remarks:

During the October 15, 2015 District Field Review ofLAK-90-23.42 L&R Projectno sources of regulated materials were identified. The proposed project will repair expansion joints and remove debris from the Grand River. The area is undeveloped and highway R/W and it is unlikely the project will encounter regulated materials. District ESA Determination IOC included in Project File/ESA/Project Information/District ESA Determination.pdf.

| According to the IOC from OES does the project require any Environmental Commitments (plan notes and/or other coordination)? | No |
| --- | --- |

Cultural Resources

Cultural Properties Present

Please describe all of the efforts made to identify Historic Properties (including lit review, field investigation, etc.):

On October 15, 2015 the District performed a Field review of the project area.

| Based on the letter or IOC you received from OES, is there an eligible or listed NRHP Historic Property in the Area of Potential Effects Pursuant to 36 CFR part 800? | No |
| --- | --- |

| OES/OSHPO Concurrence Date: | 01/11/2016 |
| --- | --- |

Remarks:

The proposed project meets  Section 106 Programmatic Agreement (PA) between the Federal Highway Administration, the Advisory Council on Historic Preservation, Ohio Historic Preservation Office and the Ohio Department of Transportation, signed on November 30, 2011, Appendix A Item #8.

(8) Bridge deck overlays, bridge deck replacements, superstructure replacement and other maintenance activities, including bridge painting projects.

These projects occur within existing right-of-way or easement.

District Section 106 Determination IOC is included in Project File/Cultural Resources/Coordination/Minimum Potential to Cause Effects - A.pdf.

| What is the Section 106 effect determination in the OES letter or IOC? | Minimal Potential to Cause Effect - A |
| --- | --- |

Archaeological Resource Adverse Effect

History/Architecture Adverse Effect

Environmental Commitments

| Are there any Environmental Commitments? | No |
| --- | --- |

Ecological

ESR

| Has an ESR been completed? | Yes |
| --- | --- |

| Date ESR was completed: | 06/03/2015 |
| --- | --- |

Wetlands

| Are Wetlands Present? | Yes |
| --- | --- |

| Are Any Wetlands impacted? | No |
| --- | --- |

| Has a Wetland Delineation been made? | Yes |
| --- | --- |

| Has the wetland Jurisdictional Determination been made? | No |
| --- | --- |

Remarks:

Two wetlands are located to the east and west of the project area along the bank of the Grand River. However, the method of raising the beam from the bridge will avoid placing equipment or fill within the wetlands. The crane will be placed on the bridge above the river.  A Level 1 Ecological Survey Report was completed for the project and is located on the On-Line CE at Project File/Ecological/Report/Level 1 Ecological Survey Report.pdf

Streams & Rivers

|  | Present | Impacted |
| --- | --- | --- |
| Streams and Rivers: | Yes | Yes |
| National Scenic River: | No |  |
| State Scenic River: | Yes | Yes |
| Sec 9: | No |  |
| Sec 10: | No |  |

| Stream Name | Total Linear feet Impacted | QHEI | Aquatic Life Use Designation |
| --- | --- | --- | --- |
| Wild and Scenic Grand River | 30 | N/A | Exceptional Warm Water Habitat (EWH) |

| Total Linear feet Impacted | 30 |
| --- | --- |

Remarks:

The stream impacts of 30-feet will occur from the removal of the beams by the crane. The Crane will be on the IR-90 Bridge over the river, therefore the equipment will not be placed in the river. However, the removal of the beam will disturb the sediment from the river and will impact the mussels in the immediate vicinity of the beam and directly downstream.  the joint repair will take place on the abutments of the bridge and will not impact the river.A Level 1 Ecological Survey Report was completed for the project and is located onthe On-Line CE at Project File/Ecological/Report/Level 1 Ecological Survey Report.pdf

ODNR Scenic Rivers approved the beam removal on 8/21/15 and this approval letter is located in the Project Files/Ecological/Coordination/ODNR SR Approval.pdf.  The joint repair was approved per the Programmatic Agreement # 11323 between ODNR and ODOT as a Tier III Minor Maintenance Project.  See the signed approval in the  Project Files/Ecological/Coordination/ODNR SR Tier 3 Approval.pdf.

Other Surface Waters

|  | Present |  |
| --- | --- | --- |
| Reservoirs: | No |  |
| Lakes: | No |  |
| Ponds: | No |  |
| Storm Water Management Facility: | No |  |
| Jurisdictional Ditch: | No |  |
| Other (If selected please explain in remarks): | No |  |

Remarks:

The ESR, dated June 3, 2015, did not identify any reservoirs, lakes, ponds, stormwater management facility or jurisdictional ditch.  A Level 1 Ecological Survey Report was completed for the project and is located on the On-Line CE at Project File/Ecological/Report/Level 1 Ecological Survey Report.pdf

Terrestrial Habitat

|  | Present | Impacted |
| --- | --- | --- |
| Terrestrial Habitat: | Yes | No |
| Unique or high Quality: | No |  |

Remarks:

The project will not impact terrestrial habitat and within the highway. The Level 1 ESR did not identify unique or high quality habitat.  A Level 1 Ecological Survey Report was completed for the project and is located on the On-Line CE at Project File/Ecological/Report/Level 1 Ecological Survey Report.pdf

Threatened or Endangered Species

|  | Present | Impacted |
| --- | --- | --- |
| Within the known range of a Federal Species? | Yes | Yes |
| Federal Species/habitat found within the project area? | Yes | Yes |
| Within the known range of a State Species? | Yes | Yes |
| State Species/habitat found within the project area? | Yes | Yes |

Remarks:

The project is located within the known range of the Indiana Bat (Myotis sodalis), Northern Long-eared Bat (Myotis septentrionalis), Bald Eagle (Haliaeetus leucocephalus), Piping Plover(Charadrius melodus), Snuffbox mussel (PE) (Epioblasma triquetra) and the Kirtland's warbler (Dendroica kirtlandii).  
The project will impact the Snuffbox mussel but will not impact the other species.  A Level 1 Ecological Survey Report and a Biological Assessment was completed  for the project and is located on the On-Line CE at Project File/Ecological/Report/Level 1 Ecological Survey Report.pdf and Biological Assessment.pdf.  USFWS issued a Biological Opinion on 8/10/15.  The ecological comments are listed in the Biological Opinion On-Line CE at Project File/Ecological/Report/ Biological Opinion.pdf and in the Ecological IOC located On-Line CE at Project File/Ecological/Project Information/ODOT response of ODNR and USFWS Comments.  The Environmental commitments are listed in Section 69 below.  ODNR accepted the Mussel Salvage Report on 1/14/16.  See the Project File/Ecological/Coordination/ODNR accepted the Mussel Salvage Report.pdf. 
The project has no tree removal required, therefore, impacts to Indiana bat , Northern long-eared bat, and Kirtland's  warbler habitat is not anticipated.   The closest known bald eagle's nest is 5.7 miles away from the project and no impacts are anticipated.The project is located 5 miles away fro the Lake Erie coast, therefore, no impacts to Piping plover are anticipated.

Agency Coordination

|  | Coordination Required | Approval/Comments Received |
| --- | --- | --- |
| Ohio Department of Natural Resources (ODNR): | Yes | Yes |
| United States Fish and Wildlife Service (USFWS): | Yes | Yes |
| Ohio Environmental Protection Agency (OEPA): | No |  |
| United States Army Corps of Engineers (USACE): | No |  |
| ODNR State Scenic River: | Yes | Yes |
| National Park Service (NPS) National Scenic River: | No |  |

Remarks:

See the Ecological IOC located On-Line CE at Project File/Ecological/Project Information/ODOT response of ODNR and USFWS Comments. The Environmental commitments are listed in Section 69 below.

| Are there any environmental commitments? | Yes |
| --- | --- |

Environmental Commitments, Agency Coordination

1. The project shall comply with all the lawful requirements of municipalities, counties, or other local agencies regarding the discharge of stormwater from construction activities. Reduction in stormwater runoff will limit the potential of sedimentation which could impact mussels.

2. All vehicles shall remain on or adjacent to the IR-90 Bridge and refueling shall occur on the roadway.

3. Regular daily inspection of all construction vehicles and equipment used shall be done to identify and control possible leakage of toxic materials including fuels, lubricants, hydraulic fluid, etc. If leakage is found, the fluids will be contained and removed immediately in accordance with applicable regulations and the equipment repaired prior to further use.

4. Any unpermitted discharges to waterways shall be reported immediately upon discovery.

5. Notify the contractor regarding the presence of T &E species and proper implementation of avoidance and minimization measures.

6. If this species is encountered within the construction limits during construction operations, we recommend that all construction operations temporarily cease and the USFWS Columbus Field Office be notified immediately (614-416-8993).

7. Since construction equipment will be used on a bridge over the Grand River, it is imperative that regular daily inspection of all construction vehicles and equipment used shall be done to identify and control possible leakage of toxic materials including fuels, lubricants, hydraulic fluid, etc. If leakage is found, the fluids will be contained and removed immediately in accordance with applicable regulations and the equipment repaired prior to further use;

8. Any unpermitted discharges to waterways shall be reported immediately upon discovery

9. The beams shall be disposed of in an appropriate fashion. There should not be any fills or other construction waste.

10. Minimize the duration and extent of in-stream construction activities. Keep the rigging team as small as possible and make the rigging process as fast and safe as possible without undue disturbance of the river bottom

11. Lower rigging chains or cables to the work area in the river at the end of the crane cable rather than dragging them to the beams over the river bottom

12. Carefully position rigging on beams to avoid slippage and to assure even lifting of beams

Other Resources

Farmlands

| The proposed project area is within an Urbanized Area on the Census Bureau Map | No |
| --- | --- |

| The proposed project area is within an Urban Area mapped with a tint overprint on the USGS Topographical Map | No |
| --- | --- |

| The proposed project involves new right-of-way | No |
| --- | --- |

| Based on the scope of work, the proposed project is in compliance with the executed Farmlands MOU, the FPPA, and 7 CFR 658. No further coordination is required. |  |
| --- | --- |

Remarks:

The project does not require additional right-of-way.

| Are there any environmental commitments? | No |
| --- | --- |

Drinking Water

| The proposed project is wholly or partially located within the USEPA designated boundaries of a Sole Source Aquifer? | No |
| --- | --- |

|  | Present |  |
| --- | --- | --- |
| The proposed project is wholly or partially located within the OEPA designated boundaries of a Source Water Protection Area(s)? | No |  |

| Coordination with the Local Public Water Administrator is required | No |
| --- | --- |

|  | Present |  |
| --- | --- | --- |
| Residential Wells are present | No |  |

Remarks:

The project will use BMP to minimize impacts to the Grand River.

| Are there any environmental commitments? | No |
| --- | --- |

Section 4(f) & 6(f)

Section 4(f) Determination

| Are there 4(f) properties within and/or adjacent to the project area? | Yes |
| --- | --- |

| Was concurrence required with the OWJ? | No |
| --- | --- |

| Why concurrence was not required? | No R/W or change in Access |
| --- | --- |

|  | Present | Impacted |
| --- | --- | --- |
| Publicly owned Park(s): | Yes | No |
| Publicly owned recreation facility(ies) and/or area(s): | Yes | No |
| Wildlife and Waterfowl refuge(s): | No |  |
| Historic Site(s): | No |  |

Identified Section 4(f) Properties

| Property Name | Type | Permanent ROW/Easement Acres | Temporary ROW/Easement Acres | 4(f) finding | Date |
| --- | --- | --- | --- | --- | --- |
| Lake Metroparks River Road Park | Publicly owned Park | 0 | 0 | Present but not impacted | 01/13/2016 |

Remarks:

Lake Metroparks River Road Park is located adjacent to the IR-90 bridge over the Grand River.  However, the project will not require right-of-way from the park and will not restrict access to the park.  This was determined by ODOT district 12 Staff during the October 15, 2015 Field review.

| Are there any environmental commitments? | No |
| --- | --- |

Section 6(f) Determination

|  | Present |  |
| --- | --- | --- |
| 6(f) Properties: | No |  |

Remarks:

No Section 6(f) properties are located within project limits.  This was determined by ODOT district 12 Staff during the October 15, 2015 Field review and reviewed the project list at http://waso-lwcf.ncrc.nps.gov/public/index.cfm  and no Section 6(f) Properties were identified within the project area.

| Are there any environmental commitments? | No |
| --- | --- |

Community Impacts

Community Impacts

| Will the proposed action comply with the local/regional development patterns for the area? | Yes |
| --- | --- |

Remarks:

The project is a maintenance project and will not impact local/regional development patterns.

| Will the proposed action result in substantial negative impacts to community cohesion? | No |
| --- | --- |

Remarks:

The project is a maintenance project and will not impact lcommunity cohesion.

| Will the proposed action result in indirect or cumulative impacts? | No |
| --- | --- |

Remarks:

The project is a maintenance project and will not cause indirect or cumulative impacts.

| Will the proposed action result in substantial impacts on health and educational facilities, public utilities, fire, police, emergency services, religious institutions, public transportation, pedestrian and bicycle facilities? | No |
| --- | --- |

Remarks:

The project is a maintenance project and will cause temporary traffic impacts due to lane closures.

| Will the proposed action displace residents, businesses, institutions or farms? | No |
| --- | --- |

Remarks:

The project does not require additional right-of-way.

| Are there any Environmental Commitments? | No |
| --- | --- |

Environmental Justice

Environmental Justice

| Census Block Group # | % Minority | % Low Income |
| --- | --- | --- |
| 2052 | 4 | 18 |

| Are there block groups that have minority and/or low-income populations that are equal to or greater than 40%? | No |
| --- | --- |

Remarks:

The Low Income Population was 18% and the Minority Population was at 4%.  The project is a maintence project and will not impact EJ Population.

| Are there any Environmental Commitments? | No |
| --- | --- |

Public Involvement

Public Involvement

Please provide a summary of the Public Involvement activities that have been conducted for this project. (For example press releases, letters to affected property owners and residents, meetings, special purpose meetings, newspaper articles, etc)

The contract will contact Local Emergency Services and School Districts fourteen (14) days prior to lane closures.

| Was Public Involvement conducted in compliance with Title VI requirements? | Yes |
| --- | --- |

| Is there any substantial environmental controversy on environmental grounds? | No |
| --- | --- |

Please summarize the Public Involvement responses received.

No Public Comments were received.

| Are there any Environmental Commitments? | No |
| --- | --- |

Permits

Waterway Permits

| Are Waterway Permits required? | No |
| --- | --- |

ODNR

| Shore Structure Permit : | No |
| --- | --- |

Remarks:

No fill or equipment will will be places in the Grand River or Wetlands.

| Are there any environmental commitments? | No |
| --- | --- |

Storm Water Permits

| NPDES Construction General Permit for Stormwater (NOI): | No |
| --- | --- |

| Watershed Specific NPDES Construction General Permit for Stormwater (NOI): | No |
| --- | --- |

Remarks:

Earth disturbance is less than 1 acre of impacts.  Therefore a NPDES Permit is not required.

| Are there any environmental commitments? | No |
| --- | --- |

Floodplains

| Is the project located in a regulated floodplain? | Yes |
| --- | --- |

| Has coordination with the local floodplain coordinator been completed? | Yes |
| --- | --- |

Remarks:

The project is located within the floodplain of the Grand River.  However, the project will not place fill within the floodplain.  Plans were sent to the Lake County Floodplain Coordinator on 1/14/15.  See the Project File/Permits/Coordination with Local Floodplain Administrator.pdf.

| Are there any environmental commitments? | No |
| --- | --- |

Landfills

| Is a 2713 Permit required? | No |
| --- | --- |

Remarks:

The project will not impact any landfills.  the project area is IR-90 R/W.

| Are there any environmental commitments? | No |
| --- | --- |

Environmental Commitments

General, Maintenance of Traffic During Construction

Commitments

1. A Plan Note Will be added for the contractor to notify Emergency Agencies and  Schools Districts two weeks prior to lane closures.

Ecological, Agency Coordination

Commitments

1. The project shall comply with all the lawful requirements of municipalities, counties, or other local agencies regarding the discharge of stormwater from construction activities. Reduction in stormwater runoff will limit the potential of sedimentation which could impact mussels.

2. All vehicles shall remain on or adjacent to the IR-90 Bridge and refueling shall occur on the roadway.

3. Regular daily inspection of all construction vehicles and equipment used shall be done to identify and control possible leakage of toxic materials including fuels, lubricants, hydraulic fluid, etc. If leakage is found, the fluids will be contained and removed immediately in accordance with applicable regulations and the equipment repaired prior to further use.

4. Any unpermitted discharges to waterways shall be reported immediately upon discovery.

5. Notify the contractor regarding the presence of T &E species and proper implementation of avoidance and minimization measures.

6. If this species is encountered within the construction limits during construction operations, we recommend that all construction operations temporarily cease and the USFWS Columbus Field Office be notified immediately (614-416-8993).

7. Since construction equipment will be used on a bridge over the Grand River, it is imperative that regular daily inspection of all construction vehicles and equipment used shall be done to identify and control possible leakage of toxic materials including fuels, lubricants, hydraulic fluid, etc. If leakage is found, the fluids will be contained and removed immediately in accordance with applicable regulations and the equipment repaired prior to further use;

8. Any unpermitted discharges to waterways shall be reported immediately upon discovery

9. The beams shall be disposed of in an appropriate fashion. There should not be any fills or other construction waste.

10. Minimize the duration and extent of in-stream construction activities. Keep the rigging team as small as possible and make the rigging process as fast and safe as possible without undue disturbance of the river bottom

11. Lower rigging chains or cables to the work area in the river at the end of the crane cable rather than dragging them to the beams over the river bottom

12. Carefully position rigging on beams to avoid slippage and to assure even lifting of beams

Preparers  & Approvals

Form Preparer(s)

| Form Preparer | District 12 |
| --- | --- |

| Supporting Form Preparer(s) | Thomas Sorge |
| --- | --- |

Approvals & Electronic Signatures

| Approved & Electronically Signed By: | Approval Date: |
| --- | --- |

| Mark Carpenter TRANS ENGR 3 | January 15, 2016 |
| --- | --- |

Appendix

Uploaded Files by Report Section

General

USGS Map.JPG

Aerial Map.pdf

ESA

District ESA Determination.pdf

Cultural Resources

Minimum Potential to Cause Effects - A.pdf

Ecological

Level 1 Ecological Survey.pdf

OES Letters to USFWS and ODNR.pdf

Biological Opinion.pdf

FHWA 8-04-15 Email to USFWS.pdf

ODNR Comments.pdf

Mussel Salvage Report.pdf

ODOT response of ODNR and USFWS Comments.pdf

Biological Assessment.pdf

ODNR accepted the Mussel Salvage Report.pdf

ODNR Scenic River Field Review Form.pdf

ODNR SR Approval.pdf

ODNR SR Tier 3 Approval.pdf

Permits

Coordination with Local Floodplain Administrator.pdf