Environmental Document

for

WOO SR 281 15.09 resurface PID 102824

Environmental Document Level: C2

Approved: April 17, 2017

Prepared by: District 2

The environmental review, consultation, and other actions required by applicable Federal environmental laws for this project are being, or have been, carried-out by ODOT pursuant to 23 U.S.C. 327 and a Memorandum of Understanding dated December 11, 2015, and executed by FHWA and ODOT.

Contents

C2	3

Environmental Commitments, 	8

Environmental Commitments	10

C2	10

Preparers  & Approvals	11

Form Preparer(s)	11

Approvals & Electronic Signatures	11

Appendix	12

Uploaded Files by Report Section	12

General	12

ESA	12

Cultural Resources	12

Ecological	12

Other Resources	12

Environmental Justice	12

Public Involvement	12

Permits	13

C2

| PID | 102824 |
| --- | --- |

| Project Sponsor | DISTRICT 2 PRODUCTION |
| --- | --- |

| ODOT District | 2 |
| --- | --- |

| Funding Source | Federal |
| --- | --- |

| Private Funding: | No |
| --- | --- |

Project Description:

This project proposes to resurface State Route 281 (SR-281) from Huffman Road to State Route 199 (SR-199) McCutcheonville Road due to poor pavement conditions. Addition work includes waterproofing and encasing the pier piles of the bridge at mile 15.61 that carries SR-281 over Eckert Ditch as routine maintenance extend the life of the bridge.  A waterway permit from the US Army Corps of Engineers is required for the work within Eckert Ditch. Construction is expected to begin in the summer of 2018 and last approximately 90 days. During construction, one lane of traffic will be maintained at all times with the exception of a 5 day closure for the work over Eckert Ditch. A detour for SR-281 will be posted.  Coordination with property owners, tenants, emergency services, and schools was conducted.  No comments were received.

| STIP Reference # | SLI 32 |
| --- | --- |

| Select the appropriate project type: | (26) Modernization of a highway by resurfacing, restoration, rehabilitation, reconstruction, adding shoulders, or adding auxiliary lanes (including parking, weaving, turning, and climbing lanes), if the action meets the constraints in paragraph (e) of this section. Examples include: Joint or limited use of right-of-way where the proposed use would have minimal or no adverse social (including highway safety), economic or environmental impacts; Installation of new noise walls and other new noise mitigation projects; Construction of highway safety and truck escape ramps; Construction of bicycle lanes and pedestrian walkways, sidewalks, shared-use paths, or facilities and trailhead parking that do not otherwise qualify for a C1 designation; Beautification or facility improvement projects (i.e. landscaping, curb and gutter installation and replacement, ADA ramps/curb ramps, installation of park benches, decorative lighting, etc.); Construction of alternative energy facilities (fuel tank farms, wind turbines, etc.) |
| --- | --- |

In accordance with 23 CFR 771.117(e), the proposed project cannot be processed as a C2 CE, if it involves -

- a. Acquisition of more than a minor amount of right-of-way

- b. Residential or non-residential displacements

- c. A Coast Guard, Individual Section 404 and/or a Section 10 permit

- d. A Section 106 finding of Adverse Effect

- e. A Section 4(f) Programmatic or Individual Evaluation

- f. A finding of May Affect, Likely to Adversely Affect to Threatened and Endangered Species

- g. Construction of temporary access, or the closure of existing road, bridge, or ramps, that would result in major traffic disruptions

- h. Changes in access control

- i. Floodplain encroachment other than functionally dependent uses (e.g., bridges, wetlands) or actions that facilitate open space use (e.g., recreational trails, bicycle and pedestrian paths)

- j. Construction activities in, across or adjacent to a river component designated or proposed for inclusion in the National System of Wild and Scenic Rivers

- k. No minor public or agency controversy on environmental grounds (no opposition from any organized groups or agencies and no unresolved environmental coordination)

- l. If an EJ Analysis Report is required, the project must be processed as a D-level CE or higher level document

For certification purposes, documentation is required to illustrate no significant impacts will occur to the following environmental resources and that no unusual circumstances exist that would warrant a higher level of NEPA document. Upload all supporting documentation to the project file.

| Waterways: | Present; No Coast Guard, Individual 404, and/or Section 10 Permit required |
| --- | --- |

| Waterways Permit Type: | Regional General Permit |
| --- | --- |

| Waterways Permit Approval Date | 03/06/2017 |
| --- | --- |

| Isolated Wetland Permit | No |
| --- | --- |

| Endangered Species: | Present; No finding of May Affect, Likely to Adversely Affect |
| --- | --- |

| Endangered Species - Coordination | No Effect |
| --- | --- |

| 100-Year Floodplain: | No Encroachment Within a Special Flood Hazard Area (SFHA) |
| --- | --- |

| Section 4(f): | Not present |
| --- | --- |

| Section 6(f): | Not present |
| --- | --- |

| Cultural Resources: | Not Present |
| --- | --- |

Projects that meet C2 criteria are not anticipated to have impacts to the following environmental resources. If resources are present, documentation is only required if there is a potential for impacts.

| Air Quality: | Studies Not Required |
| --- | --- |

| Air Quality - Coordination with OES: | No |
| --- | --- |

| Noise: | Studies Not Required |
| --- | --- |

| Noise Coordination - OES Approval Date: |  |
| --- | --- |

| Hazardous Materials - ESA Screening Conducted | Studies Not Required |
| --- | --- |

| Farmland: | Non-Urbanized Area; No Impacts in Accordance With the Farmland MOU, 7 CFR 658, and ORC 929.05 |
| --- | --- |

| Scenic Rivers | No National Wild and Scenic River Within 1000 Feet of the Proposed Project Area |
| --- | --- |

Projects that meet C2 criteria must be in accordance with ODOT's EJ Guidance and activities conducted for Public Involvement are commensurate to the project's type and scope of work.

| Environmental Justice: | Does Not Exceed EJ Guidance Criteria; No EJ Analysis Report Required and No EJ Issues Raised During Public Involvement |
| --- | --- |

| Public Involvement: | Minimum PI Requirements Met; No Minor Public or Agency Controversy on Environmental Grounds |
| --- | --- |

| Environmental Commitments | Yes |
| --- | --- |

Environmental Commitments,

1. The contractor shall perform all work within the existing right of way.

2. The contractor shall notify ODOT a minimum of 18 days prior to beginning work, any major disruptive, time delaying traffic control changes, or detours during construction. Upon receipt of notice from the contractor, ODOT will notify affected agencies including local media outlets, schools, police, fire, emergency services, bus transportation departments, and post offices 15 days prior the work.

3. Adequate erosion and sediment controls shall be implemented and maintained throughout construction by the contractor and ODOT, as required.

4. The intent of this project is to not remove any trees. The project is located within the known habitat ranges of the federally listed and protected Indiana bat and northern long-eared bat.  No trees shall be removed under this project.  This requirement is necessary to avoid and minimize impacts to these species as required by the Endangered Species Act.  For the purposes of this note, a tree is defined as a live, dying, or dead woody plant, with a trunk three inches or greater in diameter at a height of 4.5 feet above the ground surface, and with a minimum height of 13 feet.

5. The contractor shall take all precautions to prevent any and all material from going off the edge of the bridge deck(s) and edge of the culvert(s) during all construction operations. The contractor shall immediately remove any material that falls into the roadside ditches, streams, wetlands, or other waters through non-mechanical means. Under no circumstances shall the contractor work in or store equipment and/or materials in any wetlands, streams, or other waters. No work is permitted below the top of bank of any stream, except the WOO-281-15.61 bridge over Eckert Ditch.

6. The project requires a waterway permit for work within Eckert Ditch at WOO-281-15.61 which will be attached to the plans as Special Provisions.  All conditions of the permit shall be followed throughout construction by the contractor.

7. ODOT shall ensure that a mussel survey and, if necessary, relocation shall occur at WOO-281-15.61 bridge over Elchert Ditch in accordance with ODNR and USFWS protocols.  No in-stream work shall occur prior to the mussel survey/relocation.

Environmental Commitments

C2

Commitments

1. The contractor shall perform all work within the existing right of way.

2. The contractor shall notify ODOT a minimum of 18 days prior to beginning work, any major disruptive, time delaying traffic control changes, or detours during construction. Upon receipt of notice from the contractor, ODOT will notify affected agencies including local media outlets, schools, police, fire, emergency services, bus transportation departments, and post offices 15 days prior the work.

3. Adequate erosion and sediment controls shall be implemented and maintained throughout construction by the contractor and ODOT, as required.

4. The intent of this project is to not remove any trees. The project is located within the known habitat ranges of the federally listed and protected Indiana bat and northern long-eared bat.  No trees shall be removed under this project.  This requirement is necessary to avoid and minimize impacts to these species as required by the Endangered Species Act.  For the purposes of this note, a tree is defined as a live, dying, or dead woody plant, with a trunk three inches or greater in diameter at a height of 4.5 feet above the ground surface, and with a minimum height of 13 feet.

5. The contractor shall take all precautions to prevent any and all material from going off the edge of the bridge deck(s) and edge of the culvert(s) during all construction operations. The contractor shall immediately remove any material that falls into the roadside ditches, streams, wetlands, or other waters through non-mechanical means. Under no circumstances shall the contractor work in or store equipment and/or materials in any wetlands, streams, or other waters. No work is permitted below the top of bank of any stream, except the WOO-281-15.61 bridge over Eckert Ditch.

6. The project requires a waterway permit for work within Eckert Ditch at WOO-281-15.61 which will be attached to the plans as Special Provisions.  All conditions of the permit shall be followed throughout construction by the contractor.

7. ODOT shall ensure that a mussel survey and, if necessary, relocation shall occur at WOO-281-15.61 bridge over Elchert Ditch in accordance with ODNR and USFWS protocols.  No in-stream work shall occur prior to the mussel survey/relocation.

Preparers  & Approvals

Form Preparer(s)

| Form Preparer | District 2 |
| --- | --- |

| Supporting Form Preparer(s) | Eli Barber, Stacy Schimmoeller |
| --- | --- |

Approvals & Electronic Signatures

| Approved & Electronically Signed By: | Approval Date: |
| --- | --- |

| Stacy Schimmoeller PROGRAM ADMIN 3 | April 17, 2017 |
| --- | --- |
| Stacy Schimmoeller PROGRAM ADMIN 3 | April 17, 2017 |

Appendix

Uploaded Files by Report Section

General

County Map.pdf

Aerial Map.pdf

USGS Quadrangle Topographical Map.pdf

Stage 2 Design.pdf

ESA

Screening Map.pdf

Cultural Resources

Buckeye Assets Mapping.pdf

Project Study Area Map.pdf

Minimum Potential to Cause Effects - A.pdf

Ecological

Level 1 ESR WOO-281-15.09 PID 102824.pdf

Coordination with ODNR.pdf

ODNR Comments.pdf

Eco Map.pdf

Other Resources

Water Source Protection Area Map.pdf

Environmental Justice

Census Mapping.pdf

EJ Documentation C2 Form.pdf

Public Involvement

Property Owner Notification Letter.pdf

Notification Mailing List.pdf

EMA and School Notifications.pdf

Permits

FEMA FIRM.pdf

Permit Determination Request.pdf

OES Permit Determination.pdf

Special Provisions.pdf

Regional General Permit B.pdf

Floodplain Impacts.pdf

Statement of Findings.pdf