Environmental Document

for

LUC SR2 18.62 Dehumidify PID 101556

Environmental Document Level: C2

Approved: July 5, 2017

Prepared by: District 2

The environmental review, consultation, and other actions required by applicable Federal environmental laws for this project are being, or have been, carried-out by ODOT pursuant to 23 U.S.C. 327 and a Memorandum of Understanding dated December 11, 2015, and executed by FHWA and ODOT.

Contents

C2	3

Environmental Commitments, 	8

Environmental Commitments	11

C2	11

Preparers  & Approvals	13

Form Preparer(s)	13

Approvals & Electronic Signatures	13

Appendix	14

Uploaded Files by Report Section	14

Section 4(f)	14

General	14

ESA	14

Cultural Resources	14

Ecological	14

Other Resources	15

Environmental Justice	15

Public Involvement	15

Permits	15

C2

| PID | 101556 |
| --- | --- |

| Project Sponsor | DISTRICT 2 PRODUCTION |
| --- | --- |

| ODOT District | 2 |
| --- | --- |

| Funding Source | Federal |
| --- | --- |

| Private Funding: | No |
| --- | --- |

Project Description:

This project proposes to install roadway lighting,a dehumidification system on the cables, and hold down repairs on the AnthonyWayne Bridge in Lucas County. The Anthony Wayne Bridge is listed as eligible for the National Register of Historic Places. Coordination with the Ohio Historic Preservation Office determined that there would be no adverse effects to the bridge. A building, located under the structure within Middlegrounds Metropark, will also beconstructed to house the pump equipment. Easements will be purchased within the Middlegrounds Metropark for the construction of the pump equipment building. The Toledo Area Metroparks will also be able to utilize a portion of this building. Coordination with the Toledo Area Metroparks has been completed. Since there will be no work below the deck of the bridge, no US Army Corps of Engineers (USACE) or US Coast Guard (USCG) permits are required. The USACE and the USCG will both be notified prior to construction and throughout the construction process. A portion of this project is located within a special flood hazard area, however, the District 2 Hydraulic Engineer has determined that there will be no impacts to the floodplain. Construction is expected to begin in the summer of 2018 and last approximately 2 years. During construction, a minimum of one lane in each direction will be maintained and pedestrian traffic will be maintained on one side of the bridge at all times.

| STIP Reference # | 2016AM03ID0085FDPE |
| --- | --- |

| Select the appropriate project type: | (26) Modernization of a highway by resurfacing, restoration, rehabilitation, reconstruction, adding shoulders, or adding auxiliary lanes (including parking, weaving, turning, and climbing lanes), if the action meets the constraints in paragraph (e) of this section. Examples include: Joint or limited use of right-of-way where the proposed use would have minimal or no adverse social (including highway safety), economic or environmental impacts; Installation of new noise walls and other new noise mitigation projects; Construction of highway safety and truck escape ramps; Construction of bicycle lanes and pedestrian walkways, sidewalks, shared-use paths, or facilities and trailhead parking that do not otherwise qualify for a C1 designation; Beautification or facility improvement projects (i.e. landscaping, curb and gutter installation and replacement, ADA ramps/curb ramps, installation of park benches, decorative lighting, etc.); Construction of alternative energy facilities (fuel tank farms, wind turbines, etc.) |
| --- | --- |

In accordance with 23 CFR 771.117(e), the proposed project cannot be processed as a C2 CE, if it involves -

- a. Acquisition of more than a minor amount of right-of-way

- b. Residential or non-residential displacements

- c. A Coast Guard, Individual Section 404 and/or a Section 10 permit

- d. A Section 106 finding of Adverse Effect

- e. A Section 4(f) Programmatic or Individual Evaluation

- f. A finding of May Affect, Likely to Adversely Affect to Threatened and Endangered Species

- g. Construction of temporary access, or the closure of existing road, bridge, or ramps, that would result in major traffic disruptions

- h. Changes in access control

- i. Floodplain encroachment other than functionally dependent uses (e.g., bridges, wetlands) or actions that facilitate open space use (e.g., recreational trails, bicycle and pedestrian paths)

- j. Construction activities in, across or adjacent to a river component designated or proposed for inclusion in the National System of Wild and Scenic Rivers

- k. No minor public or agency controversy on environmental grounds (no opposition from any organized groups or agencies and no unresolved environmental coordination)

- l. If an EJ Analysis Report is required, the project must be processed as a D-level CE or higher level document

For certification purposes, documentation is required to illustrate no significant impacts will occur to the following environmental resources and that no unusual circumstances exist that would warrant a higher level of NEPA document. Upload all supporting documentation to the project file.

| Waterways: | Not Present |
| --- | --- |

| Endangered Species: | No Impacts to Protected Species and Critical Habitat |
| --- | --- |

| 100-Year Floodplain: | Encroachment Within the SFHA is a Functionally Dependent Use |
| --- | --- |

| EO 11988/NFIP Coordination and Documentation Completed: | Yes |
| --- | --- |

| NFIP Local Floodplain Coordinator Notification Date: | 05/24/2017 |
| --- | --- |

| Section 4(f): | Present; No Programmatic Evaluation or Individual Evaluation Required |
| --- | --- |

| Section 4(f) Determination: | de minimis |
| --- | --- |

| Section 4(f) Determination Date - de minimis | 06/23/2017 |
| --- | --- |

| Section 6(f): | Not present |
| --- | --- |

| Cultural Resources: | Present; No Finding of Adverse Effect |
| --- | --- |

| Cultural Resources Coordination: | No Adverse Effect |
| --- | --- |

| Cultural Resources Coordination - SHPO Concurrence Date: | 12/08/2016 |
| --- | --- |

Projects that meet C2 criteria are not anticipated to have impacts to the following environmental resources. If resources are present, documentation is only required if there is a potential for impacts.

| Air Quality: | Studies Not Required |
| --- | --- |

| Air Quality - Coordination with OES: | No |
| --- | --- |

| Noise: | Studies Not Required |
| --- | --- |

| Noise Coordination - OES Approval Date: | 05/24/2017 |
| --- | --- |

| Hazardous Materials - ESA Screening Conducted | Yes |
| --- | --- |

| Hazardous Materials - OES Approval Date: | 03/09/2017 |
| --- | --- |

| Phase I ESA Warranted Based on Coordination with OES: | No Further Studies Warranted |
| --- | --- |

| Farmland: | Urbanized Area; No Impacts in Accordance With the Farmland MOU and 7 CFR 658 |
| --- | --- |

| Scenic Rivers | No National Wild and Scenic River Within 1000 Feet of the Proposed Project Area |
| --- | --- |

Projects that meet C2 criteria must be in accordance with ODOT's EJ Guidance and activities conducted for Public Involvement are commensurate to the project's type and scope of work.

| Environmental Justice: | Does Not Exceed EJ Guidance Criteria; No EJ Analysis Report Required and No EJ Issues Raised During Public Involvement |
| --- | --- |

| Public Involvement: | Minimum PI Requirements Met; No Minor Public or Agency Controversy on Environmental Grounds |
| --- | --- |

| Environmental Commitments | Yes |
| --- | --- |

Environmental Commitments,

1. Access to Middlegrounds Metropark will be maintained at all times during construction activities.

2. Temporary construction fencing will be installed within Middlegrounds Metropark along proposed construction limits prior to the start of construction activities to protect the existing 4(f) property and the public.

3. Appropriate signage will be installed to alert Middlegrounds Metropark users of short term closures and to direct users to additional access points.

4. Except as necessary to facilitate construction activities, the staging and/or storage of construction equipment will not take place outside proposed construction limits, within Middlegrounds Metropark boundaries.

5. The contractor will be required to closely coordinate the construction schedule with ODOT and the Toledo Metroparks

6. Work on the structure shall not be performed between the bridge deck and the water within the navigation channel.  Nothing shall change the vertical clearance height between the lowest portion of the bridge and the surface of the water within the navigational channel.

7. The contractor shall contact the US Army Corps of Engineers prior to the start of construction to notify them of the actual start date. The contractor shall also notify the US Army Corps of Engineers of all milestone events/operations throughout construction.  Contact info: Vito Melilli, Chief, US Army Corps of Engineers- Buffalo District Ohio Area Office PH: (216) 685-1205 vito.c.melilli@usace.army.mil

8. If any incidental debris falls into the Federal Channel of the Maumee River, the contractor shall promptly remove the debris and notify ODOT.  ODOT shall notify the US Army Corps of Engineers of any such debris. Contact info: Vito Melilli, Chief, US Army Corps of Engineers- Buffalo District Ohio Area Office PH: (216) 685-1205 vito.c.melilli@usace.army.mil

9. The contractor shall ensure that floodlights are orientated so that they do not interfere or provide glare to any vessels navigating the federal navigation channel at night.

10. All navigational lighting shall remain on and functional throughout construction, with the exception of when the bulbs are replaced.

11. A solid waste management plan note shall be included in the plans for Middlegrounds Metropark requiring any excavated soils to be disposed of as solid waste in a licensed solid waste landfill if they cannot be re-used on-site.

12. Notes shall be placed in the plans by ODOT directing the contractor to comply with the notification requirement timeframes outlined in the Traffic Engineering Manual Part 642-8 and 642-58 to ensure compliance with Federal notification requirements.

13. Vehicular and pedestrian traffic shall be maintained at all times.

Environmental Commitments

C2

Commitments

1. Access to Middlegrounds Metropark will be maintained at all times during construction activities.

2. Temporary construction fencing will be installed within Middlegrounds Metropark along proposed construction limits prior to the start of construction activities to protect the existing 4(f) property and the public.

3. Appropriate signage will be installed to alert Middlegrounds Metropark users of short term closures and to direct users to additional access points.

4. Except as necessary to facilitate construction activities, the staging and/or storage of construction equipment will not take place outside proposed construction limits, within Middlegrounds Metropark boundaries.

5. The contractor will be required to closely coordinate the construction schedule with ODOT and the Toledo Metroparks

6. Work on the structure shall not be performed between the bridge deck and the water within the navigation channel.  Nothing shall change the vertical clearance height between the lowest portion of the bridge and the surface of the water within the navigational channel.

7. The contractor shall contact the US Army Corps of Engineers prior to the start of construction to notify them of the actual start date. The contractor shall also notify the US Army Corps of Engineers of all milestone events/operations throughout construction.  Contact info: Vito Melilli, Chief, US Army Corps of Engineers- Buffalo District Ohio Area Office PH: (216) 685-1205 vito.c.melilli@usace.army.mil

8. If any incidental debris falls into the Federal Channel of the Maumee River, the contractor shall promptly remove the debris and notify ODOT.  ODOT shall notify the US Army Corps of Engineers of any such debris. Contact info: Vito Melilli, Chief, US Army Corps of Engineers- Buffalo District Ohio Area Office PH: (216) 685-1205 vito.c.melilli@usace.army.mil

9. The contractor shall ensure that floodlights are orientated so that they do not interfere or provide glare to any vessels navigating the federal navigation channel at night.

10. All navigational lighting shall remain on and functional throughout construction, with the exception of when the bulbs are replaced.

11. A solid waste management plan note shall be included in the plans for Middlegrounds Metropark requiring any excavated soils to be disposed of as solid waste in a licensed solid waste landfill if they cannot be re-used on-site.

12. Notes shall be placed in the plans by ODOT directing the contractor to comply with the notification requirement timeframes outlined in the Traffic Engineering Manual Part 642-8 and 642-58 to ensure compliance with Federal notification requirements.

13. Vehicular and pedestrian traffic shall be maintained at all times.

Preparers  & Approvals

Form Preparer(s)

| Form Preparer | District 2 |
| --- | --- |

| Supporting Form Preparer(s) | Phoenix Golnick Neal |
| --- | --- |

Approvals & Electronic Signatures

| Approved & Electronically Signed By: | Approval Date: |
| --- | --- |

| Phoenix Golnick Neal ENV SPEC 2 | July 5, 2017 |
| --- | --- |
| Phoenix Golnick Neal ENV SPEC 2 | July 5, 2017 |

Appendix

Uploaded Files by Report Section

Section 4(f)

OES Section 106 Determination - Exception.pdf

4(f) Public Involvement- Website Post.pdf

Correspondence with OWJ.pdf

4(f) Public Involvement- Press Release.pdf

Determination Request Form for Recreational Properties.pdf

OES Recreational 4(f) Determination.pdf

General

Aerial Map.pdf

County Map.pdf

USGS Quadrangle Topographical Map.pdf

Stage 2 Design.pdf

ESA

ESA Screening Map.pdf

OES Recommendations - Phase I.pdf

Cultural Resources

Section 106 Request for Review.pdf

Buckeye Assets Mapping.pdf

OHPO map.pdf

SHPO Concurrence - No Adverse Effect.pdf

Ecological

Eco Map.pdf

Other Resources

Water Source Protection Area Map.pdf

Environmental Justice

Census Mapping.pdf

EJ Documentation Form.pdf

Public Involvement

News Article.pdf

Public Comment and Response.pdf

Permits

FEMA FIRM.pdf

Floodplain Impacts.pdf

Statement of Findings.pdf

Permit Determination Request.pdf

Coordination with USCG.pdf

USCG Comments.pdf

Section 408 Application.pdf

USACE Approval - Section 408.pdf

OES Permit Determination.pdf

Special Provisions.pdf