Environmental Document

for

STA SR 21/172 4.46/4.20 PID 95369

Environmental Document Level: C2

Approved: November 22, 2017

Prepared by: District 4

The environmental review, consultation, and other actions required by applicable Federal environmental laws for this project are being, or have been, carried-out by ODOT pursuant to 23 U.S.C. 327 and a Memorandum of Understanding dated December 11, 2015, and executed by FHWA and ODOT.

Contents

C2	3

Environmental Commitments, 	10

Environmental Commitments	13

C2	13

Preparers  & Approvals	16

Form Preparer(s)	16

Approvals & Electronic Signatures	16

Appendix	17

Uploaded Files by Report Section	17

General	17

ESA	17

Cultural Resources	17

Ecological	17

Environmental Justice	17

Public Involvement	17

Permits	18

C2

| PID | 95369 |
| --- | --- |

| Project Sponsor | DISTRICT 4-PLANNING |
| --- | --- |

| ODOT District | 4 |
| --- | --- |

| Funding Source | Federal |
| --- | --- |

| Private Funding: | No |
| --- | --- |

Project Description:

The OhioDepartment of Transportation (ODOT) proposes to replace Bridge No.STA-SR21-0446/Structure File Number (SFN) 7600062 over an unnamed tributary ofthe Tuscarawas River in Bethlehem and Tuscarawas Townships, Stark County, and replace Bridge No. STA-SR172-0420/SFN 7600720 over an unnamed tributary of SippoCreek in the city of Massillon, Stark County.

Theexisting deteriorated STA-SR21-0446 bridge is a 15' long by 32.75' single span concrete slabstructure and will be replaced with a 16' x 6' x 36' concrete box culvert.  Above ground vegetation within 20' of theexisting structure will be removed and rock channel protection (RCP) will beinstalled to protect the new box culvert. Approximately 0.052 acre of temporaryright-of-way will be acquired from two (2) parcels and 0.038-acre of permanentright-of-way will be acquired from three (3) parcels to install the newstructure.

Theexisting deteriorated STA-SR172-0420 bridge is a 19.5' long by 40' wide single span concrete slabstructure and will be replaced with a 20' x 6' x 40' concrete box culvert.  Above ground vegetation within 20' of theexisting structure will be removed and RCP will be installed to protect the newbox culvert. Approximately 0.055 acre oftemporary right-of-way will be acquired from four (4) parcels and 0.132-acre ofpermanent right-of-way from four (4) parcels will be acquired to install thenew structure.

Project mapping on U. S. Geological Survey (USGS)7.5-Minute Topographic Quadrangle Maps is included in the ProjectFile/General/Project Information subsection as USGS Quadrangle TopographicMap.pdf.  Plan information andthe final right-of-way plan are available in the Project File/General/ProjectInformation subsection as Stage 3 Design.pdf and Final Right of Way Plans.pdf,respectively.

The projectwill require closure of State Route (SR) 21 for a maximum duration oftwenty-one (21) consecutive days and closure of SR 172 for a maximum durationof thirty (30) consecutive days with work occurring before and after theclosures.  SR 21 motorists willbe detoured along SR 212, SR 93 and U. S. Route 62. SR 172 motorists will be detoured along SR 93, SR 241 and SR 21.  Construction and detour information will beposted prior to construction start and access shall be maintained to alladjacent properties at all times during project construction.

Inaccordance with Stipulation 4A and Appendix B of the Section 106 ProgrammaticAgreement, approved on November 30, 2011, (Agreement No. 16734) and amended onJuly 17, 2017, ODOT Office of Environmental Services determined the project is a type of undertaking with "minimal potential to causeeffects" and is not a part of a larger undertaking.  See ProjectFile/Cultural Resources/Coordination/Minimal Potential to Cause Effects-A orB.pdf.

The projectwill result in temporary and permanent impacts to 80 linear feetof an unnamed tributary of the Tuscarawas River at the STA-SR21-0446 bridge and 115 linear feet of an unnamed tributary of Sippo Creek at theSTA-SR172-0420 bridge. The project will not impact any wetlands and/orother jurisdictional waters of the United States.  See Project File/Ecological/Reports/Level 1 EcologicalSurvey STA-21-4.46.pdf and Level 1 Ecological Survey STA-172-4.20.pdf.

The projectis within the Western Management Unit as defined by the 2016 ProgrammaticAgreement/Biological Opinion for the Federally-listed Indiana Bat (Myotissodalis) and Northern Long-Eared Bat (Myotis septentrionalis).  A total 0.12 acre of Suitable WoodedHabitat (SWH) will be removed to construct the project.  All SWH to be removed is contained within 100feet from the edge of pavement (EOP) and the project shall adhere to thecutting restriction dates (October 1-March 31) set forth in the 2016Programmatic Agreement/Biological Opinion for the Federally-listed Indiana Batand Northern Long-Eared Bat.  The projectmay affect, but is not likely to adversely affect these species.  The project will not impact any other stateor federally-listed threatened or endangered species.  See Project File/Ecological/Reports/Level 1 Ecological SurveySTA-21-4.46.pdf and Level 1 Ecological Survey STA-172-4.20.pdf.

The project will not impact any other environmentally sensitiveresources within the project study areas.

Utilitypoles, overhead utility lines and underground utilities to be impacted by theproject will be relocated within the existing and proposed permanent roadwayrights-of-way.  Existing utilities willnot be permanently removed to construct the proposed project.

Affectedproperty owners and local emergency/public services were sent project and proposed maintenance of traffic information.  Comments were received about the SR 21 bridge replacement schedule.  Consequently, aninterim completion date was established to reopen SR 21 by 8/7/2018 to accommodate the FairlessLocal Schools.

Stage 3plan was utilized to evaluate the environmental impacts by and obtain NEPAapproval for the project.

| STIP Reference # | Statewide Line Item (SLI) Number 32 and is fiscally constrained. |
| --- | --- |

| Select the appropriate project type: | (28) Bridge rehabilitation, reconstruction, or replacement or the construction of grade separation to replace existing at-grade railroad crossings, if the actions meet the constraints in 23 CFR 771.117(e). Examples include: Railroad projects that close or relocate at-grade crossings |
| --- | --- |

In accordance with 23 CFR 771.117(e), the proposed project cannot be processed as a C2 CE, if it involves -

- a. Acquisition of more than a minor amount of right-of-way

- b. Residential or non-residential displacements

- c. A Coast Guard, Individual Section 404 and/or a Section 10 permit

- d. A Section 106 finding of Adverse Effect

- e. A Section 4(f) Programmatic or Individual Evaluation

- f. A finding of May Affect, Likely to Adversely Affect to Threatened and Endangered Species

- g. Construction of temporary access, or the closure of existing road, bridge, or ramps, that would result in major traffic disruptions

- h. Changes in access control

- i. Floodplain encroachment other than functionally dependent uses (e.g., bridges, wetlands) or actions that facilitate open space use (e.g., recreational trails, bicycle and pedestrian paths)

- j. Construction activities in, across or adjacent to a river component designated or proposed for inclusion in the National System of Wild and Scenic Rivers

- k. No minor public or agency controversy on environmental grounds (no opposition from any organized groups or agencies and no unresolved environmental coordination)

- l. If an EJ Analysis Report is required, the project must be processed as a D-level CE or higher level document

For certification purposes, documentation is required to illustrate no significant impacts will occur to the following environmental resources and that no unusual circumstances exist that would warrant a higher level of NEPA document. Upload all supporting documentation to the project file.

| Waterways: | Present; No Coast Guard, Individual 404, and/or Section 10 Permit required |
| --- | --- |

| Waterways Permit Type: | Regional General Permit |
| --- | --- |

| Waterways Permit Approval Date | 11/17/2017 |
| --- | --- |

| Isolated Wetland Permit | No |
| --- | --- |

| Endangered Species: | Present; No finding of May Affect, Likely to Adversely Affect |
| --- | --- |

| Endangered Species - Coordination | May Affect, Not Likely to Adversely Affect |
| --- | --- |

| Endangered Species - Coordination Date | 08/31/2017 |
| --- | --- |

| Endangered Species - Critical Habitat Present/Impacted | Indiana bat |
| --- | --- |
|  | Northern long-eared bat |

| Endangered Species - Other Critical Habitat Present/Impacted: | No |
| --- | --- |

| 100-Year Floodplain: | No Encroachment Within a Special Flood Hazard Area (SFHA) |
| --- | --- |

| Section 4(f): | Not present |
| --- | --- |

| Section 6(f): | Not present |
| --- | --- |

| Cultural Resources: | Present; No Finding of Adverse Effect |
| --- | --- |

| Cultural Resources Coordination: | Minimum Potential to Cause Effect - Appendix B |
| --- | --- |

| Cultural Resources Coordination - ODOT Approval Date: | 07/17/2017 |
| --- | --- |

Projects that meet C2 criteria are not anticipated to have impacts to the following environmental resources. If resources are present, documentation is only required if there is a potential for impacts.

| Air Quality: | Studies Not Required |
| --- | --- |

| Air Quality - Coordination with OES: | No |
| --- | --- |

| Noise: | Studies Not Required |
| --- | --- |

| Noise Coordination - OES Approval Date: |  |
| --- | --- |

| Hazardous Materials - ESA Screening Conducted | Studies Not Required |
| --- | --- |

| Farmland: | Non-Urbanized Area; No Impacts in Accordance With the Farmland MOU, 7 CFR 658, and ORC 929.05 |
| --- | --- |

| Scenic Rivers | No National Wild and Scenic River Within 1000 Feet of the Proposed Project Area |
| --- | --- |

Projects that meet C2 criteria must be in accordance with ODOT's EJ Guidance and activities conducted for Public Involvement are commensurate to the project's type and scope of work.

| Environmental Justice: | Does Not Exceed EJ Guidance Criteria; No EJ Analysis Report Required and No EJ Issues Raised During Public Involvement |
| --- | --- |

| Public Involvement: | Minimum PI Requirements Met; No Minor Public or Agency Controversy on Environmental Grounds |
| --- | --- |

| Environmental Commitments | Yes |
| --- | --- |

Environmental Commitments,

1. The Project Designer shall incorporate the following note into the plans:

THE PROJECT IS LOCATED WITHIN THE KNOWN HABITAT RANGES OF THE FEDERALLY LISTED AND PROTECTED INDIANA BAT AND NORTHERN LONG-EARED BAT.  NO TREES SHALL BE REMOVED UNDER THIS PROJECT FROM APRIL 1 THROUGH SEPTEMBER 30.  ALL NECESSARY TREE REMOVAL SHALL OCCUR FROM OCTOBER 1 THROUGH MARCH 31. THIS REQUIREMENT IS NECESSARY TO AVOID AND MINIMIZE IMPACT TO THESE SPECIES AS REQUIRED BY THE ENDANGERED SPECIES ACT. FOR THE PURPOSES OF THIS NOTE, A TREE IS DEFINED AS A LIVE, DYING, OR DEAD WOODY PLANT, WITH A TRUNK THREE INCHES OR GREATER IN DIAMETER AT A HEIGHT OF 4.5 FEET ABOVE THE GROUND SURFACE, AND WITH A MINIMUM HEIGHT OF 13 FEET.

2. The Project Designer shall incorporate the following note into the plans:

PRIOR TO ANY STRUCTURE REPLACEMENT ACTIVITIES, THE UNDERSIDE OF BRIDGE NO. STA-21-0446 SHALL BE CAREFULLY EXAMINED FOR THE PRESENCE OF BATS, ESPECIALLY FORM APRIL 1 TO SEPTEMBER 30.  IF ANY BATS ARE FOUND ROOSTING ON THE UNDERSIDE OF A BRIDGE, THE USFWS, ECOLOGICAL SERVICES DIVISION (614-416-8993), ODOT OFFICE OF ENVIRONMENTAL SERVICES (614-466-7880) AND ODOT DISTRICT 4 ENVIRONMENTAL SECTION (330-786-4930) SHALL BE CONTACTED TO PROVIDE THIS INFORMATION.

3. The Project Designer shall incorporate the following note into the plans:

PRIOR TO ANY STRUCTURE REPLACEMENT ACTIVITIES, THE UNDERSIDE OF BRIDGE NO. STA-172-0420 SHALL BE CAREFULLY EXAMINED FOR THE PRESENCE OF BATS, ESPECIALLY FORM APRIL 1 TO SEPTEMBER 30.  IF ANY BATS ARE FOUND ROOSTING ON THE UNDERSIDE OF A BRIDGE, THE USFWS, ECOLOGICAL SERVICES DIVISION (614-416-8993), ODOT OFFICE OF ENVIRONMENTAL SERVICES (614-466-7880) AND ODOT DISTRICT 4 ENVIRONMENTAL SECTION (330-786-4930) SHALL BE CONTACTED TO PROVIDE THIS INFORMATION.

4. The Project Designer shall incorporate the following note into the plans:

ALL PROJECTS INVOLVING JURISDICTIONAL WATERS OF THE UNITED STATES (STREAMS, RIVERS, NON-ISOLATED WETLANDS) AND/OR ISOLATED WETLANDS ARE SUBJECT TO REGULATION UNDER SECTIONS 404 AND 401 OF THE CLEAN WATER ACT, AND POSSIBLY OHIO EPA ISOLATED WETLAND LAW.  THE WATERWAY PERMITS CONDITIONS VERIFIED FOR THE PROJECT HAVE BEEN INCORPORATED INTO THE PROJECT CONSTRUCTION PLAN AS SPECIAL PROVISIONS. THE PROJECT CONTRACTOR SHALL REVIEW AND ADHERE TO ALL WATERWAY PERMIT TERMS AND CONDITIONS THROUGHOUT PROJECT CONSTRUCTION.

5. The Project Designer shall incorporate the following note into the plans:

NATURAL BUFFERS ADJACENT TO STREAMS AND WETLANDS SHALL BE PRESERVED TO THE MAXIMUM EXTENT PRACTICABLE.  EXISTING RIPARIAN HABITATS SHOULD BE MAINTAINED TO THE MAXIMUM EXTENT PRACTICABLE AND EQUIPMENT STAGING AREAS SHALL BE KEPT WELL AWAY FROM STREAMS AND WETLANDS TO THE EXTENT PRACTICABLE.  ODOT CONSTRUCTION AND MATERIALS SPECIFICATIONS SECTION 107.10 (PROTECTION AND RESTORATION OF PROPERTY) PROHIBIT THE CONTRACTOR FROM CREATING STAGING AREAS NEAR STREAMS/WETLANDS.  AREAS DISTURBED BY THE PROJECT SHALL BE SEEDED/REVEGETATED WITH PLANT SPECIES AND MULCHED DURING CONSTRUCTION TO ENCOURAGE ESTABLISHMENT OF VEGETATION COVER, DECREASE EROSION AND PREVENT EROSION OF SEDIMENTS INTO WATERS OF THE UNITED STATES.

6. The Project Designer shall incorporate the following note into the plans:

THE MECHANICAL EQUIPMENT USED TO EXECUTE THE WORK AUTHORIZED HEREIN SHALL BE OPERATED IN SUCH A WAY AS TO MINIMIZE TURBIDITY THAT COULD DEGRADE WATER QUALITY AND ADVERSELY AFFECT AQUATIC PLANT AND ANIMAL LIFE.

7. The Project Designer shall incorporate the following note into the plans:

THE CONTRACTOR SHALL TAKE PRECAUTIONS TO AVOID AND/OR LIMIT CONSTRUCTION AND DEMOLITION DEBRIS FROM ENTERING STREAMS.  ANY DEBRIS THAT DOES FALL INTO STREAMS SHALL BE REMOVED AS SOON AS POSSIBLE WITHIN 72 HOURS.

8. The Project Designer shall incorporate the following note into the plans:

THE CONTRACTOR SHALL TAKE ALL PRECAUTIONS NECESSARY TO PREVENT EPOXY-URETHANE SEALER, PAINT, OR OTHER MATERIALS USED TO REPAIR, CLEAN, SEAL, OR TREAT ANY BRIDGE STRUCTURE FROM ENTERING ANY STREAMS, WETLANDS OR OTHER WATERS OF THE UNITED STATES AND TAKE THE APPROPRIATE ACTIONS IN THE EVENT OF A RELEASE.

9. The Project Designer shall incorporate the following note into the plans:

WATER COLUMN AND SEDIMENTATION IMPACTS SHALL BE KEPT TO A MINIMUM THROUGH THE USE OF BEST MANAGEMENT PRACTICES FOR SOIL EROSION AND SEDIMENTATION CONTROL.  ALL SOIL EROSION AND SEDIMENT CONTROL MEASURES SHALL BE IN PLACE PRIOR TO ANY EXCAVATION, GRADING OR FILLING OPERATIONS AND INSTALLATION OF PROPOSED STRUCTURES OR UTILITIES.  THEY SHALL REMAIN IN PLACE UNTIL CONSTRUCTION IS COMPLETED AND THE AREA IS STABILIZED AS ACCEPTED BY THE ENGINEER.

Environmental Commitments

C2

Commitments

1. The Project Designer shall incorporate the following note into the plans:

THE PROJECT IS LOCATED WITHIN THE KNOWN HABITAT RANGES OF THE FEDERALLY LISTED AND PROTECTED INDIANA BAT AND NORTHERN LONG-EARED BAT.  NO TREES SHALL BE REMOVED UNDER THIS PROJECT FROM APRIL 1 THROUGH SEPTEMBER 30.  ALL NECESSARY TREE REMOVAL SHALL OCCUR FROM OCTOBER 1 THROUGH MARCH 31. THIS REQUIREMENT IS NECESSARY TO AVOID AND MINIMIZE IMPACT TO THESE SPECIES AS REQUIRED BY THE ENDANGERED SPECIES ACT. FOR THE PURPOSES OF THIS NOTE, A TREE IS DEFINED AS A LIVE, DYING, OR DEAD WOODY PLANT, WITH A TRUNK THREE INCHES OR GREATER IN DIAMETER AT A HEIGHT OF 4.5 FEET ABOVE THE GROUND SURFACE, AND WITH A MINIMUM HEIGHT OF 13 FEET.

2. The Project Designer shall incorporate the following note into the plans:

PRIOR TO ANY STRUCTURE REPLACEMENT ACTIVITIES, THE UNDERSIDE OF BRIDGE NO. STA-21-0446 SHALL BE CAREFULLY EXAMINED FOR THE PRESENCE OF BATS, ESPECIALLY FORM APRIL 1 TO SEPTEMBER 30.  IF ANY BATS ARE FOUND ROOSTING ON THE UNDERSIDE OF A BRIDGE, THE USFWS, ECOLOGICAL SERVICES DIVISION (614-416-8993), ODOT OFFICE OF ENVIRONMENTAL SERVICES (614-466-7880) AND ODOT DISTRICT 4 ENVIRONMENTAL SECTION (330-786-4930) SHALL BE CONTACTED TO PROVIDE THIS INFORMATION.

3. The Project Designer shall incorporate the following note into the plans:

PRIOR TO ANY STRUCTURE REPLACEMENT ACTIVITIES, THE UNDERSIDE OF BRIDGE NO. STA-172-0420 SHALL BE CAREFULLY EXAMINED FOR THE PRESENCE OF BATS, ESPECIALLY FORM APRIL 1 TO SEPTEMBER 30.  IF ANY BATS ARE FOUND ROOSTING ON THE UNDERSIDE OF A BRIDGE, THE USFWS, ECOLOGICAL SERVICES DIVISION (614-416-8993), ODOT OFFICE OF ENVIRONMENTAL SERVICES (614-466-7880) AND ODOT DISTRICT 4 ENVIRONMENTAL SECTION (330-786-4930) SHALL BE CONTACTED TO PROVIDE THIS INFORMATION.

4. The Project Designer shall incorporate the following note into the plans:

ALL PROJECTS INVOLVING JURISDICTIONAL WATERS OF THE UNITED STATES (STREAMS, RIVERS, NON-ISOLATED WETLANDS) AND/OR ISOLATED WETLANDS ARE SUBJECT TO REGULATION UNDER SECTIONS 404 AND 401 OF THE CLEAN WATER ACT, AND POSSIBLY OHIO EPA ISOLATED WETLAND LAW.  THE WATERWAY PERMITS CONDITIONS VERIFIED FOR THE PROJECT HAVE BEEN INCORPORATED INTO THE PROJECT CONSTRUCTION PLAN AS SPECIAL PROVISIONS. THE PROJECT CONTRACTOR SHALL REVIEW AND ADHERE TO ALL WATERWAY PERMIT TERMS AND CONDITIONS THROUGHOUT PROJECT CONSTRUCTION.

5. The Project Designer shall incorporate the following note into the plans:

NATURAL BUFFERS ADJACENT TO STREAMS AND WETLANDS SHALL BE PRESERVED TO THE MAXIMUM EXTENT PRACTICABLE.  EXISTING RIPARIAN HABITATS SHOULD BE MAINTAINED TO THE MAXIMUM EXTENT PRACTICABLE AND EQUIPMENT STAGING AREAS SHALL BE KEPT WELL AWAY FROM STREAMS AND WETLANDS TO THE EXTENT PRACTICABLE.  ODOT CONSTRUCTION AND MATERIALS SPECIFICATIONS SECTION 107.10 (PROTECTION AND RESTORATION OF PROPERTY) PROHIBIT THE CONTRACTOR FROM CREATING STAGING AREAS NEAR STREAMS/WETLANDS.  AREAS DISTURBED BY THE PROJECT SHALL BE SEEDED/REVEGETATED WITH PLANT SPECIES AND MULCHED DURING CONSTRUCTION TO ENCOURAGE ESTABLISHMENT OF VEGETATION COVER, DECREASE EROSION AND PREVENT EROSION OF SEDIMENTS INTO WATERS OF THE UNITED STATES.

6. The Project Designer shall incorporate the following note into the plans:

THE MECHANICAL EQUIPMENT USED TO EXECUTE THE WORK AUTHORIZED HEREIN SHALL BE OPERATED IN SUCH A WAY AS TO MINIMIZE TURBIDITY THAT COULD DEGRADE WATER QUALITY AND ADVERSELY AFFECT AQUATIC PLANT AND ANIMAL LIFE.

7. The Project Designer shall incorporate the following note into the plans:

THE CONTRACTOR SHALL TAKE PRECAUTIONS TO AVOID AND/OR LIMIT CONSTRUCTION AND DEMOLITION DEBRIS FROM ENTERING STREAMS.  ANY DEBRIS THAT DOES FALL INTO STREAMS SHALL BE REMOVED AS SOON AS POSSIBLE WITHIN 72 HOURS.

8. The Project Designer shall incorporate the following note into the plans:

THE CONTRACTOR SHALL TAKE ALL PRECAUTIONS NECESSARY TO PREVENT EPOXY-URETHANE SEALER, PAINT, OR OTHER MATERIALS USED TO REPAIR, CLEAN, SEAL, OR TREAT ANY BRIDGE STRUCTURE FROM ENTERING ANY STREAMS, WETLANDS OR OTHER WATERS OF THE UNITED STATES AND TAKE THE APPROPRIATE ACTIONS IN THE EVENT OF A RELEASE.

9. The Project Designer shall incorporate the following note into the plans:

WATER COLUMN AND SEDIMENTATION IMPACTS SHALL BE KEPT TO A MINIMUM THROUGH THE USE OF BEST MANAGEMENT PRACTICES FOR SOIL EROSION AND SEDIMENTATION CONTROL.  ALL SOIL EROSION AND SEDIMENT CONTROL MEASURES SHALL BE IN PLACE PRIOR TO ANY EXCAVATION, GRADING OR FILLING OPERATIONS AND INSTALLATION OF PROPOSED STRUCTURES OR UTILITIES.  THEY SHALL REMAIN IN PLACE UNTIL CONSTRUCTION IS COMPLETED AND THE AREA IS STABILIZED AS ACCEPTED BY THE ENGINEER.

Preparers  & Approvals

Form Preparer(s)

| Form Preparer | District 4 |
| --- | --- |

| Supporting Form Preparer(s) | Brian Peck, Rinaldo Spano |
| --- | --- |

Approvals & Electronic Signatures

| Approved & Electronically Signed By: | Approval Date: |
| --- | --- |

| Edward Deley PROGRAM ADMIN 3 | November 22, 2017 |
| --- | --- |
| Edward Deley PROGRAM ADMIN 3 | November 22, 2017 |

Appendix

Uploaded Files by Report Section

General

Final Right of Way Plans.pdf

Stage 3 Design.pdf

USGS Quadrangle Topographical Map.pdf

ESA

OEPA Demolition and Renovation Notification Form.pdf

Regulated Materials Review Form - STA-21-0446.pdf

Regulated Materials Review Form - STA-172-0420.pdf

Cultural Resources

Section 106 Scoping Request Form.pdf

Minimal Potential to Cause Effects - A or B.pdf

Ecological

Level 1 Ecological Survey STA-172-4.20.pdf

Level 1 Ecological Survey STA-21-4.46.pdf

Coordination with ODNR and USFWS.pdf

Project Consultation Summary Form STA-21-4.46.pdf

Project Consultation Summary Form STA-172-4.20.pdf

Response to Agency Comments.pdf

Environmental Justice

Census Mapping.pdf

Underserved Populations Documentation Form.pdf

Public Involvement

Emergency and Public Services Detour Notification.pdf

Property Owner Notification Letter.pdf

Press Release.pdf

Emergency and Public Services MOT Notification.pdf

Public Comments Received.pdf

Response to Public Comments.pdf

Permits

Permit Determination Request.pdf

FEMA FIRM.pdf

OES Permit Determination.pdf

Special Provisions.pdf