Environmental Document

for

STA US 0030 00.00 PID 87284

Environmental Document Level: C1

Approved: October 4, 2017

Prepared by: District 4

The environmental review, consultation, and other actions required by applicable Federal environmental laws for this project are being, or have been, carried-out by ODOT pursuant to 23 U.S.C. 327 and a Memorandum of Understanding dated December 11, 2015, and executed by FHWA and ODOT.

Contents

C1	3

Environmental Commitments	9

9

Preparers  & Approvals	10

Form Preparer(s)	10

Approvals & Electronic Signatures	10

Appendix	11

Uploaded Files by Report Section	11

General	11

Cultural Resources	11

C1

| PID | 87284 |
| --- | --- |

| Project Sponsor | DISTRICT 4-MAINTENANCE |
| --- | --- |

| ODOT District | 4 |
| --- | --- |

| Funding Source | Federal |
| --- | --- |

| Private Funding: | No |
| --- | --- |

Project Description:

The project proposes structure maintenance on nine (9) U. S. Route30 (US 30) bridges from the Stark/Wayne County Line at Straight Line Mile (SLM)0.00 to Carmont Avenue at SLM 6.03.   The project also proposes to repair or replace damaged anddeteriorated median catch basins along 12.6 miles of US 30 from 0.46 mile eastof the Stark/Wayne County Line at SLM 0.46 to approximately State Route 297 at SLM13.06.  The project is located inTuscarawas, Perry and Canton Townships and the cities of Massillon and Canton,Stark County Ohio.  Location mapping can be found in the ProjectFile/General/Project Information subsection as USGS Quadrangle Topographical Map.pdf.

Proposed structure maintenance activities are summarized below.

StructureNo. STA-30-0000/Structure File Number (SFN) 7607164:

- Seal the existingwearing surface and approach slabs with a concrete sealing treatment.

Clean the existing structural steel beams, girders, bearings and abutment seat.

Remove above ground vegetation within 15 feet of the bridge and install new structure identification signs.

StructureNo. STA-30-0091L/SFN 7607202 & STA-30-0092R/SFN 7607180:

- Seal the existing wearing surface and approach slabs with a concrete sealing treatment.

- Patch the concrete substructure and parapet and seal with Epoxy-Urethane sealer.

- Replace the existing strip seals, rehabilitate and reset the existing abutment bearings and clean the abutment seat.

- Remove spalled areas of concrete from the bottom of the deck floor and seal with Epoxy-Urethane sealer.

- Remove above ground vegetation within 15 feet of the bridge and install new structure identification signs.

Structure No. STA-30-0263L/SFN 7607245 & STA-30-0264R/SFN 7607229:

- Seal the existing wearing surface and approach slabs with a concrete sealing treatment.

- Patch the concrete substructure and parapet and seal with Epoxy-Urethane sealer.

- Replace the existing strip seals, rehabilitate and reset the existing abutment bearings and clean the abutment seat.

- Remove above ground vegetation within 15 feet of the bridge and install new structure identification signs.

StructureNo. STA-30-0536L/SFN 7607296 & STA-30-0535R/SFN 7607261:

StructureNo. STA-30-0602L/SFN 7607342 & STA-30-0603R/SFN 7607326:

A minimum of one 10-foot lane will be maintained in each directionon US 30, Deerfield Avenue, Alabama Avenue, State Route 93, Pigeon Run Avenueand Carmont Avenue at all times during bridge maintenance activities and twolanes will be maintained in each direction on US 30 at all times during median catchbasin repair/replacement activities.

All work shall be performed within the existing US 30 limitedaccess and other roadway rights-of-way.

Existing utilities will not be removed or relocated to constructthe proposed project.

The project will not impact any environmentally sensitive resourceswithin the project study areas.

Stage 3 plans were utilized to evaluate potential environmental impacts by and obtain NEPA approval for the project.

| STIP Reference # | Statewide Line Item (SLI) Number 32 and is fiscally constrained. |
| --- | --- |

| Cultural Resources Coordination: | Minimal Potential to Cause Effect - Appendix A |
| --- | --- |

| Cultural Resources Coordination Date: | 09/20/2017 |
| --- | --- |

| Supporting documentation has been uploaded to Project File: | Yes |
| --- | --- |

| Select the appropriate project type (more than one can be selected): | (22) Projects, as defined in 23 U.S.C. 101 that would take place entirely within the existing operational right-of-way. Existing operational right-of-way refers to right-of-way that has been disturbed for an existing transportation facility or is maintained for a transportation purpose. This area includes the features associated with the physical footprint of the transportation facility (including the roadway, bridges, interchanges, culverts, drainage, fixed guideways, mitigation areas, etc.) and other areas maintained for transportation purposes such as clear zone, traffic control signage, landscaping, any rest areas with direct access to a controlled access highway, areas maintained for safety and security of a transportation facility, parking facilities with direct access to an existing transportation facility, transit power substations, transit venting structures, and transit maintenance facilities. Portions of the right-of-way that have not been disturbed or that are not maintained for transportation purposes are not in the existing operational right-of-way. Examples include: Tower lighting within the existing operational right-of-way; Guardrail installation and replacement (including median cable barriers) where roadway ditches and backslopes will not be relocated; Improvements to existing ODOT/County maintenance facilities; Construction of new ODOT/County maintenance facilities within existing operational right-of-way; Environmental mitigation activities within existing operational right-of-way; Work on pedestrian and vehicle transfer structures and associated utilities, buildings, and terminals within existing operational right-of-way; Construction of alternative energy facilities (fuel tank farms, wind turbines, etc.) |
| --- | --- |

| Environmental Commitments: | No |
| --- | --- |

Environmental Commitments

Preparers  & Approvals

Form Preparer(s)

| Form Preparer | District 4 |
| --- | --- |

| Supporting Form Preparer(s) | Brian Peck, Matthew Chaney |
| --- | --- |

Approvals & Electronic Signatures

| Approved & Electronically Signed By: | Approval Date: |
| --- | --- |

| Edward Deley PROGRAM ADMIN 3 | October 4, 2017 |
| --- | --- |
| Edward Deley PROGRAM ADMIN 3 | October 4, 2017 |

Appendix

Uploaded Files by Report Section

General

USGS Quadrangle Topographical Map.pdf

Bridge Scope.pdf

Stage 3 Design.pdf

Cultural Resources

Records Check.pdf

Buckeye Assets Mapping.pdf

Records Check Data Files.pdf